Setting up DI 106/300 for internet access where ISP assigns dynamic single IP address

Setting up the workstation.

Tcp/ip configuration.

Make sure that you have got the tcp/ip protocol installed. This can be done in control panel networks.

Screen below shows a pc with tcp/ip protocol loaded.

If it is loaded then look at the properties by selecting the  protocol then click on properties.

Select specify an IP address and Subnet mask (IP address should be a valid IP address for your network and the Subnet mask should also be the right one for the IP address).

Specify the default gateway under the gateway tab.

The gateway will be the IP address of the router. So make a note of the gateway that you enter as you will need it later on.

Specify the DNS settings under the DNS Configuration.

First enable DNS, enter host name, this can be anything you like. In the DNS server search order  enter the DNS IP address provided by your ISP.

That completes the workstation setup.

# Router setup

Connect up the routers RS232 port to your pc’s com port (make note of the com port number)

Go to Start, programs, accessories, Hyper terminal double click on hyperterm.exe.

You will be prompted for a name

Enter anything you like(router). Then click ok.

You will be asked for phone number

Set connect using to direct to the com port that you are using.(I have got the router connected to com port 2) then do ok..

It will prompt for com port properties.

The setting should be the same as shown above. If not change the settings so that they are the same. Then do ok.

You will be prompted with a blank screen with a flashing cursor on the top left hand corner. Press enter and you will be prompted with the following screen.

The default password is 1234. Then you will be displayed with the following main menu.

To select an option press the number to go back a menu press escape this will not save the settings. To save the settings go to the bottom of the screen and confirm the settings.

Select option 1

It should read as follows

System name can be anything you like. Route IP should be YES.

Then go back a menu and select 2

ISDN data should be your local isdn number including area code.

Then go to menu 3 then menu 2

In this menu you will be specifying the IP address of the router.

My router IP address is the same as the default gateway that I have set on the PC.

Now go into menu 4, You need to have your ISP details handy.

Here you will need to enter the ISP’s name, phone number, your login and password. Set single user account to yes. IP address to 0.0.0.0.

That’s the configuration complete.