|  | Ethernet |
| --- | --- |
|  | Remote Bridge/Router |

DI-1137 is available in many different configurations, for ISDN BRI, Leased-Line or Frame Relay and Leased Line with ISDN backup as well as simultaneous Leased Line and ISDN Internet access. Compression is a Standard Option

Bridge Router for ISDN, Frame Relay, Leased-Line, Leased-Line Backup, Leased Line and Internet together.

The DI-1137 is an extremely powerful and flexible modular solution with the abilities for inter-connecting remote Ethernet LANs at up to E1 link speeds, for Internet access and for LAN to LAN with simultaneous ISDN Internet connection.

| Convenience The DI-1137’s versatile modular design means it can be configured with various WAN interfaces to suit specific requirements. Should it become necessary it can be upgraded, for instance, from ISDN to Leased Line, if the on-line time makes continued ISDN use uneconomic. ISDN charges, are however, controlled by features built into the DI-1137 such as Bonding, Spoofing and link activation timing. Security In the Wide Area Network, security is of utmost importance. The DI-1137 meets your strictest requirements to protect against unauthorised entry by providing in-built firewall protection with D-Link NetProtect Firewall. This allows only specified IP addresses in, blocking all others. Whilst using NAT and DHCP features provides natural firewall protection. The filters built into the DI-1137 allow additional tightly controlled access to your sensitive data while allowing free access to authorised users. Two levels of dial-up security are also provided through the XID and password authentication protocols. ISDN Call Line ID (CLID) provides security by specifying the CLID of those devices from which calls will be accepted and provides CLID screening on call resumption whilst ISDN Callback (also based on CLID) provides a further level of security. Other standard security features include password protection and PAP and CHAP authentication protocols. | Connectivity DI-1137s can be linked together in a point-to-point topology using one or two high-speed links up to 2.048Mbps (E1) each. It supports PPP-MP over Leased Line, ISDN and Frame Relay (RFC 1490) over leased Line, supporting up to 128 DLCIs. One ISDN channel may be used to backup a leased line connection or it can be used to simultaneously access the Internet whist the DI-1137 is connected via a Leased Line to another LAN. DI-1137s can also be used to link multiple remote LANs together in a multi-point topology, each of its two WAN ports connecting to separate remote routers. Its unique closed WAN loop feature allows all sites to be inter-connected, for optimal traffic flow, without the need for a redundant link. When the DI-1137 is used in an ISDN environment it will sense when your data transfer has ceased and will terminate the call, saving call charges, while “spoofing” the services offered by the server(s). When data is to be transferred again the access to the network service will automatically and transparently be re-established by the DI-1137. To ensure complete flexibility, the DI-1137 is available with a selection of LAN and WAN interfaces, 10Base-T, 10Base-2, 10Base-5, ISDN BRI S/T and U, V.11/X.21, V.35, RS-232/V.24, RS-530/RS-422 and G.703 offering a uniquely superior and flexible solution. |
| --- | --- |

Key Features

| - IP/IPX Routing & Transparent Bridging - ISDN Callback, PPP Switched/Permanent Channels & Digital Leased Circuit at 64 and 128K - PPP-MP over Leased Line, ISDN or Frame Relay - NAT, NAPT and Reverse NAPT - DHCP Server - D-Link NetProtect Firewall | - Dual T1/E1 Link Speeds - up to 4Mbps - RFC 1490 Frame Relay Support up to 128 DLCIs - Protocol Spoofing & Bandwidth on demand - SNMP and Telnet Manageable - Compression to 600% (Link Speeds to 256Kbps) - MTBF: over 100,000 hours - Optional G.703 Interface |  |
| --- | --- | --- |

## Ethernet

### Remote Bridge/Router

Technical Specifications

| Routing Protocols |  | LAN Interfaces |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Novell IPX |  | - Single port with choice of: - 10BaseT ( RJ-45) - 10Base2 ( BNC) - 10Base5 (AUI), compatible with FOIRL fibre transceiver |  |  |  |  |  |  |  |  |  |
| - RIP (Routing Information Protocol) |  |  |  |  |  |  |  |  |  |  |  |
| - SAP (Service Advertisement Protocol) |  |  |  |  |  |  |  |  |  |  |  |
| - SAP Spoofing |  |  |  |  |  |  |  |  |  |  |  |
| IP (Internet Protocol) |  |  |  |  |  |  |  |  |  |  |  |
| - RIP (Routing Information Protocol) |  | ________________________________ |  |  |  |  |  |  |  |  |  |
| - ARP (Address Resolution Protocol) |  | WAN Interfaces |  |  |  |  |  |  |  |  |  |
| - Proxy ARP |  | - Choice of two or one plus ISDN interface for Leased Line backup: - RS-232/V.24 - V.35 - RS-530/RS-422 - V.11/X.21 - G.703 Coax ________________________________ |  |  |  |  |  |  |  |  |  |
| - ICMP (Internet Control Message Protocol) |  |  |  |  |  |  |  |  |  |  |  |
| ________________________________ |  |  |  |  |  |  |  |  |  |  |  |
| Bridging Protocols |  |  |  |  |  |  |  |  |  |  |  |
| - IEEE 802.1D - STP (Spanning Tree Protocol) - IEEE 802.3/Ethernet - Transparent MAC layer bridging - Closed-loop bridging (Delta/Triangulation) - Tail circuiting (Intermediate LAN pass) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ISDN Interface - NI1 (North America) - NI2 (North America) - DMS100 (Canada - Northern Telecom) - 5ESSPP (AT & Switch Multi Point ) - 5ESSMP (AT & Switch Multi Point) - KDD (Japan - International) - TPH 1962 (Australia, New Zealand) - NET3/Euro ISDN (Europe, Middle East, Israel, Africa, South Africa) |  |  |  |  |  |  |  |  |  |
| ________________________________ |  |  |  |  |  |  |  |  |  |  |  |
| WAN Protocols |  |  |  |  |  |  |  |  |  |  |  |
| - HDLC/LAPB, Frame Relay, (RFC 1490 Supporting 128 DLCIs) |  |  |  |  |  |  |  |  |  |  |  |
| - X25, PPP |  |  |  |  |  |  |  |  |  |  |  |
| _________________________________ |  |  |  |  |  |  |  |  |  |  |  |
| Other Protocols |  | ________________________________ |  |  |  |  |  |  |  |  |  |
| - UDP (User Datagram Protocol) - TCP (Transmission Control Protocol) - TFTP (Trivial File Transfer Protocol) |  | CSU/DSU Interface |  |  |  |  |  |  |  |  |  |
|  |  | - Leased-line 56kbps |  |  |  |  |  |  |  |  |  |
|  |  | ________________________________ |  |  |  |  |  |  |  |  |  |
|  |  | Dimensions |  |  |  |  |  |  |  |  |  |
| ________________________________ |  | - 50mm x 250mm x 280mm |  |  |  |  |  |  |  |  |  |
| Network Management - SNMP (Simple Network Management protocol) MIB II, Bridge MIB, Enterprise MIB - TELNET (In-band management) - Local RS-232 console port (Out of band management) - NAT, NAPT and Reverse NAPT - DHCP |  | ________________________________ |  |  |  |  |  |  |  |  |  |
|  |  | Weight |  |  |  |  |  |  |  |  |  |
|  |  | - 3.5kg |  |  |  |  |  |  |  |  |  |
|  |  | ________________________________ |  |  |  |  |  |  |  |  |  |
|  |  | Temperature |  |  |  |  |  |  |  |  |  |
|  |  | - 2º to 50ºC |  |  |  |  |  |  |  |  |  |
| _______________________________ |  | ________________________________ |  |  |  | Ordering Information |  |  |  |  |  |
| Wide-Area Bandwidth Management |  | Humidity |  |  |  | DI-1137-11/TP |  | ISDN 10Base T, compression |  |  |  |
| - Transparent link activation/deactivation on: - Time of Day - IP or IPX destination address - LAN traffic |  | - 20% to 80% |  |  |  |  |  |  |  |  |  |
|  |  | - Non Condensing |  |  |  | DI-1137-22/TP |  | Dual Port X.21/ V.11 10BaseT, compression |  |  |  |
|  |  | ________________________________ |  |  |  |  |  |  |  |  |  |
|  |  | Power |  |  |  | DI-1137-33/TP |  | Dual Port V.35 10BaseT, compression |  |  |  |
| - Automatic activation of 2nd Link on main link threshold - Automatic dial-up Link Recovery |  | - 80 to 240VAC |  |  |  |  |  |  |  |  |  |
|  |  | - 50 to 60Hz |  |  |  | DI-1137-44/TP |  | Dual RS-232/V.24 10BaseT, compression |  |  |  |
|  |  | - 25 Watts max |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | DI-1137-55/TP |  | Dual RS-530 10BaseT, compression |  |  |  |
| _______________________________ |  |  |  |  |  |  |  |  |  |  |  |
| WAN Speeds - 9.6 to 2.048 Mbps - Up to 600% data compression (on links up to 256Kbps) |  |  |  |  |  | DI-1137 is also available with single WAN interface, with ISDN and other WAN interface and with alternative BNC or AUI LAN connectivity |  |  |  |  |  |
|  | Europe |  | TEL: 44 181 235 5555 | FAX: 44 181 235 5500 | Singapore |  | TEL: 65 774 6233 |  | FAX: 65 774 6322 |  |  |
|  | Germany |  | TEL: 49 6192 97110 | FAX: 49 -6192-971111 | Taiwan |  | TEL: 886 2 2916 1600 |  | FAX: 886 2 2914 6299 |  |  |
| Specifications subject to change without prior notice. D-Link is a registered trademark of D-Link Corporation/D-Link System inc. All other trademarks belong to their proprietors. | France |  | TEL: 33 1 30 23 86 88 | FAX: 33 1 30 23 86 89 | China |  | TEL: 86 10 6846 7106 |  | FAX: 86 10 6846 7110 |  |  |
|  | Sweden |  | TEL: 46 8 700 62 11 | FAX: 46 8 21 96 40 | Japan |  | TEL: 81 3 5434 9678 |  | FAX: 81 3 5434 9868 |  |  |
|  | Denmark |  | TEL: 45 4396 9040 | FAX: 45 4342 4347 | Australia |  | TEL: 61 2 9417 7100 |  | FAX: 61 2 9417 1077 |  |  |
|  | # Italy |  | TEL: 39 2 2900 0676 | FAX: 39 2 2900 1723 | India |  | TEL: 91 22 617 2478 |  | FAX: 91 22 611 2476 |  |  |
|  | Middle East |  | TEL: 202-2456176 | FAX: 202-2456192 | U.S.A. |  | TEL: 1 714 455 1688 |  | FAX: 1 714 455 2521 |  |  |
|  | U.K |  | TEL: 44 181 235 5555 | FAX: 44 181 235 5500 | Canada |  | TEL: 1 905 829 5033 |  | FAX: 1 905 829 5223 |  |  |