DI-524

Date: July14, 2005

| Product name: | 802.11g 54Mbps Wireless Cable/DSL Internet Gateway w/USB Printer Port |  |
| --- | --- | --- |
| Product Group & Type: | SOHO Router |  |
| Revision: | B1 |  |
| Availability: | B/Feb, 2005 |  |
| Picture: |  |  |
| Why Sell This Product: | Last year we introduced the widely sold DI-514 as a low-cost wireless router to supplement your 802.11b product line. Now, we’re repeating this success by introducing the DI-524 to supplement your low cost 802.11g product line. This wireless router targets home/SOHO users upgrading to higher-speed wireless LAN. This router is ISP independent, it can connect to any external DSL or cable modem. Selling Points - Low cost, entry-level router, but has many software features normally available in more expensive routers. - Setup wizard for quick installation should help entry level users. - Additional hardware benefits like 4 built-in switch ports. |  |
| Key Features: | - Remote router with 802.11g wireless LAN interface - Cable/DSL modem Internet sharing - Wireless LAN speed up to 54Mbps - 10/100BASE-TX port for DSL/cable modem connection - WAN port supports PPPoE for Internet dial-up mode - 4 built-in 10/100BASE-TX switch ports with auto MDI/MDIX support - Firewall features include NAT with VPN pass-through, MAC/IP/URL filtering, domain blocking and scheduling - Web-based configuration with setup wizard for quick installation |  |
| Technical Details: | Standards - IEEE 802.11g - IEEE 802.11b - IEEE 802.3 - IEEE 802.3u Wireless Modulation Technology Orthogonal Frequency Division Multiplexing (OFDM) Wireless Frequency Range 2.4GHz to 2.462GHz Wireless Signal Rates (With automatic fallback) 54Mbps, 48Mbps, 36Mbps, 24Mbps, 18Mbps, 12Mbps, 11Mbps, 9Mbps, 6Mbps, 5, 5Mbps, 2Mbps, 1Mbps Wireless Receiver Sensitivity - 54Mbps OFDM, 10% PER, -68dBm - 48Mbps OFDM, 10% PER, -68dBm - 36Mbps OFDM, 10% PER, -75dBm - 24Mbps OFDM, 10% PER, -79dBm - 18Mbps OFDM, 10% PER, -82dBm - 12Mbps OFDM, 10% PER, -84dBm - 11Mbps CCK, 8% PER, -82dBm - 9Mbps OFDM, 10% PER, -87dBm - 6Mbps OFDM, 10% PER, -88dBm - 5.5Mbps CCK, 8% PER, -85dBm - 2Mbps QPSK, 8% PER, -86dBm - 1Mbps BPSK, 8% PER, -89dBm Wireless Transmit Output Power 15dBm +/- 2dBm External Antenna Type Single detachable reverse SMA Wireless Operating Range - Indoors: up to 100 m - Outdoors: up to 300 m * Environmental factors may adversely affect operating range Security - 802.1x user authentication support - 64-, 128-WEP wireless data encryption - WPA Wi-Fi Protected Access (WEP with TKIP, MIC, IV Expansion, Shared Key Authentication) Firewall Features - NAT (Network Address Translation) with VPN pass-through - MAC address filtering - IP address filtering - URL filtering - Domain blocking - Scheduling VPN Pass-Through/Multi-Sessions - PPTP - L2TP - IPSec Device Management - Web-based Internet Explorer v.6 or Netscape Navigator v6 or other Java-enabled browsers - DHCP Server and Client Diagnostic LEDs - Power - Status - WAN - LAN (10/100Mbps) - WLAN Physical & Environmental Power Input Through external power adapter Dimensions 273 (L) x 205 (W) x 66 (H) mm Weight 1.32 kg Operating Temperature 0 to 40 ℃ Humidity 90% maximum (non-condensing) Safety and Emissions - FCC - CE - C-Tick |  |
| EMI/EMC Report | FCC No.  CE No.  VCCI No.  CTR-21 No.  C-Tick No.  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  Others |  |
| Safety |  TUV No.  UL No.  CSA No.  Others |  |
| Packaging Style |  Clamshell  Brown Box  Gift Box  Bulk  Others |  |
| Packaging Contents: | ## DI-524 Wireless Router ## CD-ROM with Manual - Quick Installation Guide - Ethernet Cable - AC Power Adapter |  |
| UPC Code: | Part Number; UPC code. II524BEU..B1; 790069; 274572. II524EIS..B1; 790069; 273056. II524EEU..B1; 790069; 273063. II524ENA..B1; 790069; 273070. II524ANA..B1; 790069; 273087. II524NEU..B1; 790069; 273094. TI524ANA..B1; 790069; 273100. NI524CEU..B1; 790069; 273117. EI524BEU..B1; 790069; 282539. EI524EEU..B1; 790069; 282546. |
| Units Per Carton: | 5 |  |
| Cartons Per Pallet: | For WW by Air: 72 Sea: 72 | For UK by Air: 72 Sea: 72 |
| Dimensions and Weight: | Box: , Others mm, 1.28 kg Carton: 370*290*230 mm, 7 kg Pallet: (by Air) 110*120*205 cm, 525 kg (for WW&UK) (by Sea) 110*120*205 cm, 525 kg (for WW&UK) INTRASTAT weight: 824g (Box) Packing material weight (Box): - Paper: 437g - Plastic: 15.5g - Mixed: 3.64g |  |