# DI-624 Firmware Release Note

Firmware: 2.53b69

Hardware: C3

Date: Apr 20, 2005

Problems Resolved:

- Remove Cipher type(TKIP/AES) from wireless page.

- Remove Super-G with static function for Wi-Fi reason.

Enhancements:

None

Firmware: 2.52b66

Hardware: C3

Date: Apr 8, 2005

Problems Resolved:

1.Remove Super G Static Turbo mode, and the default value is Dynamic Turbo.

2.When time select to auto NTP, then apply will update the system time immediately.

Enhancements:

None

Firmware: 2.52b65

Hardware: C3

Date: Apr 1, 2005

Problems Resolved:

1.Fixed CTS mode function can’t work normally in web page.

2.Fixed Auto select channel function can’t work in wireless page.

Enhancements:

- Add WAN-L2TP, WAN-PPTP gateway selection in the Wizard DNS function page.

- Add WMM(wireless QoS) function in the wireless page

Firmware: PURE_DEMOb64

Hardware: C3

Date: Apr 1, 2005

Problems Resolved:

1.Modify the TEADevInfo file.

Enhancements:

None

Firmware: PURE_DEMOb63

Hardware: C3

Date: Mar 31, 2005

Problems Resolved:

1.provide port speed/duplex information in string.

Enhancements:

None

Firmware: 2.52b62

Hardware: C3

Date: Mar 23, 2005

Problems Resolved:

1.The web page is Germany version.

Enhancements:

None

2005/3/02  Version v2.52 B61 (with JS)

1.Fix WPA2 can't get ip issue.

2.Remove JumpStart function at Tools-System, and Status_Device.

2005/3/02  Version v2.52 B60 (WW no JS)

1.Fix wpa2 can't get ip issue.

2.Remove JumpStart function at Tools-System, and Status_Device.

2005/3/01  Version v2.52 B59 (with JS)

1.This version has JumpStart function.

2005/3/01  Version v2.52 B58 (WW no JS)

1.Remove JumpStart function at Tools-System, and Status_Device.

2.fixed "cable test" bug: wan port link status error.(at More Info)

3.fixed low Rx bug: change the value in nml->ar531x.bld->defines=AR_BUF_SIZE=(2048) to (3196).

2005/2/24  Version v2.52 B57

1.Remove JumpStart function at Tools-System, and Status_Device.

2005/2/17  Version v2.52 B56

1.Fix dynamic turbo reboot.

2005/2/15  Version v2.52 B55

1.fixed bug: the WPA2 can't get ip.

2005/2/01  Version v2.52 B54

1.modify device_info : status Encryption.

2.fixed bug: the WPA2 can't get ip.

2005/2/01  Version v2.52 B53

1.fixed bug: the WAN port speed settings on the DI-624 do not work.

2.fixed bug: the WPA2 can't get ip.

2005/1/20  Version v2.52 B52

1. Fixed when use super G with dynamic mode, Auto Select button can't selection.

2005/1/20  Version v2.52 B51

1. Fixed WPA mode , when sta connect to device , the device wiil reboot.

2005/1/13  Version v2.52 B50

1. Fixed Change wireless page ，Wep WPA WPA2 WPA2-Auto，can’t apply.

2005/1/13  Version v2.52 B49

1. Fixed DI-624(AP) in WDS mode will drop the unicast packets from station.

2. Add Cipher Type & PSK / EAP setting in wireless page.

2005/1/07  Version v2.52 B48

1. Fixed JumpStart should be able to be reset by pressing the reset button in the device.

reboot: press the reset button for 1 sec.

reset JumpStart : press the reset button for 3 sec.

restore to factory default: press  the reset button for 8 sec.

2. Fixed JumpStart settings should be able to be saved unless to press the reset button in the GUI or physical button in the device.

3. Fix ed When AP/RT was in JS-P2 mode, and disable it JS function, wireless config shold back to default SSID and Open Auth.

2005/1/03  Version v2.52 B47

1. Fixed Channel auto select can’t work

2. Fixed In JumpStart mode, station can’t IP by Router’s DHCP server

2004/12/29  Version v2.52 B46

1. Fixed Reset JumpStart Function

2. Fixed When Station connected with WPA-PSK, on webpage still show WEP encryption.

3. Fixed multicast pass through can’t work with upnp.

2004/12/28  Version v2.52 B45

1. Fixed Reset JumpStart Function

2. Fixed When Station connected with WPA-PSK, on webpage still show WEP encryption.

3. Fixed Router will keep rebooting when the Station connected with encryption.

4. Fixed When we select "Shared key" then click Apply button, the webpage still show "Open System".

5. Fixed DI-624 will reboot when run eMule or BT

6. Fixed Under Firewall page if we want to modify IP, System always show "xxx has been already existing " . We must modify name together.

7. Fixed Under Virtual Server page if we want to Modify IP, System always show "xxx has been already existing " . We must modify name together.

2004/12/22  Version v2.52 B44

1. Add JumpStart Function

2. Fixed Virtual server fail: have to change the name when you modify the IP address

2004/12/22  Version v2.52 B43

1. Add JumpStart Function

2. Fixed Virtual server fail: have to change the name when you modify the IP address

2004/12/10  Version v2.52 B42

1. fixed WPA / WPA-PSK not work

2. fixed DI-624 will reboot when run eMule or BT

Version v2.52 B41

1. fixed unable to save settings when set PPTP as dynamic PPTP.

Version V2.52 B40

1. Add " Dynamic PPTP " option in the WAN configuration page

2. Change default wireless channel from "1" to "6".

3. Fixed rules name of URL Filter can key-in the same key word

4. Fixed In Allow to ping WAN port , Destination should be WAN , not LAN ,192.168.0.1

5. Fixed rules name of Firewall can key-in the same name.

6. Fixed bug no.2043 : Parental Control function's help info link error.

7. Fixed bug no.2044 : DDNS help page link error.

8. Fixed under wizard page set password function can't be set empty.

9. Fixed bug no.2041 : Under wireless page if we choose 802.11g only mode enabled, the choose after we command apply button.

Version V2.52 B39

1. Fixed LT2P & PPTP WAN clients are failed to connect to the internet.

2. Fixed Max characters in Server IP field is limit again to 16 characters instead of 32 characters as should be.

3. Fixed No Connect/ Disconnect buttons in L2TP & PPTP WAN clients.

Version V2.52 B38

1. fix ZAP redirect to correct website.

2. Change the TCP SYN ACK response timeout value in nat_timeisup() but eccept the 604DDM & 624DDM

3. Disable the dtrap(152), D-Link customer ask (PM:Maqqie)

4. fix unable to access two French website:www.laronde.com & www.lemondialsaq.com

5. change WAN port default to 10/100Mbps auto

Version V2.52 B37

1. Fixed the issue :In case to use the DI-624 as pure AP with LAN connection, 3 sets of DI-624 will reboot.

Version V2.52 B36

1. fixed UPNP bug

2. fixed WPA-PSK cannot work

Version V2.52 B35

1. update wireless and stack version.

Version V2.52 B34

1. fix from upnp show vs rule must reboot bug

Version V2.52 B33

1. have XR Mode

Non-XR-------------------------------------------------------------------------------Non-XR

Version V2.51 B32

1. update new stack version.

Version V2.51 B31

1. fixed UPnP issue

Version V2.51 B30

1. Fixed connection between WPA with MacOS Airport unstable.

2. Checksum error while trying to upload setting after firmware update

3. UPnP issue

Version V2.50b29  09/13/04

1. Enable AR mode in default setting.

2. Modify L2TP/PPTP "Server IP" name length can set to 32 character.

Version V2.50 B28

1.fixed XR bug

Version V2.50 B27

1) Modify NTP server update timeing, when user apply setting, NTP server will update immediately.

2)Add web UI from (Dial On Demand/Auto-reconnect) to (Always-on/Manual/Connect-on demand) for PPPoE connection type (same with 624+)

3)fixed tools/time linnk to status page

Version V2.50 B26

1) rename V2.45 B25

2) Fixed unstable when use E-Mule and BT.

Version V2.45 B25

1) Fixed unstable when use Bigpond.

Version V2.45 B24

1) Firewall entries disappear after setting it.........

2) Move DDNS selection from cable test page to Tools.

3) DHCP IP will conflict with static IP once you set to DHCP Range 2-254 and assign Static IP within that range.

Version V2.45 B22

1. Change Atheros driver to V3.3.0.156

Version V2.45 B21

1. Improve throughput in dynamic turbo mode

Version V2.45 B20

1.	Add warning message in Wireless Page

*Enabling Extended Range Mode will automatically disable Super G with Static Turbo

and SSID Broadcast mode.

*Super G with Dynamic Turbo and Super G with Static Turbo mode only operates in

Channel 6.

Version V2.45 B19

1. Change GUI. Move "Super-G", "SSID Broadcast", "802.11g Only" from

"Performance" page to "Wireless" page.

When XR mode Enable, no "Super-G with Static Turbo",

"SSID Broadcast disable" and fixed TX rate

Version V2.45 B18

1.	Router will auto clear DHCP reserved IP once user change LAN IP address.

2.	Fixed WAN port and LAN port connect to same switch cause traffic broadcast packet

3.	Add static and dynamic IP in PPTP, L2TP page

4.	Add Dial on demand and Auto-reconnect in PPTP, L2TP page.

5.	Change virtual server entry port from Virtual Server to Firewall page.

6.	Fixed DHCP reserve IP can shown in MAC filter Clone field.

7.	Add Multicast stream selection in MISC page.

Version V2.45 B17

1. Update Atheros code v3.3.0.136

Version V2.45 B16

1. Update Atheros newest driver v3.3.0.134

Version V2.45 B15

1. Update Atheros patch code, station switch base and xr mode in some distance.

Version V2.45 B14

1. Fixed XR GUI bug.

Version V2.45 B12

1. Update Atheros patch, Dynamic turbo to base mode, beacon stop sending.

2. Fixed XR GUI bug.

3. change WAN speed default setting Auto to 10M.

4. change virtual server list

Version V2.45 b11

1. Fixed shared key unstable with TI Client.

Version V2.45 b9

1. Fixed shared key unstable with non-atheros client.

Version V2.45 B0008

1. Support XR function

2. Remove static turbo when XR mode enabled, and TX rate can't modify

Version v2.44 B0010

1. This build fixed a bug in stack/natrt/natmms.c

Version v2.44 B0002

1. This build has backup mode re-built.

Version v2.44 B0001

1. Support ATMEL flesh.

Version v2.43 B0003

1. Fixed L2TP(PPPoE) problem.

2. Add 2 virtual server entries by Cephalus.

3. Define LARGEBUFSIZE=12288 in ipport.h.

Version v2.42 B0003

1. Fix bug that WAN Ethernet port proformance is slow (by Leo).

2. Fix bug that Chklst.txt can't be downloaded.

3. Add PPTP function in Israel domain.(by Raymomd)

4. Fix bug that 624 will reboot when running under SuperG with dynamic Turbo mode(No.1480)

5. Accept both 0x07 and 0x51 for Israel domain

6. Re-open Isarel domain (0x07) ,channel : 5 - 13

Version 2.38

1.Support Atmel(2M)

2.Support SDRAM 32 bits

Version v2.37

1.Fixed aging time bug.

Version v2.36

change v2.32 to v2.36

Version v2.35

change v2.31 to v2.35 for MP

Version 2.32

1.Add DDM

2.Add WDS support

3.Add Atmel flash support

Version 2.31bx

1.Add DDM

Version 2.31 (2003/12/24)

1.Add micErrorProcess

2.change WPA-PSK character max. length from 64 to 63

Version 2.30

1. Add Isarel domain (0x07), channel : 5 - 13

2. Fixed PPPOE bug.

Version 2.29 (2003/12/15)

1.Add another button on left menu in the Advanced page call "Parental Control". Move URL Filtering and Domain Filtering" to this page. Leave IP and MAC Filtering in the Advanced->Filters page.

2.Add option to save log files to local drive as a text file.

3.add the option for wireless routers to auto scan channels, so it can automatically select the best channel to use

4.Add an option to WAN section to allow user to select 10/100 Auto, 100 Mbps, or 10Mbps.

5.Add the option to allow user to select DNS pass-through or DNS Relay.By default, it is DNS Relay. Put this option in Home->LAN.-- Move this to Home->LAN.

6.fix DynDNS.org will drop an account if it does not receive an update from the client after 20days.

7.Add Clone MAC feature for Static IP and PPPoE.

8.For wireless portion, need to set WEP entry to allow user to enter in the max appropriate value. Do not let users enter more than available chracters.

9.Fix Netscape browse wireless page can view the channel value.

10.extend NTP server field to 30 chaacters.

11.change antenna TxPower selection to only 4 options.

12.make a DDNS a drop down list. Add easydns.com and no-ip.com to the list.

13.fix telnet time-out too short.

Version 2.29

1. Change Super G default value to "Super G with Dynamic Turbo"

2. Support 2312 small package

Version 2.28

1. Fix Setup Wizard Fail Bug.

Version 2.27

1. Fix Wep Key Enable cause Reboot bug.

Version 2.26

1. Support Super G

Version 2.25

1. Atheros SOC initial Version

Version 1.25

1. fix bug of #13 "Dlink Canada is reporting that customers are having problems

connecting Checkpoint VPN clients behind the routers to the VPN servers.

They need to forward port 500 to the client, and force UDP Encapsulation on

the checkpoint client software."

2. fix bug of #14 "There is no edit or delete buttons when you add entries to

the Static DHCP client list.  The only way to get rid of the entry is to copy

the MAC address from the dropdown menu and then disable the entry."

3. fix bug of #16 "Since the addition of Static DHCP there is also many log

entries now about Spoof attacks.  It seems like any client that tries to

renew an IP address gets logged as doing a spoof attack on the router."

4. fix bug of #17 "once a computer has obtained an IP address from the router,

the computer name is only held until the router is rebooted.  The router

will hold onto the MAC address but the computer name becomes Unknown.

If the same computer then releases and renews it's IP address and the

computer name is lees that the amount of characters in the word "Unknown",

it will add the remaining letters onto the name in the DHCP client table."

5. fix bug "AP reboot when STA re-associated.

Version 1.24b2

1. suppport WPA, WPA-PSK

Version 1.24

1. upgrade Atheros firmware 2.4.0.71

Version 1.23

1. Fixed wizard wep key bug.

2. Change Intersil firmware to 1.0.1.0

3. Add Frmae Burst

Version 1.22

1. Fixed Reboot time-out bug( UPNP Task)

Version 1.21

1. Fixed DHCP data abort bug.

Version 1.20

1. Add Reserved IP

2. Change BLKD_DOMAIN_LEN from 31 to 40

3. Add SUPPORT_DCS2000, DVC1000

4. Add MAIL_ATTACK_LOG

5. Add SMURF_ATTACK

6. Add LAND_ATTACK

7. Add SPOOF_ATTACK

8. Add NETMEETING

9. Support Maximum packet to 8K

Version 1.16

1. use Intersil Firmware version 1.0.3.0

Version 1.20 B4

1. fix pppoe buffer free bug

Version 1.20 B3

1. reboot when buffer equal zero

pkalloc.c

2. UPNP timeout fix on 90 second

Version 1.20 B2 May/27/2003

1. stack No. 1124

2. fix dwl-650+ connection bug on WINXP

3. reduce UPNP timeout to 90 sec

4. add pkt buffer is 0 then reboot

5. Intersil firmware 0.8.0.0

Version 1.14 Apr/25/2003

1.Intersil support FrameBurst, gOnly, ssid suppress

2.Atheros support gOnly, ssid suppress

3.change default channel to atheros default channel when empty slot (factory request)

Version 1.13 Apr/14/2003

1.Intersil Firmware go back to Ver 0.8.0.0

Version 1.12 Mar/25/2003

1.Remove noise immunity

Version 1.12 Mar/12/2003

1.add noise immunity

Version 1.11 Feb/21/2003

1. fix bug of "Static DNS option in WAN Dynamic IP Address page does not allow

"0.0.0.0" to be used. If you do not want to use Static DNS, then you must

clear out the DNS box in order for router to accept configuration. Please

allow "0.0.0.0" to be entered in DNS field. Entering "0.0.0.0" should be

the same as leaving the field blank, meaning Router will use dynamic DNS

address provided by ISP."

2. fix bug of "When doing a traceroute from a computer within the Internal LAN, the

first hop is the ISP gateway IP address. The second hop is the same IP.

This is incorrect, the first hop should be the router's LAN IP

(192.168.0.1). I thought this was a problem with the PC because the PC

should know the first hop but it is not the PC. We replaced the 624 with

another router/gateway and the problem does not exist. The first hop is

192.168.0.1 which is correct."

3. fix bug of "For the scheduling function, users cannot set a weekend schedule to

take effect. Setting schedule from Sat. to Sun. will cause an "Invalid"

error message"

4. add MTU value to all WAN type(DHCP, FIX, PPPoE, PPTP)

5. support primary and secondary DNS for WAN(DHCP)

6. Remove wep key 256 support in help.

7. fix bug Intersil wep key 2,3,4 fail.

8. update Atheros firmware to 2.3.0.70

10. stack no. 1089

11. fix b, g running reboot bug

12. update Intersil firmware to 0.8.1.0

Version 1.10 Feb/12/2003

1. support Atheros AR5212 MB22g, MB22ag

Version 1.09 Feb/4/2003

1. update Intersil firmware 0.8.0.0