# DI-624S Firmware Release Note

D-link Firmware: v1.00

Alpha Firmware: v4.0.0b13

Hardware: RealTek8651B + Atheros 2414 + NEC USB2.0

Date:2005/09/21

Problems Resolved:

- Can’t copy large file (ex. 800MB) to USB disk

- After copy files to USB disk, unplug the USB disk, the copied files would disappear.

- BTComet complain UPNP AddPortMapping fail.

- Topology show WAN connection is down when WAN type is static ip mode.

- Set IP Filter with protocol to both would cause Internet access fail.

- DHCP client blind accept ACK packets.

- Active session fail.

Features Added:

- Support directory indexing for personal web server.

- PPTP/L2TP server can be domain name.

- Support TTL don’t touch for ISP may set TTL to 1.

- Add, “email to” and email authentication support in Log Email setting.

D-link Firmware: v1.00

Alpha Firmware: v4.0.0b12

Hardware: RealTek8651B + Atheros 2414 + NEC USB2.0

Date:2005/08/17

Problems Resolved:

- Router would drop the wireless UDP packet which checksum is zero.

- Device would crash when user running Emule and enable Domain block function.

- Printer can’t print out completely.

- Disable router DNS relay cache function to avoid user can’t get correct ip address when DDNS ip changed.

- Improve webpage access speed.

- Change UDP session timeout time from 15 minutes to 1 minute.

Features Added:

- Support ext2 file system for usb storage device.

- Support Bonjour with FTP, HTTP, Printer services.

D-link Firmware: v1.00

Alpha Firmware: v4.0.0b11

Hardware: RealTek8651B + Atheros 2414 + NEC USB2.0

Date:2005/07/07

Problems Resolved:

- BigPond hear beat can’t work.

- Change WAN DHCP client Disconnect as release and won’t try to
 renew.

- LAN to LAN upnp multicast can not forward.

Known issues:

- USB device plug into USB 2.0 HUB can’t be detected.

D-link Firmware: v1.00

Alpha Firmware: v4.0.0b10

Hardware: RealTek8651B + Atheros 2414 + NEC USB2.0

Date:2005/07/01

Problems Resolved:

- Bigpond can not connect to server

- the special flash disk(FAT16) can not detect.

- Media server can not fast forward issue.

Known issues:

- USB device plug into USB 2.0 HUB can’t be detected.

D-link Firmware: v4.0.0b9

Alpha Firmware: v4.0.0b9

Hardware: RealTek8651B + Atheros 2414 + NEC USB2.0

Date:2005/06/02

Problems Resolved:

- Public port can't be changed, and can't test.

- Connect to error IP when reboot.

- QOS check error issue on web

- Do not use hi throughput of samba download.

- Printer bug when using 2 printers.

Known issues:

1. TCP session can NOT close complete.

- USB device plug into USB 2.0 HUB can’t be detected.

- QOS physical port have bug with some NIC card (SIS NIC on mainboard).

D-link Firmware: v4.0.0b8

Alpha Firmware: v4.0.0b8

Hardware: RealTek8651B + Atheros 2414 + NEC USB2.0

Date: Tue May 31, 2005

Fix:

1. The WAN do not response Ping Request

2. PPTP L2TP do not get DNS from server

3.Read Country Code From board Information.

Known issues:

- TCP session can NOT close complete.

- USB device plug into USB 2.0 HUB can’t be detected.

D-link Firmware: v4.0.0b7

Alpha Firmware: v4.0.0b7

Hardware: RealTek8651B + Atheros 2414 + NEC USB2.0

Date: Tue May 19, 2005

Fix:

- QOS setting fail

2.  Add all help file

Known issues:

- TCP session can NOT close complete.

- USB device plug into USB 2.0 HUB can’t be detected.

D-link Firmware: v4.0.0b6

Alpha Firmware: v4.0.0b6

Hardware: RealTek8651B + Atheros 2414 + NEC USB2.0

Date: Tue Apr 25, 2005

Fix:

- Firewall Fail.

- Some hostname input fail.

Known issues:

- TCP session can NOT close complete.

- USB device plug into USB 2.0 HUB can’t be detected.

D-link Firmware: v4.0.0b5

Alpha Firmware: v4.0.0b5

Hardware: RealTek8651B + Atheros 2414 + NEC USB2.0

Date: Tue Apr 19, 2005

Fix:

- PPTP dial let device Crash

- wireless QOS packets let device crash

- super G show in wireless status instead of 11g.

- WCN not works in special words.

Known issues:

- TCP session can NOT close complete.

- USB device plug into USB 2.0 HUB can’t be detected.

Features:

- bluetooth dongle support(PAN)

- network topology.

- printer que name auto generate.

D-link Firmware: v4.0.0b4

Alpha Firmware: v4.0.0b4

Hardware: RealTek8651B + Atheros 2414 + NEC USB2.0

Date: Fri Mar 30, 2005

Fix:

- "Load Settings From Local Hard Drive" function fail.

- media server MAC filter fail

- Group-delete function fail and make web hang up

- Can create a file in device without plug in any USB disk.

- Device can't close all sessions when connection is terminated.

- SAMBA share and group member lost after reboot.

- L2TP and PPTP doesn't work

- Plug in 2 storage devices, one is card reader and another is USB storage device, the disk information in status page list 3 disks.

Known issues:

- USB device plug into USB 2.0 HUB can’t be detected.

Features:

- Personal web support.

D-link Firmware: v4.0.0b3

Alpha Firmware: v4.0.0b3

Hardware: RealTek8651B + Atheros 2414 + NEC USB2.0

Date: Fri Mar 8, 2005

Fix:

1.Just remove wireless debug message ,Increase wireless through.

Known issues:

1.WPA-AUTO-PSK can’t work with D-Link G650 WPA2-PSK.(Maybe D-Link G650 driver bug)

2.USB device plug into USB 2.0 HUB can’t be detected.

D-link Firmware: v4.0.0b2

Alpha Firmware: v4.0.0b2

Hardware: RealTek8651B + Atheros 2414 + NEC USB2.0

Date: Fri Mar 4, 2005

Fix:

- Media server compatible with dlink DSM-320 media player.

- Samba add user or group will fail if samba is not enable.

- Ftp WAN access fail, when LAN and WAN use different port.

- USB port LED.

- Wireless LED.

- L2 switching fail when add ACL rule.

- Using WPA-PSK-AUTO setting in Wizard page.

- Printer web pages added.

- Remove multi-PPPOE web pages.

- WCN not work in SSL web pages.

Known issues:

- WPA-AUTO-PSK can’t work with D-Link G650 WPA2-PSK.(Maybe D-Link G650 driver bug)

- USB device plug into USB 2.0 HUB can’t be detected.

D-link Firmware: v4.0.0b1

Alpha Firmware: v4.0.0b1

Hardware: RealTek8651B + Atheros 2414 + NEC USB2.0

Date: Feb 25, 2005

First time release.

Features:

- Atheros SuperG & WPA2 included.

- QoS with bandwidth and priority control.

- HTTPS always on to protect information.

- Have SAMBA, FTP server and UpnP AV supported.

Known issues:

- WPA-AUTO-PSK can’t work with D-Link G650 WPA2-PSK.

- USB device plug into USB 2.0 HUB can’t be detected.

- UpnP AV not stable yet.

- LEDs of USB ports, wireless not function yet, software not light it yet.

Problems Resolved:

Enhancements: