# DI-724P+ Firmware Release Note

Firmware: v1.03

Hardware: A1

Date: Jan 11, 2005

Problems Resolved:

1.Fix "Router can’t allow 32 and more keys input in WPA_PSK."

2.Fix "daylight saving can not work to span two years."

3.Fix “LAN PCs can't access remote FTP server if router is in SPI mode.”

4.Fix “Router can't drop PPPoE connection even though the idle time has expired.”

5.Fix “can’t Auto-detect i2eye device, and can’t open/close port 1720 automatically.”

6.Fix “ The router updates to Peanut-Hull too frequently if users set wrong user name or password.”

7. Fix “ Using "user" to remote login can't go back correctly when click "Continue" button.”

8. Fix “Router not reply ARP request in specific time leads to ping time out.”

Enhancements:

- Add “Support to detect the key is HEX or ASCII key when the client encrypts with WPA-PSK.”

- Add “downgrade-presented function for IC+175c chipset.”

- Add “Router will jump a window to remind users the configurations of DDNS were not completed.”

- Add “ Change Peanut-Hull server to HPH002.oray.net.”

Firmware: v1.02

Hardware: A1

Date: Oct 19, 2004

Problems Resolved:

- Remove Multi Global IP.

- Fix "Wireless Setting UI(Radio on/off).

=============================================================

Firmware: v1.02

Hardware: A1

Date: Oct 04, 2004

Problems Resolved:

- Support Syslog server on WAN side.

- Fix the device will hang down when WAN & LAN loop-back.

- Fix speed Enhance Mode function will be disabled when enables WPA-PSK function.

=============================================================

Firmware: v1.01b05

Hardware: A1

Date: Sep 20, 2004

Problems Resolved:

- Fix failed items of CTRouter test.

- Change firewall destination IP from * to real destination IP if the rule is derived from Virtual Server.

- Support NO-IP DDNS service.

- Fix Netmeeting (on Microsoft Windows 2000) is unable to connected success through VPN tunnel.

- Can't establish Manual IPSec tunnel with early firmware version.

- Add Radio on/off in wireless Configuration.

- Improve wireless Stable.

=============================================================

First Release

******************************

Section A. UPGRADE INSTRUCTION

******************************

For users of Microsoft Windows, please run 724P+_100b04.EXE

directly. It will search the Router on your LAN and ask

you to type system password. Just type in the password

and click "Upgrade" button. It takes about 30 seconds

to finish the whole procedure.

For users of other platforms, please refer Section D for

platform-independent upgrade.

******************************

Section B. UPGRADE INFORMATION

******************************

The files will upgrade the firmware of your Cable/DSL

Router to version 1.00b04.

1. NOTE! DO NOT POWER OFF THE UNIT WHEN THE UNIT IS BEING

UPGRADED, OR THE UNIT MAY BE DAMAGED.

2. The configuration will not be changed. You need not

configure it again.

*************************

Section C. CHANGE HISTORY

*************************

*** V1.02(2004-10-07) TO V1.02(2004-10-19)

1.Remove Multi Global IP.

2.Fix"Wireless Setting UI(Radio on/off).

*** V1.01b05 TO V1.02(2004-10-04)

1. Support Syslog server on WAN side.

2. Fix the device will hang down when WAN & LAN loop-back.

3. Fix speed Enhance Mode function will be disabled when enables WPA-PSK function.

*** V1.01b04 TO V1.01b05(2004-09-20)

1.Fix failed items of CDRouter test.

2.Change firewall destination IP from * to real destination IP if the rule is derived from Virtual Server.

3.Support NO-IP DDNS service.

4.Fix the following problem:

(1) Netmeeting (on Microsoft Windows 2000) is unable to connected success through VPN tunnel.

(2) Can't establish Manual IPSec tunnel with early firmware version.

5.Add Radio on/off in wireless Configuration.

6.Improve wireless Stable.

*** V1.01b03 TO V1.01b04(2004-08-30)

1.Support i2eye ALG.

2.Prevent different devices from using the same XID to send DHCP request.

3.Virtual Server doesn't work if public port and private port are not the same.

4.Enhance the wireless performance.(Call G+)

5.Change the source interface of firewall rule "Allow to Ping WAN port" from LAN to WAN.

(The change will take effect after resetting router setting to factory default.)

6.Support Multi Global IP (Virtual Computer) for Static and Dynamic connection.

*** V1.01b02 TO V1.01b03(2004-07-26)

1.Fix " DI-724P+ can not be keepalive in Bigpond ISP.

2.Support Lotus Mail Server in E-mail Alert.

*** V1.01b01 TO V1.01b02

1.Fixed "Linux-Dns server can not query under Nat,when SPI mode is enabled.

2.Chnage Default Setting "Connect mode select changes as Connect-on-denand

Maximunm Idle Time is 5 minutes."

*** V1.01 TO V1.01b01

1.Support IGMP for Singapore

2.Change Ti wireless Card Module and verify wireless stable.

3.Verify BigPond problem in  australia.

4.the certification of Centrino.

*** 1.00b8 TO V1.01

1.Change Firmware Version as V1.01.

2.Change the sender of Format.(change admin@wan-ip address to admin@domain-name).

But there is still a bug,some mail-servers access to deny for admin@domain-name,

like admin@pchome.com.tw,Amit will modify the same address of Sender and Receiver.

For example,the receiver is D-Link@dlink.com.tw and the Sender is also D-Link@dlink.com.tw.

Amit will modify this by next Fw.

*** 1.00b7 TO 1.00b08

1.Support X-Box about TCP Fin-test of UPNP function.

2.Fix "The device can not renew leased-time automatically when wan type is dynnamic ip address."

3.Modify Firewall rule list(change Lan ip 192.168.0.1 to *).

4.Modify "the wireless connection is down,when DWL-G650+ enables 8x Mode.

*** 1.00b5 TO 1.00b07

1.Fix "When Setuping Mac filter,the setting will skip to Domain Blocking." in Filter Page.

2.Fix "The System-log will not show filter-log when enabling the rules of domain filter." in the Filter page.

3.Fix "The settings,like Authentication type ,SSID broadcast and 8X Mode will skip in the Perfromance page."

4.Fix "The wireless can not work(The wireless button is gone) when the

wan type is ppp(pppoe,l2tp,pptp or dial-up) and enables WPA ro 802.1x."

5.Fix "Power save makes the wirelss connection down,for example,Centrino or wireless-clients enable power-save mode.

*** 1.00b4 TO 1.00b05

1. Add Connection Mode select in WAN Page.

2. Add Tx Power function in Performance Page.

*** 1.00b3 TO 1.00b04

1.Fix Wireless SSID will lost "+" when click WEP disable to enable.

2.Improve the wireless throughput.

3.Fix when Wep setting is not working in Wizard mode.

4.Fix that Wireless tab under Status is not working.

5.Fix that Intersil 11g  can not associate when G mode only.

6.Fix DWL-650+ can not associate when SSID broadcast disables.

*** 1.00b02 TO 1.00b03

1.Change UI title image.

*** 1.00b01 TO 1.00b02

1. Add WPA.

2. Add L2TP.

3. Add PeanutHull.

4. Add Daylight Saving.

5. Change Domail Filter seat.

*** 1.00 TO 1.00b01

1. Fix remote management.

2. Fix wireless card fail.

3. Fix DynDNS.org.

3. Fix Wireless SSID will lost "+" when click WEP disable to enable.

*** 1.00

First release.

************************************************

Section D. Platform-independent Firmware Upgrade

************************************************

First, go to the D-Link Web site to download firmware image

file, 724P+_100b04.BIN. Remember the location you save this file.

And use the following method to upgrade:

* WEB UPGRADE:

1. Connect the Web user interface and login as system

administrator.

2. Click the "Firmware Upgrade" button of "Tools" page.

And the "Firmware Upgrade" screen will appear.

3. Enter the location of the previously downloaded file in

the text box, or click "Browse" button to search for the

file. And then click "Upgrade" button.