DI-808HV

Date: October 7, 2003

| Product name: | Ethernet Broadband Router with 8-Port Switch and USB Printer Server |
| --- | --- |
| Product Group & Type: | Broadband Router |
| Revision: | A1 |
| Available: |  |
| Picture: |  |
| Why Sell This Product: | - Supplementing current product DI-804HV: the new DI-808HV is similar to the DI-804HV, but instead of having 4 integrated switch ports, the new DI-808HV has 8 built-in switch ports to meet the requirement of today's office environments. Selling Points - Good price/performance: routing, VPN IPSec and firewall functions performed by hardware, relieving the device's CPU a lot of overheads. - Has additional advanced functions like enabled NetBIOS broadcast, Dynamic DNS support and SDI Service. - Many other vendor routers don't have backup WAN port like this DI-808HV's RS-232 for ISDN/analog modem backup. |
| Keyfeatures: | - 168-bit 3DES IPSec advanced VPN security - 3DES with SHA-1 advanced secure configuration - Up to 40 VPN tunnels to 40 remote sites - DoS, SPI, packet filtering firewall protection - 10/100BASE-TX port for DSL/cable modem connection - Additional RS-232 COM port for ISDN or 56Kbps analog modem backup supporting dial-on-demand & idle timer auto-disconnection - 8 10/100BASE-TX switch ports for 8 workstation/server connections - Aggressive/main client mold for VPN - Web-based configuration setup - Enabled NetBIOS broadcast - Dynamic DNS support - SDI Service (Home Internet Solution) port - Universal Plug-n-Play (UPnP) enabled |
| Technical Details: | Hardware WAN Ports - 1 10/100BASE-TX port for DSL/cable modem connection - 1 RS-232 port (460Kbps Baud) for ISDN TA/analog modem connection (for WAN backup) LAN Ports - 8 10/100BASE-TX switch ports with auto MDI/MDIX support Broadband Support - DSL modem - Cable modem - PPP over Ethernet (PPPoE) Backup Link Support - Dial-up ISDN TA - Dial-up Analog modem - PPP - Dial-on-Demand with auto login to ISP - Auto-disconnect with user-defined idle timer - PAP/CHAP security Diagnostic LEDs - Power - Status - WAN Port (Link/Act) - COM Port - 8 LAN Ports (Link/Act) Routing Routed Packet Type IP packets Routing Protocols & Methods - RIP-1, RIP-2 - Static Routing - Dynamic Routing VPN & IPSec Application Support - IPSec pass-through - PPPTP pass-through - L2TP pass-through Data Encryption Methods - IP ESP (Encapsulating Security Payload) Confidentiality over IP payload Optional authentication of IP payload and ESP header - Null encryption Encryption Transforms - 56-bit DES (Data Encryption Standard) - 168-bit 3DES Authentication Transforms (for ESP and AH) - MD5 - SHA-1 Security - Pre-shared key - Manual key - IKE Security Association (SA) assignment Connection Modes - Site to Site - Client to Site Number of Tunnels 40 IPSec tunnels (max.) Internet Gateway & Firewall Supported Functions - Network Address Translation (NAT) - DHCP server automatic IP assignment - Virtual server Packet Inspection Static Packet Inspection (SPI) Number of DMZ Ports 1 port (user-assignable to any LAN port) Firewall Security - Stateful Packet Inspection (SPI) to prevent Denial of Service (DoS) attacks (Syn flood, ICMP flood, UDP flood, "ping of death", IP spoofing, land attack, tear drop attack, IP address sweep attack, Win Nuke attack) - Intrusion Detection Logging - LAN/WAN filtering Management & Configuration IP Number Self-identification Through DHCP client Local & Remote Management Web-based utilities (via browser) Firmware Upgrade Web-based (via browser) Physical & Environmental Power Supply - DC 5V 2A - Through external power adapter Dimensions 235 x 162 x 35 mm (device only) (7.56 x 4.64 x 1.22 inches) Weight 310 grams (0.68 lb.) (device only) Operating Temperature 5 to 55 C Storage Temperature -20 to 70 C Humidity 10% to 90% non-condensing EMI Certification - FCC Class B - CE Class B Safety UL |
| EMI/EMC Report | FCC No.  CE No.  VCCI No.  CTR-21 No.  C-Tick No.  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  Others |
| Safety |  TUV No.  UL No.  CSA No.  Others |
| Packaging Style |  Clamshell  White Box  Gift Box  Bulk  Others |
| Packaging Contents: | Device, CD, QIG, RJ45 cable, rubber feet, power adapter |
| UPC Code: | 790069-262272/USA, 790069-262227/A, 790069-262234/B, 790069-262241/E, 790069-262258/N |
| Units Per Carton: | 5 |
| Cartons Per Pallet: | Air: 30, Sea: 40 |
| Dimensions and Weight: | Box: 275 x 206 x 118 mm, 1.48 kg Carton: 640 x 300 x 240 mm, 8.0 kg Pallet: (by Air) 1100 x 1200 x 1560 mm, 270 kg (by Sea)1100 x 1200 x 1960 mm, 360 kg INTRASTAT weight: kg (Box) Packing material weight (Box): - Paper: kg - Plastic: kg - Mixed: kg |