Noahsark Configuration Guide

History

- 2006/Mar/9, EricKuo, first release.

- 2006/Mar/14, EricKuo,

  - Add dhcpd_enable in D-2

  - Add default_html in D-0

  - Change wan_static_ipaddr in D-6 to wan_current_ipaddr

  - Change 2nd wan_pppoe_ipaddr_xy in D-7, to wan_pppoe_netmask_xy

  - Remove wan_pptp_service in D-8

  - Remove wan_l2tp_service in D-9

  - Change wan_pptp_dynamic in D-9 to wan_l2tp_dynamic

- 2006/Mar/14, EricKuo

  - Separate remote management’s fields

  - Separate time setting’s fields

- 2006/Mar/14, EricKuo, 19:00

  - Remove mpppoe in wan_proto, if user wants to enable mpppoe, enable the different pppoe_xy session

- 2006/Mar/15, EricKuo, 17:40

  - D.24, change pppoe pass through to IPSEC pass through

- 2006/Mar/15, 18:00, EricKuo

  - Add one field for time_zone_area in D-20 for UI to save the area

  - Add CGI for set_system_time

- 2006/Mar/17, 19:00, EricKuo

  - Add version, version_date in D-0

  - Add system_time in D-0, remove the CGI of set_system_time (modified on 2006/Mar/15)

  - Add log_per_page in D-25

  - Add CmoGetLog(“type”, “page”) to retrieve Log

  - CGI to send log email send_log_email

  - Add status CmoGetStatus(“get_current_user”), the value should be admin/user

- 2006/Mar/20, 17:00, EricKuo

  - Move wan_current_ipaddr from part 1 to part 2.

  - Change pppoe_x_..... to pppoe_xy_..... for all,

  - Define all cgi name in 4.

- 2006/Mar/20, 18:00, EricKuo

  - Add mac_clone_addr in 2 CmoGetStatus

- 2006/Mar/21, 9:30, EricKuo

  - Add 3 parameters (in D-28) for UI to control the back page. html_response_page, html_response_message, html_response_return_page

- 2006/Mar/22, 12:10, EricKuo

  - Move version, version_date from D-0 to status

  - In upgrade cgi, should specify name=file

- 2006/Mar/23, 14:50, EricKuo

  - Move system_time from D-0 to CmoGetStatus(“system_time”) and CGI

- 2006/Mar/24, 18:00, EricKuo

  - Add schedule control for mac filter (D-15) and url/domain filter (D-17)

  - Add lan_bridge in D-0

  - Remove system_time in D-0

  - Remove D-6

  - Add wireless_driver_version, wireless_domain, wireless_mac in 2 (status)

  - Change wan_current_ipaddr to wan_current_ipaddr_xy in 2 (status)

  - Add reboot_type in apply.cgi

- 2006/Mar/27, 12:00, EricKuo

  - Change reboot_type (default is all)

  - dmz_schedule in D-23

- 2006/Mar/27, 14:00, EricKuo

  - remove wlan0_channel_start, wlan0_channel_end in D-26

  - add status for wlan0_channel_list

- 2006/Mar/27, 17:00, Anderson.Chen

  - Didn’t change value even restore default setting.

admin_username, admin_password, wan_mac, lan_eth, lan_bridge and wan_eth in D-0

  - domain D-26.

- 2006/Mar/27, 18:00, Anderson.Chen

  - Remove log_email_addr, increase log_email_sender and log_email_server in D-25

- 2006/Mar/27, 19:00, Anderson.Chen

  - Remove time_zone in D-20

- 2006/Mar/28, 10:00, Anderson.Chen

  - Add time_zone in D-20

  - Remove time_zone_area in D-20

- 2006/Mar/28, 12:40, Anderson.Chen

  - Add asp_temp_wep_display  in D-28

- 2006/Mar/28, 17:10, Anderson.Chen

  - Add log_current_page and log_response_type in D-25

  - Add log_first_page, log_last_page, log_next_page, and log_prefix_page in CGI function.

  - Add log_total_page in Status

  - Modify CmoGetLog(type,page) to CmoGetLog()

- 2006/Mar/29, 12:30, Anderson.Chen

  - Change value = hardware | all | lan | wan | wireless | application | filter | none, and default = all in cgi

- 2006/Mar/30, 10:00 Anderson.Chen

  - Add ntp_default_server. value = Ipaddr | hostname

- 2006/Apr/3, 14:00, EricKuo

  - In 2. status, add wan_port_status, wan_port_speed_duplex, lan_port_status_xy, lan_port_speed_duplex_xy

  - Add cgi. vct_wan.cgi, vct_lan_xy.cgi, reset_packet_counter.cgi, save_log.cgi

- 2006/Apr/6, 11:00, Anderson.Chen

  - Add current_channel in Status

- 2006/Apr/11, 11:00 Anderson.Chen

  - Remove system_time.cgi in cgi function

  - Add system_time in D0 and define new format.

- 2006/Apr/13, 18:00 Anderson.Chen

  - Add countdown_time in D0 for count down value

- 2006/Apr/18, 15:00 Anderson.Chen

  - Add wlan0_auto_txrate in D-26 for tx fix rate or auto rate

  - Add wlan0_11g_txrate in D-26

  - Add wlan0_11b_txrate in D-26

  - Remove wlan0_txrate in D-26

- 2006/Apr/24, 18:00 Anderson.Chen

  - Add usb_test.cgi in CGI

  - Add usb_test_result in Status

- 2006/Apr/24, 19:00 Anderson.Chen

  - Add DLINK US UI style and rules.

- 2006/Apr/24, 19:30 Anderson.Chen

  - Remove DLINK US UI style, rules and define to D-LINK-UI-Style.doc for all DLINK Style products.

- 2006/Apr/25, 10:30 Anderson.Chen

  - Remove pppoe_gateway_xy in D-7

- 2006/Apr/25, 12:30 Anderson.Chen

  - Add wlan0_wmm_11n_ap_x in D-34, its for WMM value in 802.11n in D-34.

- 2006/May/9, 11:00 Anderson.Chen

  - Rename bigpond_login to bigpond_connect in CGI

  - Rename bigpond_logout to bigpond_disconnect in CGI

- 2006/May/15, 15:00 Anderson.Chen

  - Add bigpond_connection_status in Status

- 2006/May/23, 14:00 Anderson.Chen

  - Add D-29 for UPnP information data.

- 2006/July/24, 12:00 Anderson.Chen

  - Remove rip1_enable and rip2_enable in D-18

  - Add rip_enable, rip_rx_version and rip_tx_version in D-18

  - Modify static_routing_xy in D-19

- 2006/July/26, 10:30 Anderson.Chen

  - Add upnp_advertisement_period and upnp_advertisement_ttl in D-29

- 2006/July/27, 10:30 Anderson.Chen

  - Add spi_enable in D-30

- 2006/Aug/10, 15:30 Anderson.Chen

  - Add block_service_xy in D-31

- 2006/Aug/15, 14:00 Anderson.Chen

  - Define 1 = enable, 0 = disable.

  - Add router_mac_option in D-0

  - Add wan_pptp_connection_id in D-8

  - Add wan_bigpond_max_idle_time in D-10

  - Add url_domain_filter_trust_ip_xy in D-17

  - Add log_email_enable in D-25

  - Add log_email_block_enable D-25

  - Add log_email_schedule in D-25

  - Add wan_connecting_time in Status

  - Add wan_collision, lan_collision, wlan_collision, wan_uptime,lan_uptime,wlan_uptime in Status.

  - Add wlan_sta in Status

  - Add application_timeout in D-14

  - Add ddns_wildcards_enable in D-22

- 2006/Aug/24, 11:30 Anderson.Chen

  - Add wan_pppoe_mtu in D-7

  - Add wan_pptp_mtu in D-8

  - Add wan_l2tp_mtu in D-9….

- 2006/Aug/25, 14:00 Anderson Chen

  - Substitute wlan_sta for wireless_station_list (CmoGetList)

  - Add status lan_tx_bytes, lan_rx_bytes,

  - Add status wan_tx_bytes, wan_rx_bytes,

  - Add status wlan_tx_bytes, wlan_rx_bytes,

  - Add status dhcpc_obtain_time , dhcpc_expire_time

- 2006/Aug/30, 14:00 Anderson Chen

  - Add blank_state in D-0

  - Add internet_connect in Status

- 2006/Sep/22, 17:00 Anderson.Chen

  - Add wlan0_antenna_control in D-26

- 2006/Sep/27, 12:00 Anderson.Chen

  - Add pingto_addr in D-0

  - Add check_wan_connection.cgi in CGI

- 2006/Oct/02 , 18:00 Anderson Chen

  - Add apply_config.cgi in CGI

  - Add apply_confirm.cgi in CGI

  - Add apply_discard.cgi in CGI

- 2006/Oct/03, 14:00 Anderson.Chen

  - Add wlan0_mode in D-26

- 2006/0ct/03, 16:00 Anderson.Chen

  - Add wlan0_eth in D-26

- 2006/0ct/04, 14:00 Anderson.Chen

  - Add wlan0_11bg_txrate in D-26

- 2006/Oct/04, 20:00 Anderson.Chen

  - Add wlan0_radio_enable in D-26

  - Add wlan0_antenna_diversity in D-26

- 2006/Oct/12,10:00 Albert.Chen

  - Repair wlan0_txrate in D-26

  - Add wlan0_vapx_enable x=1~3 in D-26

  - Add wlan0_vapx_ssid x= 1~3 in D-26

  - Add wlan0_xr_enable in D-26

  - Add wlan0_adptive_radio_enable in D-26

  - Add wlan0_vapx_ssid_broadcast x =1~3 in D-26

  - Add wlan0_vapx_security x= 1~3 in D-26

  - Add wlan0_vapy_wep64_key_x x=1~4 y=1~3 in D-26

  - Add wlan0_vapy_wep128_key_x x=1~4 y=1~3 in D-26

  - Add wlan0_vapy_wep512_key_x x=1~4 y=1~3 in D-26

  - Add wlan0_vapx_wep_display x=1~3 in D-26

  - Add wlan0_vapx_wep_display_key x=1~3 in D-26

  - Add wlan0_vapx_psk_cipher_type x =1~3 in D-26

  - Add wlan0_vapx_psk_pass_phrase x =1~3 in D-26

  - Add wlan0_vapx_txrate x=1~3 in D-26

  - Add wlan0_vapx_beacon_interval x=1~3 in D-26

  - Add wlan0_vapx_rts_threshold x=1~3 in D-26

  - Add wlan0_vapx_fragmemtation x=1~3 in D-26

  - Add wlan0_vapx_dtim x=1~3 in D-26

  - Add wlan0_vapx_cts x=1~3 in D-26

  - Add wlan0_vapx_wmm_enable x =1~3 in D-26

  - Add wlan0_wmm_vapy_ap_x x =0~3 y= 1~3 in D-26

  - Add wlan0_wmm_vapy_bss_x x =0~3 y= 1~3 in D-26

  - Add wlan0_super_g in D-26

  - Add wlan0_vapx_super_g x=1~3 in D-26

  - Add wlan0_protection in D-26

  - Add wlan0_vapx_protection x =1~3 in D-26

  - Add wlan0_wds_enable in D-26

  - Add wlan0_vapx_wds_enable x =1~3 in D-26

- 2006/Oct/13,12:00 Nick.Chou

  - Add wlan0_remoteAP_ssid in D-26

  - Modify wlan0_super_g value in D-26

  - Modify wlan0_vapx_super_g value in D-26

  - Del wlan0_txrate in D-26

  - Add wlan0_vapx_auto_txrate in D-26

  - Add wlan0_vapx_11b_txrate in D-26

  - Add wlan0_vapx_11g_txrate in D-26

  - Add wlan0_vapx_11bg_txrate in D-26

- 2006/Oct/17, 1600 Anderson.Chen

  - Add model_number in D29

  - Add wlan_rx_frames, wlan_tx_frames(interface) in Status

- 2006/Oct/17, 1800 Anderson.Chen

  - Add limit_adm_enable in D-0

  - Add limit_adm_ip_connect in D-0

- 2006/Oct/18, 1000 Albert.Chen

A.   Add dhcpc_enable in D-1

- 2006/Oct/18, 1130 Jackey.Chen

  - Add snmpv3_enable in D-32

  - Add snmpv3_security_user_name in D-32

  - Add auth_protocol in D-32

  - Add auth_password in D-32

  - Add privacy_protocol	in D-32

  - Add privacy_password in D-32

- 2006/Oct/18, 1624 Albert Chen

  - Add login_html in D-0

- 2006/Oct/26, 1200 Anderson.Chen

  - Add console_enable in D-33

  - Add console_timeout in D-33

- 2006/Oct/31, 2013 Roy Tseng

  - Add wlan0_tls_passwd in D-26

- 2006/Nov/1, 1030 Nick Chou

  - Del RemoteAP_ssid in D-26

  - Modify wlan0_wmm_vapy_ap_x to wlan0_vapy_wmm_ap_x in D-26

  - Modify wlan0_wmm_vapy_bss_x to wlan0_vapy_wmm_bss_x in D-26

- 2006/Nov/1, 2000 Anderson.Chen

  - Modify wlan0_security in D-26 (add wpawpa2_eap option)

- 2006/Nov/2, 1240 Solo Wu

A.	Add application_enable in D-14

- 2006/Nov/3, 1400 Stella Huang

  - Add ntp_primary_server in D-20

  - Add ntp_secondary_server in D-20

- 2006/Nov/3, 1700 Roy Tseng

  - Add wlan0_eap_proto in D-26

  - Add wlan0_ttls_inner_proto in D-26

  - Add wlan0_peap_inner_proto in D-26

  - Add wlan0_ttls_login_name in D-26

  - Add wlan0_ttls_passwd in D-26

  - Add wlan0_peap_login_name in D-26

  - Add wlan0_peap_passwd in D-26

- 2006/Nov/6, 1500 Anderson.Chen

  - Remove wlan0_security in D-26 (wpawpa2_eap option)

  - Add default_lan_mac and default_wan_mac in D-0

- 2006/Nov/6, 1530 Anderson.Chen

  - Remove default_lan_mac and default_wan_mac in D-0

- 2006/Nov/7, 1200 Roy Tseng

  - Modify wan0_ttls_login_name in D-26 (it should be wlan0_ttls_login_name)

  - Modify wan0_peap_login_name in D-26 (it should be wlan0_peap_login_name)

- 2006/Nov/9 Solo Wu

  - Add httpd_login_timeout in D-0

- 2006/Nov/21 Jackey Chen

  - Add getStatus function of ddns_status.

- 2006/Dec/1 Jackey Chen

  - Add getStatus function of intel_viiv.

- 2006/Dec/4 Jackey Chen

  - Add rip_v2_type in D-18

- 2006/Dec/5 Rock Chen

  - Add static_routing_private in D-19

- 2006/Dec/6 , 18:00 Cosmo Chang

  - Add log_email_recipient in D-25

- 2006/Dec/6 , 21:00 Cosmo Chang

  - wlan0_wds_mac in D-26

- 2006/Dec/7 , 22:30 Solo Wu

A.	firmware_available in D-34

B	login_auto_upgrade in D-34

- 2006/Dec/11, 13:00 Solo Wu

A.	firmware_check_status in 76

- 2006/Dec/13, 19:50 Rock Chen

A.	ddns_dyndns_kinds in D-22

- 2006/Dec/14, 19:10 Jackey Chen

A.	wan_ppp_auth_type in D-4

- 2006/Dec/20, 14:30 YufaHuang

A.    add qos_enable in D-35

B.    add qos_uplink in D-35

C.    add qos_downlink in D-35

D.    add qos_rule_xy in D-35

- 2006/Dec/20, 17:10 Solo Wu

A.	Modify application_xy in D-14

- 2006/Dec/22, 17:25 Yufa Huang

A.	fix qos_rule_xy example in D-35

- 2006/Dec/25, 17:40 Solo Wu

A.	Add nat_filter in D-36

- 2006/Dec/26, 15:46 Jackey Chen

  - Add pptp_uptime in Status

- 2007/Jan/9, 15:40 Solo Wu

  - POT

  - POT_Backup

  - POT_Resolution

  - POT_MaxValue

- 2007/Jan/10, 19:00 Chun Lin

  - Add wps_enable in D-39

  - Add wps_configured_mode in D-39

- 2007/Jan/17, 15:20 Jackey Chen

  - Add dns_hijack in D-0

  - Add restore_mac in Status

- 2007/Jan/18, 12:30 Solo Wu

  - Add wan_set_mac in D-0

  - Remove restore_mac in Status

- 2007/Jan/19, 14:15 Jackey Chen

  - Add dhcp_server_ip in Status

- 2007/Mar/13, 16:30 Roy Tseng

  - Add tftpd_enable in D-40

- 2007/Mar/15, 13:00 Stella Huang

  - Add wps_pin in D-39

- 2007/Mar/19, 18:30 Cosmo Chang

  - Add tftpd_hardwareId_check in D-40

- 2007/April/3, 14:00 Rock Chen

  - Add wizard_flag in D-41

- 2007/April/3, 14:00 Jackey Chen

  - Add help_flag in D-27

- 2007/April/4, 10:00 Chun Lin

  - Add wps_lock in D-39

  - Add wps_default_pin in D-39

  - Add wps_generate_pin in status

- 2007/April/10, 14:00 Jackey Chen

  - Add session_id_xy in D-7

- 2007/April/10, 15:00 Jackey Chen

  - Add peer_mac_xy in D-7

- 2007/April/16, 12:00 Chun Lin

  - Add wps_generate_default_pin in status

- 2007/April/18, 11:30 Anderson.Chen

  - Add time_zone_area in D-20

- 2007/April/25, 15:00 Nick Chou

  - Add log_email_auth in D-25

- 2007/April/26, 17:00 Jackey Chen

  - Add wan_pppoe_server_ip_xy in D-7

- 2007/Mayl/03, 12:00 Nick Chou

  - Add syslog_server in D-25

- 2007/May/14, 15:00 Chun Lin

  - Add firewall_rule_xy in D-42

  - Modify port_forward_xy in D-13

  - Add wps_sta_enrollee_pin in D-39

  - Add url_domain_filter_enable_xy in D-17

  - Add wan_pptp_russia_enable in D-8

  - Add in wan_pppoe_russia_enable D-7

  - Add in wan_pppoe_russia_dynamic D-7

  - Add in wan_pppoe_russia_ipaddr D-7

  - Add in wan_pppoe_russia_netmask D-7

  - Add in wan_pppoe_russia_gateway D-7

  - Add in wan_pppoe_russia_primary_dns D-7

  - Add in wan_pppoe_russia_secondary_dns D-7

  - Modify static_routing_xy in D-19

1.nvram configuration

A. format: nvname_name=nvname_value

B. UI (WEB pages) may access via CmoGetCfg(“name”, “hex”|”ascii”|”none”) to get value)

C. UI (WEB pages) should save these value via the same “name”.

D. Content:

D-0. basic system configuration

| Hostname | wbr_3310 |  |
| --- | --- | --- |
| default_html | Ex. Index.asp |  |
| admin_username |  |  |
| admin_password |  |  |
| user_usrename |  |  |
| user_password |  |  |
| httpd_login_timeout | 5 (min) |  |
| lan_mac | ex. 00:01:23:45:67:89 |  |
| wan_mac | ex. 00:01:23:45:67:8a |  |
| wan_set_mac | 00:01:23:45:67:8a |  |
| lan_eth | ex. Eth1 |  |
| lan_bridge | br0 |  |
| wan_eth | ex. Eth0 |  |
| system_time | month/day/hour/min/year ex. 110223012006 |  |
| countdown_time | default = 40 |  |
| router_mac_option | 0 / 1 / 2 (0 = default, 1 = use computer, 2 = use manual) |  |
| blank_state | 0:wizard mode 1:normal |  |
| pingto_addr | ex. 192.168.0.1 , www.cameo.com.tw , …… |  |
| limit_adm_enable | enable (0¦1) |  |
| limit_adm_ip_connect | Ip_addr1/ip_addr2 Ex: 192.168.0.1/192.168.0.2 |  |
| dns_hijack | enable (0¦1) |  |

D-1. Lan configuration

| lan_ipaddr | ex. 192.168.0.1 |
| --- | --- |
| lan_netmask | ex. 255.255.255.0 |
| dhcpc_enable | enable (0¦1) |

D-2. DHCP server configuration (xy = 00 ~ 19)

| dhcpd_enable | enable(0¦1) |
| --- | --- |
| dhcpd_start | ex. 192.168.0.100 |
| dhcpd_end | ex. 192.168.0.200 |
| dhcpd_lease_time | ex. 1440 (minutes) |
| dhcpd_domain_name | string.com |
| dhcpd_reservation | enable(0¦1) |
| dhcpd_reserve_xy | enable(0¦1)/name/ip_addr/mac ex. 1/name01/192.168.0.11/00:01:02:03:04:01 |

D-3. DNS relay (proxy)

| dns_relay | enable(0¦1) |
| --- | --- |

D-4. WAN basic configuration

| wan_mtu | ex. 1500 |
| --- | --- |
| wan_specify_dns | enable(0¦1) |
| wan_primary_dns | ex. 192.168.32.1 |
| wan_secondary_dns | ex. 0.0.0.0 |
| wan_proto | static¦dhcpc¦pppoe¦pptp¦l2tp¦bigpond |

D-5. WAN static IP configuration

| wan_static_ipaddr | ex. 172.21.65.150 |
| --- | --- |
| wan_static_netmask | ex. 255.255.240.0 |
| wan_static_gateway | ex. 172.21.65.254 |

D-6. WAN current address information (UI: read only)

| wan_current_ipaddr | ipaddr/netmask/gateway/dns1/dns2 ex. 172.21.65.150/255.255.255.0/172.21.65.254/192.168.2.1/192.168.3.1 |
| --- | --- |

D-7. WAN PPPOE configuration (xy = 00 ~ 04)

| wan_pppoe_enable_xy | enable(0¦1) |
| --- | --- |
| wan_pppoe_dynamic_xy | enable(0¦1) |
| wan_pppoe_username_xy |  |
| wan_pppoe_password_xy |  |
| wan_pppoe_service_xy |  |
| wan_pppoe_max_idle_time_xy | (minutes) |
| wan_pppoe_connect_mode_xy | always_on¦manual¦on_demand |
| wan_pppoe_ipaddr_xy |  |
| wan_pppoe_netmask_xy |  |
| wan_pppoe_ gateway _xy |  |
| wan_pppoe_mtu |  |
| session_id_xy |  |
| peer_mac_xy |  |
| wan_pppoe_server_ip_xy |  |
| wan_pppoe_russia_enable | enable(0¦1) |
| wan_pppoe_russia_dynamic | enable(0¦1) |
| wan_pppoe_russia_ipaddr |  |
| wan_pppoe_russia_netmask |  |
| wan_pppoe_russia_gateway |  |
| wan_pppoe_russia_primary_dns |  |
| wan_pppoe_russia_secondary_dns |  |

D-8. WAN pptp configuration

| wan_pptp_dynamic | enable(0¦1) |
| --- | --- |
| wan_pptp_username |  |
| wan_pptp_password |  |
| wan_pptp_max_idle_time |  |
| wan_pptp_connect_mode | always_on¦manual¦on_demand |
| wan_pptp_ipaddr |  |
| wan_pptp_netmask |  |
| wan_pptp_gateway |  |
| wan_pptp_server_ip |  |
| wan_pptp_connection_id |  |
| wan_pptp_mtu |  |
| wan_pptp_russia_enable | enable(0¦1) |

D-9. WAN l2tp configuration

| wan_l2tp_dynamic | enable(0¦1) |
| --- | --- |
| wan_l2tp_username |  |
| wan_l2tp_password |  |
| wan_l2tp_max_idle_time |  |
| wan_l2tp_connect_mode | always_on¦manual¦on_demand |
| wan_l2tp_ipaddr |  |
| wan_l2tp_netmask |  |
| wan_l2tp_gateway |  |
| wan_l2tp_server_ip |  |
| wan_l2tp_mtu |  |

D-10. WAN bigpond configuration

| wan_bigpond_username | username/password |
| --- | --- |
| wan_bigpond_password |  |
| wan_bigpond_auth | sm-server¦dce-server |
| wan_bigpond_server | host¦ipaddr ex. Abc.com.tw 172.1.2.3 |
| wan_bigpond_max_idle_time | wan_bigpond_max_idle_time |

D-11. Schedule configuration (xy = 00 ~ 19)

| schedule_rule_xy | name/weekdays/start_time/end_time ex. Always/1111111/00:00/24:00 Test/1101011/08:00/18:00 |
| --- | --- |

D-12. Virtual Server 15onfiguration (xy = 00 ~ 19)

| vs_rule_xy | enable(0¦1)/name/prot/public_port/private_port/ip_addr/schedule_rule_00_name ex. 0/HTTP/TCP/80/80/192.168.0.100/Always 1/TELNET/TCP/23/23/192.168.0.100/Test |
| --- | --- |

D-13. Port Forwarding configuration (xy = 00 ~ 19)

| port_forward_xy | enable(0¦1)/name/UDP¦TCP¦Any/public_port_start/public_port_send/private_port_start/private_port_end/ip_addr/schedule_rule_name ex. 1/pf1/UDP/100/110/350/400//192.168.0.100/Always 0/pf2/TCP/1122/1150/2010/2030/192.168.0.100/Test |
| --- | --- |

D-14. Application Rule configuration (xy = 00 ~ 19)

| application_timeout | default = 10 (mins) |
| --- | --- |
| application_xy | enable(0¦1)/name/lanip/trigger_prot/trigger_ports/public_prot/public_ports/schedule_rule_name ex. 1/app1/192.168.0.100/UDP/2000-2012/UDP/2000-2006/Always 0/app2/Any/UDP/1000-1006/TCP/1002,1005,1020-1024/Test |
| application_enable | 1 (0/1) |

D-15. Mac Filter (xy = 00 ~ 19)

| mac_filter_type | disable¦list_allow¦list_deny/schedule_rule_00_name |
| --- | --- |
| mac_filter_xy | ex. 00:01:02:03:04:00/Always |

D-16. Filter configuration (xy = 00 ~ 19)

| filter_enable | enable(0¦1) |
| --- | --- |
| filter_xy | enable(0¦1)/name/local_ipaddr_range/remote_ipaddr_range/remote_prot/remote_port_range/schedule_rule_00_name ex. 1/filter1/192.168.0.102-192.168.0.104/168.95.1.1-168.95.1.2/TCP/80-80/Always 0/filter2/192.168.0.102-192.168.0.102/168.95.1.1-168.95.1.1/UDP/63-63/Test |

D-17. URL/Domain filter (xy = 00 ~ 39)

| url_domain_filter_type | disable¦list_allow¦list_deny |
| --- | --- |
| url_domain_filter _xy | ex. www.yahoo.com www.nba.com/index.htm |
| url_domain_filter _schedule_xy |  |
| url_domain_filter_trust_ip_xy | Disable/ipaddr[-ipaddr,ipaddr] Ex: 0/192.168.1.10 |
| url_domain_filter_enable_xy | (0¦1) |

D-18. RIP 1/2 configuration

| rip_enable | (0¦1) |
| --- | --- |
| rip_rx_version | (0¦1¦2) |
| rip_tx_version | (0¦1¦2) |
| rip_v2_type | (B¦M) |

D-19. Static Routing configuration (xy = 00 ~ 39)

| static_routing_xy | enable/name/dest_addr/dest_mask/gateway/interface/metric ex. 0/name/192.168.31.0/255.255.255.0/192.168.31.1/LAN¦WAN¦WAN_PHY/20 |
| --- | --- |
| static_routing_private | (0¦1) ex: when static_routing_private set 1. static_routing_xy set as following 1/name/192.168.31.0/255.255.255.0/192.168.31.1/br0/20 when static_routing_private set 0. static_routing_xy set as following 1/name/192.168.31.0/255.255.255.0/192.168.31.1/eth1/20 |

D-20. Time and NTP configuration

| time_zone | Integer |
| --- | --- |
| time_zone_area | (0-12) ex: Taipei = 1, HongKong = 2…etc. |
| time_daylight_saving_enable | Enable(0¦1) |
| time_daylight_saving_start_month | (1-12) |
| time_daylight_saving_start_week | (1-5) |
| time_daylight_saving_start_day_of_weeek | (1-7) |
| time_daylight_saving_start_time | (0-23) |
| time_daylight_saving_end_month | (1-12) |
| time_daylight_saving_end_week | (1-5) |
| time_daylight_saving_end_day_of_week | (1-7) |
| time_daylight_saving_end_time | (0-23) |
| ntp_client_enable | enable(0¦1) |
| ntp_sync_interval | (hr) ex. 12 |
| ntp_server | Ipaddr ¦ hostname |
| ntp_default_server | Ipaddr ¦ hostname ex: 192.168.3.1, ntp.ms.com, ntp.cameo.com, 182.1.1.3…. |

D-21. Misc configuration

| wan_port_ping_response_enable | enable(0¦1) |
| --- | --- |
| remote_http_management_enable | enable(0¦1) |
| remote_http_management_port | Ex. 80 8080 |
| remote_http_management_ipaddr_range | Ex. 0.0.0.0 172.168.0.1-172.168.0.10 |
| remote_http_management_schedule_name | Ex All |
| upnp_enable | enable(0¦1) |
| wan_port_speed | auto¦10half¦10full¦100half¦100full¦giga |
| multicast_stream_enable | enable(0¦1) |

D-22. DDNS configuration

| ddns_enable | enable(0¦1) |
| --- | --- |
| ddns_type | DynDns.org¦EasyDns.com¦No-IP.com |
| ddns_hostname | Hostmane |
| ddns_username | Username |
| ddns_password | Password |
| ddns_wildcards_enable | enable(0¦1) |
| ddns_init_ip | ex. (the init wan ip address) 192.168.0.250 |
| ddns_type_old |  |
| ddns_hostname_old |  |
| ddns_username_old |  |
| ddns_password_old |  |
| ddns_wildcards_enable_old |  |
| ddns_dyndns_kinds | Dynamic ¦ Static ¦ Custom |

D-23. DMZ configuration

| dmz_enable | enable(0¦1) |
| --- | --- |
| dmz_ipaddr | ex. 192.168.0.250 |
| dmz_schedule |  |

D-24. VPN Pass Through configuration

| pptp_pass_through | enable(0¦1) |
| --- | --- |
| ipsec_pass_through | enable(0¦1) |
| l2tp_pass_through | enable(0¦1) |

D-25. Log configuration

| log_system_activity | enable(0¦1) |
| --- | --- |
| log_debug_information | enable(0¦1) |
| log_attacks | enable(0¦1) |
| log_dropped_packets | enable(0¦1) |
| log_notice | enable(0¦1) |
| log_email_sender | ex:19nderson@mail.cameo.com.tw |
| log_email_server | ex:smtp.mail.cameo.com.tw / xxx.xxx.xxx.xxx |
| log_email_recipien | ex:D-Link |
| log_email_username |  |
| log_email_password |  |
| log_per_page |  |
| log_current_page | 0, 1, 2, … |
| log_response_type | system¦debug¦¦attack¦dropped¦notice |
| log_email_enable | enable(0 ¦ 1) |
| log_email_block_enable | enable(0 ¦ 1) |
| log_email_schedule | type/time/day (type = full ¦ none ¦ daily ¦ hourly ¦ weekly, time = 0 – 23, day = 0 – 6) ex:0/10/0 |
| log_email_auth | enable(0 ¦ 1) |
| syslog_server | enable(0 ¦ 1)/Ipaddr , ex. 1/192.168.0.100 |

D-26. Wireless configuration

| wlan0_eth | Ex:eth0, eth1, ap0…etc |
| --- | --- |
| wlan0_enable | enable(0¦1) |
| wlan0_radio_enable | enable(0¦1) |
| wlan0_emi_test | enable(0¦1) |
| wlan0_ssid |  |
| wlan0_domain | 0x10 |
| wlan0_channel |  |
| wlan0_auto_channel_enable | enable(0¦1) |
| wlan0_dot11_mode | 11b¦11g¦11bg¦11bgn¦11n |
| wlan0_ssid_broadcast | enable(0¦1) |
| wlan0_mode | ap ¦ sta ¦ repeat ¦ bridge ¦ mbridge ¦ adhoc |
| wlan0_idle_time | (minutes) |
| wlan0_security | disable¦wep_open_64¦wep_open_128¦wep_open_152¦wep_share_64¦wep_shaer_128¦wep_share_152¦wpa_psk¦wpa_eap¦wpa2_psk¦wpa2_eap¦wpa2auto_psk¦wpa2auto_eap¦wpawpa2_eap |
| wlan0_wep64_key_x x = 1 ~ 4 | 0123456789 |
| wlan0_wep128_key_x x = 1 ~ 4 | 01234567890123456789012345 |
| wlan0_wep152_key_x x = 1 ~ 4 | 01234567890123456789012345678901 |
| wlan0_wep_display | hex¦ascii |
| wlan0_wep_default_key | 1¦2¦3¦4 |
| wlan0_psk_cipher_type | tkip¦aes¦both |
| wlan0_psk_pass_phrase | pass_phrase |
| wlan0_eap_radius_server_x x = 0, 1 | hostname¦ipaddr/port/secret |
| wlan0_wpa_groupkey_rekey_tim | (minutes) |
| wlan0_auto_txrate | 0 = fix rate ¦ 1 = auto rate |
| wlan0_11g_txrate | 6¦9¦12¦18¦24¦36¦48¦54 |
| wlan0_11b_txrate | 1¦2¦5.5¦11 |
| wlan0_11bg_txrate | 1¦2¦5.5¦6¦9¦11¦12¦18¦24¦36¦48¦54 |
| wlan0_antenna_diversity | auto¦0¦1¦2¦3 |
| wlan0_txpower | 19¦18¦17¦16¦15¦14¦13¦12¦11¦10 ….1 |
| wlan0_beacon_interval |  |
| wlan0_rts_threshold |  |
| wlan0_fragmemtation |  |
| wlan0_dtim |  |
| wlan0_short_preamble | enable(0¦1) |
| wlan0_cts | enable(0¦1) |
| wlan0_wmm_enable | enable(0¦1) |
| wlan0_wmm_11n_ap_x x= 0 ~ 3 | log_cw_min/log_cw_max/aifs/tx_op_limit_b/tx_op_limit_g/acm/ack_policy wlan0_wmm_ap_0=4/6/3/0/0/0/0 wlan0_wmm_ap_1=4/10/7/0/0/0/0 wlan0_wmm_ap_2=3/4/1/3008/3008/0/0 wlan0_wmm_ap_3=2/3/1/1504/1504/0/0 |
| wlan0_wmm_ap_x x = 0 ~ 3 | log_cw_min/log_cw_max/aifs/tx_op_limit_b/tx_op_limit_g/acm/ack_policy ex. Wlan0_wmm_ap_0=4/6/3/0/0/0/0 wlan0_wmm_ap_1=4/10/7/0/0/0/0 wlan0_wmm_ap_2=3/4/1/3008/3008/0/0 wlan0_wmm_ap_3=2/3/1/1504/1504/0/0 |
| wlan0_wmm_bss_x x = 0 ~ 3 | log_cw_min/log_cw_max/aifs/tx_op_limit_b/tx_op_limit_g/acm ex. Wlan0_wmm_bss_0=4/6/3/0/0/0 wlan0_wmm_bss_1=3/10/7/0/0/0 wlan0_wmm_bss_2=2/4/1/3008/3008/0 wlan0_wmm_bss_3=2/3/1/1504/1504/0 |
| wlan0_11n_protection | disable¦20¦20_40¦auto |
| wlan0_11n_txrate | MCS0¦MCS1¦MCS2¦MCS3¦MCS4¦MCS5¦MCS6¦MCS7¦MCS8¦MCS9¦MCS10¦MCS11¦MCS12¦MCS13¦MCS14¦MCS15 |
| wlan0_amsdu | disable¦4k¦8k |
| wlan0_channel_bandwidth | auto¦20¦40 |
| wlan0_guard_interval | auto¦short¦long |
| wlan0_ext_subchannel | auto¦lower¦upper |
| wlan0_antenna_control | auto = 0, 1 = one antenna, 2 = two antenna, 3 = three antenna. |
| Wlan0_vapx_enable x=1~3 | enable(0¦1) |
| wlan0_vapx_ssid x= 1~3 |  |
| wlan0_xr_enable | enable(0¦1) |
| wlan0_adptive_radio_enable | enable(0¦1) |
| wlan0_vapx_ssid_broadcast x =1 ~3 | enable(0¦1) |
| wlan0_vapx_security x= 1~3 | disable¦wep_open_64¦wep_open_128¦wep_open_152¦wep_share_64¦wep_shaer_128¦wep_share_152¦wpa_psk¦wpa_eap¦wpa2_psk¦wpa2_eap¦wpa2auto_psk¦wpa2auto_eap |
| wlan0_vapy_wep64_key_x x = 1 ~ 4 y =1 ~3 | 0123456789 |
| wlan0_vapy_wep128_key_x x = 1 ~ 4 y= 1~3 | 01234567890123456789012345 |
| wlan0_vapy_wep152_key_x x = 1 ~ 4 y=1~3 | 01234567890123456789012345678901 |
| wlan0_vapx_wep_display x=1~3 | hex¦ascii |
| wlan0_vapx_wep_default_key x = 1~3 | 1¦2¦3¦4 |
| wlan0_vapx_psk_cipher_type x =1~3 | tkip¦aes¦both |
| wlan0_vapx_psk_pass_phrase x = 1~3 | pass_phrase |
| wlan0_vapx_auto_txrate x=1~3 | 0 = fix rate ¦ 1 = auto rate |
| wlan0_vapx_11b_txrate x=1~3 | 1¦2¦5.5¦11 |
| wlan0_vapx_11g_txrate x=1~3 | 6¦9¦12¦18¦24¦36¦48¦54 |
| wlan0_vapx_11bg_txrate x=1~3 | 1¦2¦5.5¦6¦9¦11¦12¦18¦24¦36¦48¦54 |
| wlan0_vapx_beacon_interval x=1~3 |  |
| wlan0_vapx_rts_threshold x=1~3 |  |
| wlan0_vapx_fragmemtation x=1~3 |  |
| wlan0_vapx_dtim x=1~3 |  |
| wlan0_vapx_cts x=1~3 | enable(0¦1) |
| wlan0_vapx_wmm_enable x=1~3 | enable(0¦1) |
| wlan0_vapy_wmm_ap_x x = 0 ~ 3 y= 1 ~ 3 | log_cw_min/log_cw_max/aifs/tx_op_limit_b/tx_op_limit_g/acm/ack_policy ex. Wlan0_wmm_ap_0=4/6/3/0/0/0/0 wlan0_wmm_ap_1=4/10/7/0/0/0/0 wlan0_wmm_ap_2=3/4/1/3008/3008/0/0 wlan0_wmm_ap_3=2/3/1/1504/1504/0/0 |
| wlan0_vapy_wmm_bss_x x = 0 ~ 3 y= 1 ~ 3 | log_cw_min/log_cw_max/aifs/tx_op_limit_b/tx_op_limit_g/acm ex. Wlan0_wmm_bss_0=4/6/3/0/0/0 wlan0_wmm_bss_1=3/10/7/0/0/0 wlan0_wmm_bss_2=2/4/1/3008/3008/0 wlan0_wmm_bss_3=2/3/1/1504/1504/0 |
| wlan0_super_g | static¦dynamic-turbo¦without-turbo¦disable |
| wlan0_vapx_super_g x = 1~3 | static¦dynamic-turbo¦without-turbo¦disable |
| wlan0_protection | enable(0¦1) |
| wlan0_vapx_protection x = 1~3 | enable(0¦1) |
| wlan0_wds_enable | enable(0¦1) |
| wlan0_vapx_wds_enable x = 1~3 | enable(0¦1) |
| wlan0_tls_passwd |  |
| wlan0_eap_proto | tls¦ttls¦peap |
| wlan0_ttls_inner_proto | mschapv2¦pap |
| wlan0_peap_inner_proto | mschapv2¦gtc |
| wlan0_ttls_login_name |  |
| wlan0_ttls_passwd |  |
| wlan0_peap_login_name |  |
| wlan0_peap_passwd |  |
| wlan0_wds_mac | ex:00:11:22:33:44:55/00:22:33:44:55:66/../../../../../..( 8組) |

D-27. UI blank_status (UI specific)

| blank_status |  |
| --- | --- |
| help_flag |  |

D-28. Temperary for UI, will be saved to flash, but not exported to users. During booting, these value should be cleared.

| Asp_temp_wep_display | hex or ascii |
| --- | --- |
| asp_temp_xy xy = 00 ~ 99 |  |
| asp_temp_wep64_key_x x = 1 ~ 4 |  |
| asp_temp_wep128_key_x x = 1 ~ 4 |  |
| asp_temp_wep152_key_x x = 1 ~ 4 |  |
| html_response_page | When user presses apply, httpd should follow the value to response |
| html_response_message | Response page may have one message |
| html_response_return_page | Response page may return to this page. |

D-29. UpnP configuration value, for upnp using.

| Friendlyname |  | ex:D-Link Router |
| --- | --- | --- |
| manufacturer |  | ex:D-Link |
| manufacturer_url |  | ex:http://www.dlink.com |
| model_name |  | ex:D-Link Router |
| model_number | ex:2.0 |  |
| model_url | ex:http://support.dlink.com |  |
| upnp_advertisement_period |  | (1-1440) |
| upnp_advertisement_ttl |  | (1-255) |

D-30. SPI configuration value

| spi_enable | (0 ¦ 1) |
| --- | --- |

D-31. Block Services configuration value

| block_service_xy xy = 00 ~ 09 | enable/name/wan_proto/start_port/end_port/start_ip/end_ip/schedule_name ex:0/name/both/0/0/0.0.0.0/0.0.0.0/always |
| --- | --- |

D-32. SNMPv3 configuration value

| snmpv3_enable | enable(0¦1) |
| --- | --- |
| snmpv3_security_user_name | ex:jackey |
| auth_protocol | none/md5/sha1 |
| auth_password | ex:11111111 (at least 8 characters) |
| privacy_protocol | none/des/aes |
| privacy_password | ex:22222222 (at least 8 characters) |

D-33 Console (telnet) configuration value

| console_enable | (0 ¦ 1) |
| --- | --- |
| console_timeout | default = 3 (mins) |

D-34 Firmware value

| firmware_available | (0 ¦ 1) (read only) |
| --- | --- |
| login_auto_upgrade | (0 ¦ 1) |

D-35. QoS configuration value

| qos_enable | (0¦1) |
| --- | --- |
| qos_uplink | (0-99999) |
| qos_downlink | (0-99999) |
| qos_rule_xy xy = 00 ~ 19 | enable/name/priority/uplink/downlink/protocol/local_start_ip/local_end_ip/local_start_port/local_end_port/local_mac_address/remote_start_ip/remote_end_ip/remote_start_port/remote_end_port ex. 1/http/2/256/2048/6/0.0.0.0/255.255.255.255/00:00:00:00:00:00/80/80/0.0.0.0/255.255.255.255/80/80 |

D-36 NAT

| nat_filter | 0 : open(disable) 1 : secured |
| --- | --- |

D-37 Pure Network

| pure_type | Ex: GatewayWithWiFi |
| --- | --- |
| pure_device_name | Ex: My Favourite Device |
| pure_vendor_name | Ex: Netgear |
| pure_model_description | Ex: Wireless 108G MIMO Router |

D-38 POT (read only for HTTP)

| POT |  |
| --- | --- |
| POT_Backup |  |
| POT_Resolution | (default: 1) set to 1, then POT value will be incremented by 1 in 1 minute |
| POT_MaxValue | (1440¦4320) Current value is set to 4320 (3 days) |

D-39 WPS

| wps_enable | (0¦1) |
| --- | --- |
| wps_configured_mode | (1-5) |
| wps_pin | 8 characters(ex.12345670) and read only |
| wps_default_pin | 8 characters(ex.12345670) and read only |
| wps_lock | (0¦1) |
| wps_sta_enrollee_pin | 8 characters(ex.12345670) |

D-40 TFTP server

| tftpd_enable | (0¦1) |
| --- | --- |
| tftpd_hardwareId_check | (0¦1) |

D-41 Smart Wizard

| wizard_flag | (0¦1) ex: the wizard_flag set 1 when user use CD to trigger off router. The wizard_flag set 0 noramlly. |
| --- | --- |

D-42. Firewall rule configuration (xy = 00 ~ 49)

| firewall_rule_xy | enable(0¦1)/name/iface_src/TCP¦UDP¦ICMP¦Any/src_ip_start/src_ip_end/iface_dest/dest_ip_start/dest_ip_end/dest_port_start/dest_port_end/schedule/action ex. 1/rule1/LAN/TCP/192.168.0.1/192.168.0.10/WAN/17.0.0.1/17.0.0.10/80/90/Always/Allow |
| --- | --- |

- Status

  - UI (WEB pages) may access via CmoGetStatus(“name”)

| wan_current_ipaddr_xy xy = 00 ~ 04 | ipaddr/netmask/gateway/dns1/dns2 ex. 172.21.65.150/255.255.255.0/172.21.65.254/192.168.2.1/192.168.3.1 |
| --- | --- |
| Version | 0.89 build 32 |
| version_date | 2006, 3, 17 |
| wireless_driver_info | Marvell 2.0.3.p4 |
| wireless_domain | 0x10/FCC |
| wireless_mac | 00:05:12:34:56 |
| lan_tx_packets |  |
| lan_rx_packets |  |
| lan_lost_packets |  |
| wan_tx_packets |  |
| wan_rx_packets |  |
| wan_lost_packets |  |
| wlan_tx_packets |  |
| wlan_rx_packets |  |
| wlan_lost_packets |  |
| dhcpc_connection_status |  |
| pppoe_xy_connection_status xy = 00 ~ 04 | contected/conntecting/disconnect |
| l2tp_connection_status | contected/conntecting/disconnect |
| bigpond_connection_status | connected/connecting/disconnect |
| pptp_connection_status | contected/conntecting/disconnect |
| get_current_user | admin/user |
| mac_clone_addr | 00:11:22:33:44:55 |
| system_time | year/month/day/hour/min/year ex. 2006/11/23/22/10 |
| wlan0_channel_list | 1,2,3,4…13 (list each channel) |
| log_total_page |  |
| wan_port_status | connect/disconnect |
| wan_port_speed_duplex | 10/100/1000 ¦ half/full Ex. 100, full |
| lan_port_status_xy xy = 00 ~ 03 | Connect/disconnect |
| lan_port_speed_duplex_xy xy = 00 ~ 03 | 10/100/1000 ¦ half/full Ex. 100, full |
| current_channel | Ex. 1,2,3….12 |
| usb_test_result | Pass / Fail |
| dhcpc_obtain_time | (sec) |
| dhcpc_expire_time | (sec) |
| wan_collision |  |
| lan_collision |  |
| wlan_collision |  |
| wan_uptime | (sec) |
| lan_uptime | (sec) |
| wlan_uptime | (sec) |
| lan_tx_bytes |  |
| lan_rx_bytes |  |
| wan_tx_bytes |  |
| wan_rx_bytes |  |
| wlan_tx_bytes |  |
| wlan_rx_bytes |  |
| wlan_rx_frames |  |
| wlan_tx_frames |  |
| internet_connect |  |
| ddns_status | - ddns disable return ”Dynamic DNS service is not enabled.” - not get IP  return “No update action” - with IP but server has no response return “time¦date” - auth failurereturn “BAD AUTH” - host look up failurereturn “BAD HOST” - successreturn “SUCCESS¦time¦date” |
| Intel_viiv | DeviceID;/ HardwareVersion;/ FirmwareVersion;/ WLAN-Congigurations……. |
| Firmware_check_status |  |
| ppp_auth_type | PAP¦CHAP¦MS-CHAP¦MS-CHAP-v2 |
| pptp_uptime |  |
| dhcp_server_ip |  |
| wps_generate_pin |  |
| wps_generate_default_pin |  |

- List

  - UI (WEB pages) may access via CmoGetList(“name”)

| routing_table | dest_addr/dest_mask/gateway/interface/metric |
| --- | --- |
| dhcpd_leased_table | hostname/ip_addr/mac_addr/expired_at |
| wireless_station_list | name1/mac1,name2/mac2,… |

- CGI

| apply | apply.cgi name=reboot_type, value = hardware ¦ all ¦ lan ¦ wan ¦ wireless ¦ application ¦ filter ¦ none, default is all |
| --- | --- |
| system_time | system_time.cgi name=date value = year/month/day/hour/min/second ex. 2006/3/12/16/31/55 |
| dhcp_release | dhcp_release.cgi |
| dhcp_renew | dhcp_renew.cgi |
| pppoe_xy_connect xy = 00 ~ 04 | pppoe_xy_connect.cgi |
| pppoe_xy_disconnect xy = 00 ~ 04 | pppoe_xy_disconnect.cgi |
| l2tp_connect | l2tp_connect.cgi |
| l2tp_disconnect | l2tp_disconnect.cgi |
| pptp_connect | pptp_connect.cgi |
| pptp_disconnect | pptp_disconnect.cgi |
| bigpond_connect | bigpond_connect.cgi |
| bigpond_disconnect | bigpond_disconnect.cgi |
| ping_response | ping_response.cgi name=”host” |
| mac_clone | mac_clone.cgi |
| restore_default | restore_default.cgi |
| reboot | reboot.cgi |
| save_configuratoin | save_configuration.cgi (response: config.txt) |
| upload_configuration | upload_configuratoin.cgi |
| Firmware _upgrade | firmware_upgrade.cgi name=file |
| send_log_email | send_log_email.cgi |
| log_first_page | log_first_page.cgi |
| log_last_page | log_last_page.cgi |
| log_next_page | log_next_page.cgi |
| log_previous_page | log_previous_page.cgi |
| vct_wan | vct_wan.cgi ex. disconnect at xx m / connected |
| vct_lan_xy xy = 00 ~ 03 | vct_lan_xy.cgi ex. disconnect at xx m / connected |
| reset_packet_counter | reset_packet_counter.cgi reponse_page |
| save_log | save_log.cgi reponse_page |
| usb_test | usb_test.cgi |
| check_wan_connection | check_wan_connection.cgi |
| apply_config (only set) | apply_config.cgi (co-operate with apply_confirm & apply discard) |
| apply_confirm (only commit & rc) | apply_confirm.cgi (co-operate with apply_config & apply_discard) |
| apply_discard.cgi | apply_discard.cgi (co-operate with apply_config & apply_confirm) |

- Log, may access via CmoGetLog()

Format: |syslog|time/type/message

|syslog|time/type/message…..

- others