Date: Dec. 20,1999

To: All Locations

Subject:  New Product Release for DES-1016R/1024R

10/100Mbps Unmanaged Switches

D-Link is pleased to make this formal announcement for the following new product.

Please start to promote for this new product and advise if we can be helpful in any aspects.

## MODEL NO.:	DES-1016R/1024R Rev.A1

PART NO.:  DES1016R..A1. DES1024R..A1

10/100Mbps Switches for Workgroups

These unmanaged 10/100Mbps switches are designed to enhance workgroup performance while providing a high level of flexibility.  Powerful yet easy to use, these devices allow users to simply plug any port to either a 10Mbps or 100Mbps network to multiply bandwidths, boost response time and satisfy heavy load

demands.

Choices of 16 and 24 Ports

Two choices are available: one with 16 10/100Mbps ports, the other with 24 10/100Mbps ports.  These ports detect the network speed and auto-negotiate between 100BASE-TX and 10BASE-T, as well as full- and half-duplex.

Flow Control for Secure Transmission

In the fast 200Mbps full-duplex mode, the switch's embedded flow control provides a means to protect users against possible data loss during transmission on the network.  When connected to a LAN adapter (server PC) that supports flow control, the switch sends signals to the server regarding the switch's buffer overrun during peak usage periods.  The server then delays transmission until the switch is ready again to accept new data.

Easy Expansion

An MDI uplink port supporting 10/100Mbps connection is provided, allowing the workgroup to easily expand.  2 switches can be cascaded together through this MDI uplink to increase the port density.  Running at 200Mbps full-duplex speed, this port provides a fast switch-to-switch dedicated bandwidth.

Workgroup Plug-and-Play

With 16 to 24 plug-and-play ports, the switches are perfect choices for workgroups to upgrade performance in a client/server environment.  The ports can be connected to servers in full-duplex, or to hubs in half-duplex.

Power Users

With low-cost connection per-port, the switches can also be set up for direct connection from the PCs.  This relieves data bottleneck by giving each workstation a dedicated bandwidth on the network.

Key Features

- 16 or 24 10/100Mbps ports per switch

- 1 MDI uplink port per switch

- All ports support full/half-duplex

- Dynamic buffer allocation for each port

- IEEE 802.3x flow control for full-duplex

- Back-pressure for half-duplex

- Wire-speed filtering/forwarding rates

- Auto-learning of network configuration

- Auto-correction of reverse twisted-pair polarity

Technical Specifications

General

Standards

- IEEE 802.3 10BASE-T Ethernet

- IEEE 802.3u 100BASE-TX Fast Ethernet

- ANSI/IEEE 802.3 NWay auto-negotiation

Protocol

CSMA/CD

Data Transfer Rates

- Ethernet:

10Mbps (half duplex)

20Mbps (full duplex)

- Fast Ethernet:

100Mbps (half duplex)

200Mbps (full duplex)

Topology

Star

Network Cables

- 10BASE-T:

2-pair UTP Cat. 3, 4, 5 (100 m)

- 10BASE-T, 100BASE-TX:

EIA/TIA-586 100-ohm screened twisted-pair (STP) (100 m)

- 100BASE-TX:

2-pair UTP Cat. 5 (100 m)

Number of Ports

- DES-1016R: 10/100Mbps port x 16

- DES-1024R: 10/100Mbps port x 24

Media Interface Exchange

MDI RJ-45 x 1 (shared with 1 10/100Mbps ports)

Twisted-pair Rx Reverse Polarity

Auto-correction

LED Report

Per device:

- Power

Per port:

- 100Mbps speeds

- Link/activity

Performance

Transmission Method:

Store-and-forward

Filtering Address Table

1K entries per device

MAC Address Learning

- Automatic update

- Max Age: 5 minutes

Packet Filtering Rates

- 10BASE-T: 14,880 pps per port (half-duplex)

- 100BASE-TX: 148,800 pps per port (half-duplex)

Packet Forwarding Rates

- 10BASE-T: 14,880 pps per port (half-duplex)

- 100BASE-TX: 148,800 pps per port (half-duplex)

RAM Buffer

- DES-1016R: 2MB per device

- DES-1024R: 3MB per device

- Dynamic allocation for each port

Physical & Environmental

AC Input

100  -  240 VAC, 50/60 Hz

Internal universal power supply

Power Consumption

40 watts (max.)

Operating Temperature

0°   -  50° C

(32°  -  122°F )

Storage Temperature

-30°  -  60°C

(-22°  -  140°F )

Humidity

5%  -  95% non-condensing

Dimensions

441 x 207 x 44 mm

(17.4 x 8.15 x 1.732 inches)

19-inch rack-mount width, 1 U height

Weight

DES-1016R: 2.5 kg (5.5 lb.)

DES-1024R: 3 kg (6.6 lb.)

Emission (EMI)

- FCC Class A

- CE Mark

- C-Tick Class A

- VCCI Class A

- BSMI Class A

Safety

- UL (UL1950)

- CSA (CSA950)

- TUV/GS (EN60950)

Pilot Run sample shipment Date:

DES-1016R:  The end of November, 1999

DES-1024R:  The end of October, 1999

Mass Production Date: 
  DES-1016R/DES-1024R:   March 1, 2000

Ordering Information:

Ethernet/Fast Ethernet Switches

DES-1016R      16 auto-sensing 10/100Mbps ports

DES-1024R      24 auto-sensing 10/100Mbps ports

Ordering Price:

Please check with each contact window in PM, D-Link