DES-128

8-port 10/100Base-TX Switch Module

DES-121T

1-port Gigabit 1000Base-T Switch Module

DES-124F

4-port 100Baes-FX Fiber Module

DES-121G

1-port Gigabit 1000Base-SX Fiber Module

User’s Guide

Rev. 03 (MAR. 2001)

6012-9830016

Printed In Taiwan

RECYCLABLE

# Table of Contents

Table of Contents	i

DES-128 8-port 10/100M Switch Module	1

Features	1

DIP-Switch with Link Mode	1

LED Indicators	2

DES-121T Gigabit 1000Base-T Switch Module	4

Features	4

LED Indicators	4

DES-124F 100Base-FX Fiber Module Series	6

Features:	6

DES-121G Gigabit 1000Base-X Fiber Modules	7

DES-121G Gigabit 1000Base-SX Module	7

Features	7

LED Indicators	7

Technical Specifications	9

# DES-128 8-port 10/100M Switch Module

Figure 1. 8-port 10/100 N-Way Module Front View

## Features

  8x auto-sensing 10/100Mbps Ethernet switch ports

- Automatic MDI/MDIX crossover for 10Base-T and 100Base-TX ports

- Auto-negotiation supported

- Back-Pressure-Base flow control on Half-duplex ports

- Pause-Frame-Base flow control on Full-duplex ports

- Store-and-forward technology for abnormal packet filtering

  Non-blocking and Full wire speed forwarding rate

  4K-entry MAC Address Table

  LED-indicators for 100M, LK/ACT, FD/COL status

## DIP-Switch with Link Mode

The DES-128 8-port 10/100-TX switch module provides DIP-switches from 1st to 4th to adjust link mode with other network devices. Each port was configured by 2 DIPs, so there are 8 DIPs to set link mode of the first 4 ports. There are three types of link mode can be chosen, which include Auto- negotiation, 100Mbps/Full-duplex and 10Mbps/Full-duplex. The last 4 ports was not designed DIP-switches to do any link mode setting, but these ports support auto-negotiation protocol only.

Figure 2. DIP-switch location and mode settings

If you set DIP to Auto-negotiation, then the other DIP for 100Mbps and 10Mbps is invalid. If you set DIP to Full-duplex, then to set the other DIP between 100Mbps and 10Mbps mode.

## LED Indicators

There are three types of LED indicators for each of the RJ-45 Ports. If there is no connection to a port, the corresponding LED indicators are not lit.

- 100M   The LED indicator is green when the corresponding port is connected to a 100Mbps devices. This LED indicator is not lit when the corresponding port is connected to a 10Mbps.

- LK/ACT   The LED indicator is green when there is a connection secured to a device ( Link ) to the corresponding port . This LED indicator blinks when there is a data transmission ( Activity ) taking place at the corresponding port.

- FD/COL The LED indicator is yellow when the corresponding port is operating in Full-duplex mode. When the corresponding port is operating in Half-duplex mode, the LED indicator is not lit.  When collisions are occurring at the corresponding port, the LED indicator blinks.

# DES-121T Gigabit 1000Base-T Switch Module

Figure 3. Gigabit 1000Base-T Switch Module Front View

## Features

- Compliant Gigabit Media Independent Interface (GMII )

- 1 X 100/1000Mbps N-Way auto-negotiation switch port

- Automatic MDI crossover function

- Up to 2.4Gbps Backplane forwarding rate

- 3MB memory buffer sharing

- 12K-entry Mac Address Table

- Store-and-forward technology for abnormal packet filtering

- Half-duplex and full-duplex modes supported

- LED-indicators for 1000M Link, 100M Link, Activity, and Full-duplex statuses

## LED Indicators

The are 4 LED indicators for one RJ-45 connection port. If there is no connection to a port, the corresponding LED indicators are not lit.

- LK1000  The LED indicator is green when the Port is linking with 1000Mbps mode. When the port is operating in 10Mbps/100Mbps mode or not connected, the LED indicator is not lit.

- ACT  This LED indicator blinks green when the port is transmitting or receiving packets. When the device is not attached, the LED indicator is not lit.

- LK100  The LED indicator is green when the Port is linking with 100Mbps mode. When the port is operating in 10Mbps mode or not connected, the LED indicator is not lit.

- FD  The LED indicator is yellow when the port is operating in Full-duplex mode. When the port is operating in Half-duplex mode, the LED indicator is not lit.

# DES-124F 100Base-FX Fiber Module Series

Figure 4.  4 port 100Base-FX ( SC ) Module Front View

## Features:

  4 X 100Mbps fiber ports (SC connectors)

  Up to 2.4Gbps Backplane forwarding rate

  5MB memory buffer

  12K-entry MAC Address Table

  Store-and-forward technology for abnormal packet filtering

  Half-duplex and full-duplex modes supported

  DIP-switches to set between 100M Full-duplex and 100M Half-duplex modes

  LED-indicators for LK/ACT, FD/COL statuses

  For establishing fiber links cable distance up to 2 Km (multi-mode) or 60 Km (single-mode)

# DES-121G Gigabit 1000Base-X Fiber Modules

## DES-121G Gigabit 1000Base-SX Module

Figure 9.  Gigabit 1000Base-SX Fiber Module Front View

## Features

  1x 1000Mbps fiber port ( SC multi-mode connectors )

  4K MAC address table

  Support store-and-forward switching architecture

- Standard auto-negotiation for Speed, Duplex mode and flow control

- LED-indicators for Link, Activity, Full-duplex, Collision

- For fiber links cable distance up to 500 m ( use 50/125um MMF ), 220 m (use 62.5/125um MMF ).

## LED Indicators

The are four LED indicators for one Gigabit Fiber connection port. If there is no connection to a port, the corresponding LED indicators are not lit.

- LK  The LED indicator is green when there is a connection securely to a device ( Link ) to the corresponding port.

- ACT  This LED indicator blinks green when there is a data transmission ( Activity ) taking place at the corresponding port.

- FD  The LED indicator is yellow when the corresponding port is operating in Full-duplex mode. When the corresponding port is operating in Half-duplex mode, the LED indicator is not lit.

- COL  The LED indicator blinks yellow when collisions are occurring at the corresponding port

# Technical Specifications

| Standards Compliance | IEEE 802.3 10BASE-T Ethernet, IEEE 802.3u 100BASE-TX/FX Fast Ethernet, IEEE 802.3z 1000BASE-X Fiber Gigabit Ethernet IEEE 802.3ab 1000BASE-T UTP Gigabit Ethernet ANSI/IEEE standard 802.3 N-Way Auto-negotiation |
| --- | --- |
| Protocol | CSMA/CD |
| Max Forwarding and Max Filtering Rate | 14,880 pps to Ethernet port, 148,800 pps to Fast Ethernet port 1,488,000pps to Gigabit Ethernet port |
| LED-Indicators | Modules: 8 port N-Way: Per Port:100M, LK/ACT, FD/COL ( 3 LEDs ) 100M Fiber: LK/ACT, FD/COL ( 2 LEDs ) Gigabit Fiber: LK, ACT, FD, COL ( 4 LEDs )100 LK, ACT, FD ( 4 LEDs ) Gigabit UTP: 1000 LK, 100 LK, ACT, FD ( 4 LEDs ) |
| Network Cables | 10Base-T: 2-pair UTP/STP Cat. 3, 4, 5 cable EIA/TIA-568 (100m ) 100Base-TX: 2-pair UTP/STP Cat. 5 cable EIA/TIA-568 (100m ) 1000Base-T: 4-pair UTP/STP Cat. 5 or above EIA/TIA-568 (100m ) 100Base-FX: 50, 62.5/125 micron multi-mode fiber-optics ( 2Km) 8,9/125 micron single-mode fiber-optics ( 60Km ) 1000Base-X: 50/125 micron multi-mode fiber-optics ( 500m ) 10/125 micron single-mode fiber-optics (10Km ) |
| Dimensions | 178 mm x 152 mm x 25 mm ( L x W x H ) |
| Weight | 213 ~ 283 g |
| Operating Temperature | 0ºC to 45ºC (32ºF to 113ºF) |
| Operating Humidity | 10% to 90% (Non-condensing) |
| Power Supply | Modular Unit : Input rate: 100~240VAC, 50~60Hz |
| EMI | FCC Class A, CE Mark |

- FCC Certifications

This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to Part 15 of the FCC Rules.  These limits are designed to provide reasonable protection against harmful interference in a residential installation.  This equipment generates, uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications.  However, there is no guarantee that interference will not occur in a particular installation.  If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

- Reorient or relocate the receiving antenna.

- Increase the separation between the equipment and receiver.

- Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

- Consult the dealer or an experienced radio/TV technician for help.

Shielded interface cables must be used in order to comply with emission limits.

You are cautioned that changes or modifications not expressly approved by the party responsible for compliance could void your authority to operate the equipment.

This device complies with Part 15 of the FCC rules.  Operation is subject to the following two conditions:  (1) This device may not cause harmful interference, and (2) This device must accept any interference received, including interference that may cause undesired operation.

#### CE Mark Warning

This is a Class B product. In a domestic environment, this product may cause radio interference, in which case the user may be required to take adequate measures.

#### VCCI  Warning

#### LIMITED WARRANTY

D-Link provides this limited warranty for its product only to the person or entity who originally purchased the product from D-Link or its authorized reseller or distributor.

Limited Hardware Warranty: D-Link warrants that the hardware portion of the D-Link products described below (“Hardware”) will be free from material defects in workmanship and materials from the date of original retail purchase of the Hardware, for the period set forth below applicable to the product type (“Warranty Period”) if the Hardware is used and serviced in accordance with applicable documentation; provided that a completed Registration Card is returned to an Authorized D-Link Service Office within ninety (90) days after the date of original retail purchase of the Hardware.  If a completed Registration Card is not received by an authorized D-Link Service Office within such ninety (90) period, then the Warranty Period shall be ninety (90) days from the date of purchase.

| Product Type | Warranty Period |
| --- | --- |
| Product (excluding power supplies and fans) | One (1) Year |
| Power Supplies and Fans | One (1) Year |
| Spare parts and spare kits | Ninety (90) days |

D-Link’s sole obligation shall be to repair or replace the defective Hardware at no charge to the original owner.  Such repair or replacement will be rendered by D-Link at an Authorized D-Link Service Office.  The replacement Hardware need not be new or of an identical make, model or part; D-Link may in its discretion may replace the defective Hardware (or any part thereof) with any reconditioned product that D-Link reasonably determines is substantially equivalent (or superior) in all material respects to the defective Hardware.  The Warranty Period shall extend for an additional ninety (90) days after any repaired or replaced Hardware is delivered. If a material defect is incapable of correction, or if D-Link determines in its sole discretion that it is not practical to repair or replace the defective Hardware, the price paid by the original purchaser for the defective Hardware will be refunded by D-Link upon return to D-Link of the defective Hardware.  All Hardware (or part thereof) that is replaced by D-Link, or for which the purchase price is refunded, shall become the property of D-Link upon replacement or refund.

Limited Software Warranty: D-Link warrants that the software portion of the product (“Software”) will substantially conform to D-Link’s then current functional specifications for the Software, as set forth in the applicable documentation, from the date of original delivery of the Software for a period of ninety (90) days (“Warranty Period”), if the Software is properly installed on approved hardware and operated as contemplated in its documentation.  D-Link further warrants that, during the Warranty Period, the magnetic media on which D-Link delivers the Software will be free of physical defects.  D-Link’s sole obligation shall be to replace the non-conforming Software (or defective media) with software that substantially conforms to D-Link’s functional specifications for the Software.  Except as otherwise agreed by D-Link in writing, the replacement Software is provided only to the original licensee, and is subject to the terms and conditions of the license granted by D-Link for the Software.  The Warranty Period shall extend for an additional ninety (90) days after any replacement Software is delivered.  If a material non-conformance is incapable of correction, or if D-Link determines in its sole discretion that it is not practical to replace the non-conforming Software, the price paid by the original licensee for the non-conforming Software will be refunded by D-Link; provided that the non-conforming Software (and all copies thereof) is first returned to D-Link.  The license granted respecting any Software for which a refund is given automatically terminates.

What You Must Do For Warranty Service:

Registration Card.  The Registration Card provided at the back of this manual must be completed and returned to an Authorized D-Link Service Office for each D-Link product within ninety (90) days after the product is purchased and/or licensed.  The addresses/telephone/fax list of the nearest Authorized D-Link Service Office is provided in the back of this manual.  FAILURE TO PROPERLY COMPLETE AND TIMELY RETURN THE REGISTRATION CARD MAY AFFECT THE WARRANTY FOR THIS PRODUCT.

Submitting A Claim.  Any claim under this limited warranty must be submitted in writing before the end of the Warranty Period to an Authorized D-Link Service Office.  The claim must include a written description of the Hardware defect or Software nonconformance in sufficient detail to allow D-Link to confirm the same.  The original product owner must obtain a Return Material Authorization (RMA) number from the Authorized D-Link Service Office and, if requested, provide written proof of purchase of the product (such as a copy of the dated purchase invoice for the product) before the warranty service is provided.  After an RMA number is issued, the defective product must be packaged securely in the original or other suitable shipping package to ensure that it will not be damaged in transit, and the RMA number must be prominently marked on the outside of the package.  The packaged product shall be insured and shipped to D-Link, 53 Discovery Drive, Irvine CA 92618, with all shipping costs prepaid.  D-Link may reject or return any product that is not packaged and shipped in strict compliance with the foregoing requirements, or for which an RMA number is not visible from the outside of the package.  The product owner agrees to pay D-Link’s reasonable handling and return shipping charges for any product that is not packaged and shipped in accordance with the foregoing requirements, or that is determined by D-Link not to be defective or non-conforming.

What Is Not Covered:

This limited warranty provided by D-Link does not cover:

Products that have been subjected to abuse, accident, alteration, modification, tampering, negligence, misuse, faulty installation, lack of reasonable care, repair or service in any way that is not contemplated in the documentation for the product, or if the model or serial number has been altered, tampered with, defaced or removed;

Initial installation, installation and removal of the product for repair, and shipping costs;

Operational adjustments covered in the operating manual for the product, and normal maintenance;

Damage that occurs in shipment, due to act of God, failures due to power surge, and cosmetic damage; and

Any hardware, software, firmware or other products or services provided by anyone other than D-Link.

Disclaimer of Other Warranties: EXCEPT FOR THE LIMITED WARRANTY SPECIFIED HEREIN, THE PRODUCT IS PROVIDED “AS-IS” WITHOUT ANY WARRANTY OF ANY KIND INCLUDING, WITHOUT LIMITATION, ANY WARRANTY OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NON-INFRINGEMENT.  IF ANY IMPLIED WARRANTY CANNOT BE DISCLAIMED IN ANY TERRITORY WHERE A PRODUCT IS SOLD, THE DURATION OF SUCH IMPLIED WARRANTY SHALL BE LIMITED TO NINETY (90) DAYS.  EXCEPT AS EXPRESSLY COVERED UNDER THE LIMITED WARRANTY PROVIDED HEREIN, THE ENTIRE RISK AS TO THE QUALITY, SELECTION AND PERFORMANCE OF THE PRODUCT IS WITH THE PURCHASER OF THE PRODUCT.

Limitation of Liability: TO THE MAXIMUM EXTENT PERMITTED BY LAW, D-LINK IS NOT LIABLE UNDER ANY CONTRACT, NEGLIGENCE, STRICT LIABILITY OR OTHER LEGAL OR EQUITABLE THEORY FOR ANY LOSS OF USE OF THE PRODUCT, INCONVENIENCE OR DAMAGES OF ANY CHARACTER, WHETHER DIRECT, SPECIAL, INCIDENTAL OR CONSEQUENTIAL (INCLUDING, BUT NOT LIMITED TO, DAMAGES FOR LOSS OF GOODWILL, WORK STOPPAGE, COMPUTER FAILURE OR MALFUNCTION, LOSS OF INFORMATION OR DATA CONTAINED IN, STORED ON, OR INTEGRATED WITH ANY PRODUCT RETURNED TO D-LINK FOR WARRANTY SERVICE) RESULTING FROM THE USE OF THE PRODUCT, RELATING TO WARRANTY SERVICE, OR ARISING OUT OF ANY BREACH OF THIS LIMITED WARRANTY, EVEN IF D-LINK HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.  THE SOLE REMEDY FOR A BREACH OF THE FOREGOING LIMITED WARRANTY IS REPAIR, REPLACEMENT OR REFUND OF THE DEFECTIVE OR NON-CONFORMING PRODUCT.

GOVERNING LAW: This Limited Warranty shall be governed by the laws of the state of California.

Some states do not allow exclusion or limitation of incidental or consequential damages, or limitations on how long an implied warranty lasts, so the foregoing limitations and exclusions may not apply.  This limited warranty provides specific legal rights and the product owner may also have other rights which vary from state to state.

Wichtige Sicherheitshinweise

- Bitte lesen Sie sich diese Hinweise sorgfältig durch.

- Heben Sie diese Anleitung für den spätern Gebrauch auf.

- Vor jedem Reinigen ist das Gerät vom Stromnetz zu trennen.  Vervenden Sie keine Flüssig- oder Aerosolreiniger.  Am besten dient ein angefeuchtetes Tuch zur Reinigung.

- Um eine Beschädigung des Gerätes zu vermeiden sollten Sie nur Zubehörteile verwenden, die vom Hersteller zugelassen sind.

- Das Gerät is vor Feuchtigkeit zu schützen.

- Bei der Aufstellung des Gerätes ist auf sichern Stand zu achten.  Ein Kippen oder Fallen könnte Verletzungen hervorrufen. Verwenden Sie nur sichere Standorte und beachten Sie die Aufstellhinweise des Herstellers.

- Die Belüftungsöffnungen dienen zur Luftzirkulation die das Gerät vor Überhitzung schützt.  Sorgen Sie dafür, daß diese Öffnungen nicht abgedeckt werden.

- Beachten Sie beim Anschluß an das Stromnetz die Anschlußwerte.

- Die Netzanschlußsteckdose muß aus Gründen der elektrischen Sicherheit einen Schutzleiterkontakt haben.

- Verlegen Sie die Netzanschlußleitung so, daß niemand darüber fallen kann.  Es sollete auch nichts auf der Leitung abgestellt werden.

- Alle Hinweise und Warnungen die sich am Geräten befinden sind zu beachten.

- Wird das Gerät über einen längeren Zeitraum nicht benutzt, sollten Sie es vom Stromnetz trennen.   Somit wird im Falle einer Überspannung eine Beschädigung vermieden.

- Durch die Lüftungsöffnungen dürfen niemals Gegenstände oder Flüssigkeiten in das Gerät gelangen.  Dies könnte einen Brand bzw. Elektrischen Schlag auslösen.

- Öffnen Sie niemals das Gerät.  Das Gerät darf aus Gründen der elektrischen Sicherheit nur von authorisiertem Servicepersonal geöffnet werden.

- Wenn folgende Situationen auftreten ist das Gerät vom Stromnetz zu trennen und von einer qualifizierten Servicestelle zu überprüfen:

  - Netzkabel oder Netzstecker sint beschädigt.

  - Flüssigkeit ist in das Gerät eingedrungen.

  - Das Gerät war Feuchtigkeit ausgesetzt.

  - Wenn das Gerät nicht der Bedienungsanleitung ensprechend funktioniert oder Sie mit Hilfe dieser Anleitung keine Verbesserung erzielen.

  - Das Gerät ist gefallen und/oder das Gehäuse ist beschädigt.

  - Wenn das Gerät deutliche Anzeichen eines Defektes aufweist.

- Bei Reparaturen dürfen nur Orginalersatzteile bzw. den Orginalteilen entsprechende Teile verwendet werden.  Der Einsatz von ungeeigneten Ersatzteilen kann eine weitere Beschädigung hervorrufen.

- Wenden Sie sich mit allen Fragen die Service und Repartur betreffen an Ihren Servicepartner.  Somit stellen Sie die Betriebssicherheit des Gerätes sicher.

- Zum Netzanschluß dieses Gerätes ist eine geprüfte Leitung zu verwenden, Für einen Nennstrom bis 6A und einem Gerätegewicht gr ßer 3kg ist eine Leitung nicht leichter als H05VV-F, 3G, 0.75mm2  einzusetzen.

Trademarks

Copyright 1999 D-Link Corporation.  Contents subject to change without prior notice.  D-Link is a registered trademark of D-Link Corporation/D-Link Systems, Inc.  All other trademarks belong to their respective proprietors.

Copyright Statement

No part of this publication may be reproduced in any form or by any means or used to make any derivative such as translation, transformation, or adaptation without permission from D-Link Corporation/D-Link Systems Inc., as stipulated by the United States Copyright Act of 1976.

Offices

AUSTRALIA	D-LINK AUSTRALIA

Unit 16, 390 Eastern Valley Way, Roseville, NSW 2069, Australia

TEL: 61-2-9417-7100  FAX: 61-2-9417-1077

TOLL FREE: 1800-177-100 (Australia), 0800-900900 (New Zealand)

URL: www.dlink.com.au  E-MAIL: support@dlink.com.au, info@dlink.com.au

CANADA	D-LINK CANADA

#2180 Winston Park Drive, Oakville, Ontario, L6H 5W1 Canada

TEL: 1-905-829-5033  FAX: 1-905-829-5095  BBS: 1-965-279-8732  FREE CALL: 1-800-354-6522

URL: www.dlink.ca   E-MAIL: techsup@dlink.ca  FTP: ftp.dlinknet.com

CHILE	D-LINK SOUTH AMERICA

Isidora Goyeechea 2934 of 702, Las Condes, Santiago – Chile S.A.

TEL:	56-2-232-3185  FAX: 56-2-232-0923  URL: www.dlink.cl  E-MAIL: ccasassu@dlink.cl, tsilva@dlink.cl

CHINA	D-LINK CHINA

2F., Sigma Building, 49 Zhichun Road, Haidian District, 100080 Beijing, China

TEL: 86-10-88097777  FAX: 86-10-88096789

URL: www.dlink.com.cn

DENMARK	D-LINK DENMARK

Naverland 2, DK-2600 Glostrup, Copenhagen, Denmark

TEL:45-43-969040  FAX:45-43-424347  URL: www.dlink.dk

E-MAIL: info@dlink.dk

EGYPT	D-LINK MIDDLE EAST

7 Assem Ebn Sabet Street, Heliopolis Cairo, Egypt

TEL: 202-2456176  FAX: 202-2456192  URL: www.dlink-me.com

E-MAIL: support@dlink-me.com, fateen@dlink-me.com

FRANCE	D-LINK FRANCE

Le Florilege #2, Allee de la Fresnerie

78330 Fontenay le Fleury France

TEL: 33-1-302-38688  FAX: 33-1-3023-8689  URL: www.dlink-france.fr

E-MAIL: info@dlink-france.fr

GERMANY	D-LINK Central Europe/D-Link Deutschland GmbH

Schwalbacher Strasse 74

D-65760 Eschborn, Germany

TEL: 49-6196-77990  FAX: 49-6196-7799300

URL: www.dlink.de  BBS: 49-(0)6192-971199 (Analog) 49-(0)6192-971198 (ISDN)

INFO LINE: 00800-7250-0000 (toll free)  HELP LINE: 00800-7250-4000 (toll free)

REPAIR LINE: 00800-7250-8000  E-MAIL: info@dlink.de

INDIA	D-LINK INDIA

Plot No.5, Kurla-Bandra Complex Road,

Off Cst Road, Santacruz (E), Bombay - 400 098 India

TEL: 91-22-652-6696  FAX: 91-22-652-8914  URL: www.dlink-india.com

E-MAIL: service@dlink.india.com

ITALY	D-LINK ITALIA

Via Nino Bonnet No. 6/b, 20154 Milano, Italy

TEL: 39-02-2900-0676  FAX: 39-02-2900-1723    URL: www.dlink.it

E-MAIL: info@dlink.it

JAPAN	D-LINK JAPAN

10F, 8-8-15 Nishi-Gotanda, Shinagawa-ku, Tokyo 141, Japan

TEL: 81-3-5434-9678  FAX: 81-3-5434-9868  URL: www.d-link.co.jp

E-MAIL: kida@d-link.co.jp

RUSSIA	D-LINK RUSSIA

Michurinski Prospekt 49, 117607 Moscow, Russia

TEL: 7-095-737-3389, 7-095-737-3492  FAX: 7-095-737-3390

URL: www.dlink.ru  E-MAIL: vl@dlink.ru

SINGAPORE	D-LINK INTERNATIONAL

1 International Business Park, #03-12 The Synergy, Singapore 609917

TEL: 65-774-6233  FAX: 65-774-6322

URL: www.dlink-intl.com  E-MAIL: info@dlink.com.sg

S. AFRICA	D-LINK SOUTH AFRICA

102-106 Witchhazel Avenue, Einetein Park 2, Block B,

Highveld Technopark Centurion, South Africa

TEL: 27(0)126652165  FAX: 27(0)126652186

URL: www.d-link.co.za  E-MAIL: attie@d-link.co.za

SWEDEN	D-LINK SWEDEN

P.O. Box 15036, S-167 15 Bromma Sweden

TEL: 46-(0)8564-61900  FAX: 46-(0)8564-61901  E-MAIL: info@dlink.se

URL: www.dlink.se

TAIWAN	D-LINK TAIWAN

2F, No. 119 Pao-Chung Road, Hsin-Tien, Taipei, Taiwan,

TEL: 886-2-2910-2626  FAX: 886-2-2910-1515  URL: www.dlinktw.com.tw

E-MAIL: dssqa@tsc.dlinktw.com.tw

U.K.	D-LINK EUROPE

4th Floor, Merit House, Edgware Road, Colindale, London,  NW9 5AB, U.K.

TEL: 44-20-8731-5555  FAX: 44-20-8731-5511

URL: www.dlink.co.uk  E-MAIL: info@dlink.co.uk

U.S.A.	D-LINK U.S.A.

53 Discovery Drive, Irvine, CA 92618  USA

TEL: 1-949-788-0805  FAX: 1-949-753-7033  INFO LINE: 1-800-326-1688

BBS: 1-949-455-1779, 1-949-455-9616

URL: www.dlink.com  E-MAIL: tech@dlink.com, support@dlink.com

Registration Card

Print, type or use block letters.

Your name: Mr./Ms	_____________________________________________________________________________

Organization: ________________________________________________	Dept. ____________________________

Your title at organization:	________________________________________________________________________

Telephone:	_______________________________________ Fax:________________________________________

Organization's full address:	______________________________________________________________________

____________________________________________________________________________________________

Country:	_____________________________________________________________________________________

Date of purchase (Month/Day/Year):	_______________________________________________________________

| Product Model | Product Serial No. | * Product installed in type of computer (e.g., Compaq 486) | * Product installed in computer serial No. |
| --- | --- | --- | --- |

(* Applies to adapters only)

Product was purchased from:

Reseller's name:	______________________________________________________________________________

Telephone:	_______________________________________ Fax:________________________________________

Reseller's full address:	_________________________________________________________________________

_________________________________________________________________________

_________________________________________________________________________

Answers to the following questions help us to support your product:

1.	Where and how will the product primarily be used?

Home Office Travel Company Business Home Business Personal Use

2. How many employees work at installation site?

1 employee 2-9 10-49 50-99 100-499 500-999 1000 or more

3. What network protocol(s) does your organization use ?

XNS/IPX TCP/IP DECnet Others_____________________________

4. What network operating system(s) does your organization use ?

D-Link LANsmart Novell NetWare NetWare Lite SCO Unix/Xenix PC NFS 3Com 3+Open

Banyan Vines DECnet Pathwork Windows NT Windows NTAS Windows '95

Others__________________________________________

5. What network management program does your organization use ?

D-View HP OpenView/Windows HP OpenView/Unix SunNet Manager Novell NMS

NetView 6000 Others________________________________________

6. What network medium/media does your organization use ?

Fiber-optics Thick coax Ethernet Thin coax Ethernet 10BASE-T UTP/STP

100BASE-TX 100BASE-T4 100VGAnyLAN Others_________________

7. What applications are used on your network?

Desktop publishing Spreadsheet Word processing CAD/CAM

Database management Accounting Others_____________________

8. What category best describes your company?

Aerospace Engineering Education Finance Hospital Legal Insurance/Real Estate Manufacturing

Retail/Chainstore/Wholesale Government Transportation/Utilities/Communication VAR

System house/company Other________________________________

9. Would you recommend your D-Link product to a friend?

Yes No Don't know yet

10.Your comments on this product?

__________________________________________________________________________________________

__________________________________________________________________________________________