DES-1316K

Date: September 14, 2004

| Product name: | A PoE bundling kit of one 8 10/100Mpbs Ports + 8 10/100 PoE Ports Web Smart Switch and two DWL-P50 |
| --- | --- |
| Product Group & Type: |  |
| Revision: | A1 |
| Available: |  |
| Picture: |  |
| Why Sell This Product: | - Convenient kit consisting of a 802.3af standard PoE DES-1316 switch and 2 DWL-P50 PoE adapters. It’s a good way to introduce the DWL-P50 to your customers when you sell the DES-1316. - Great for selling to installation sites with multiple (10 or above) wireless AP and Internet cameras. Selling Points - Convenient one-in-all PoE kit. - Guaranteed operation with D-Link products. |
| Key Features: | DES-1316 - 16 10/100BASE-TX Ethernet ports (8 PoE ports, 8 non-PoE ports) - Complies to 802.3af Power over Ethernet standard - Supplies up to 15.4 watts per PoE port - Auto-detection of network devices needing PoE - Active circuit protection with auto-disable of PoE in case of over current - PoE ports operate with all 802.3af compliant and non-compliant devices (DWL-P50 required for non-compliant devices) - Auto MDI/MDIX cross over for all ports - 802.3x Flow Control for protection against data loss - Port mirroring for traffic monitoring - Port trunk for server/network backbone attachment - 802.1Q VLAN to enhance security/network performance - 802.1p Priority Queues, port-based QoS - Easy configuration through web-based management - Standard rack-mount size DWL-P50 - Provides DC power over existing Category 5 cable to wireless LAN device - Cuts costs and provides convenience - Transforms incoming AC power into low-voltage DC power - Protects wireless LAN device from possible power-surges - Supports both 5VDC and 12VDC power (selectable through a slide switch) - Very compact size, lightweight - Complete plug-and-play installation, no configuration setting or installation tool required - No software required, O.S.-independent |
| Technical Details: | DES-1316 General Port Standards & Functions - IEEE 802.3af Power over Ethernet (PoE) - IEEE 802.3 10BASE-T Ethernet (twisted-pair copper) - IEEE 802.3u 100BASE-TX Fast Ethernet (twisted-pair copper) - ANSI/IEEE 802.3 NWay auto-negotiation - IEEE 802.3x Flow Control - Port mirroring Number of Ports 16 10BASE-T/100BASE-TX ports (8 PoE ports, 8 non-PoE ports) Protocol CSMA/CD Data Transfer Rates - Ethernet: 10Mbps (half duplex) 20Mbps (full duplex) - Fast Ethernet: 100Mbps (half duplex) 200Mbps (full duplex) Topology Star Network Cables - UTP Cat. 5, Cat. 5e (100 m max.) - EIA/TIA-568 100-ohm STP (100 m max.) Media Interface Exchange Auto MDI/MDIX adjustment for all ports LED Indicators - Power (per device) - CPU (per device) - Link/Act, Speed (per port) - 100Mbps (per port) - PoE (per PoE port) PoE PoE Ports Port 1 to Port 8 Power Supplied Up to 15.4 watts per port Auto-Detection Automatic recognition of connected Powered Device (PD) and sending power to PD Auto-Disable Automatically disables the port in case port current is over 350mA Active Circuit Protection Automatically disables the port in case of is a short PoE Pinouts Pins 1, 2, 3, and 6 of Cat. 5 cable Software VLAN - Standard: IEEE 802.1Q - Maximum number of VLAN: 16 per device Quality of Service (QoS) - Standard: IEEE 802.1p - Maximum number of queues: 2 - Port-based Port Trunks - Standard: IEEE 802.3ad - Max. 4 ports per trunk - 1 port trunk per device Configuration/Management Methods - Web-based configuration - Windows-based utility - Configuration reset through software or hardware (reset button) Configuration/Management Functions - Port speed/full/half duplex setting - Port power setting - Switch auto-discovery Performance Switch Fabric 3.2Gbps Transmission Method Store-and-forward MAC Address Table 4K entries per device MAC Address Learning Automatic update Packet Filtering/Forwarding Rates (half duplex) Maximum 1,488,095 pps per port RAM Buffer 512KB per device Physical & Environmental AC Input 100 to 240 VAC 50/60Hz internal universal power supply Power Consumption - Maximum 9 watts per device (without any PoE devices connected) - Maximum 130 watts per device (when all 8 PoE devices are connected) Dimensions 440 (W) x 140 (D) x 44 (H) mm 19-inch standard rack mount width, 1 U height Weight 2.15 kg Operating Temperature 0 to 55 ℃ Storage Temperature -10 to 70 ℃ Operating Humidity 10% to 90% RH Storage Humidity 5% to 90% RH Emission (EMI) - FCC Class A - CE Class A - VCCI Class A Safety - UL - CUL - TUV DWL-P50 Output Voltage - Input: 48VDC, 400mA - Output: 5VDC (11.5 watts) or 12VDC (12 watts) (slide switch selectable) Ethernet Cable - TIA/EIA 568 Cat. 5 - Connector: RJ-45 Diagnostic LED Power Dimensions 80.9 x 52 x 28 mm (adapter only) Weight 80 grams (adapter only) Operating Temperature 0 to 40 ℃ (32 to 104 ℃) Storage Temperature -40 to 70 ℃ (-40 to 158 ℃) Operating Humidity 10% to 80% RH Storage Humidity 5% to 90% RH EMI - FCC Part 15 Class B - CE Class B Safety - UL - TUV Box Includes - DWL-P50 PoE adapter - Quick Installation Guide - Ethernet cable - DC power cable with female power connectors |
| EMI/EMC Report | DES-1316 FCC No. NEI-FCCE-1-04E0539  CE No. NEI-EMC-1-04E0539  VCCI No. NEI-VC-1-04E0539  CTR-21 No.  C-Tick No.  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  Others DWL-P50 FCC No. NEI-FCCE-1-04E0565  CE No. NEI-EMC-1-04E0565  VCCI No. NEI-VC-1-04E0565  CTR-21 No.  C-Tick No.  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  Others |
| Safety | DES-1316  TUV No.  CUL No.  CSA No.  Others DWL-P50  TUV No.  CUL No.  CSA No.  Others |
| Packaging Style |  Clamshell  White Box with D-Link logo  Gift Box  Bulk  Others |
| Packaging Contents: | - One DES-1316 Device - One set accessory including 2 Rack mount ear/4 Rubber feet and 8 Screws - One AC power cord - One CD Kit for DES-1316 Manual / QIG / Utility - One hardcopy QIG for DES-1316 (BES1316KA.A1 doesn’t have) - One hardcopy QIG for DWL-P50 - Two DWL-P50 PoE adapters - Two Cat.5 Ethernet cable - Two Female-to-Female power connector |
| UPC Code: | BES1316KA.A1; 790069 271717. TES1316KA.A1; 790069 271199. NES1316KCNA1; 790069 271182. IES1316KN.A1; 790069 271175. IES1316KE.A1; 790069 271168. IES1316KA..A1; 790069 271151. EES1316KE.A1; 790069 271144. EES1316KB.A1; 790069 271137. EES1316KBEA1; 790069 271120. |
| Units Per Carton: | 6 |
| Cartons Per Pallet: | Air: 16, Sea: 24 |
| Dimensions and Weight: | Box: 545 x 210 x 94 mm, 3 .8 kg Carton: 576 x 451 x 325 mm, 23.8 kg Pallet: (by Air) 1200 x 1100 x 1460 mm, 395.8 kg (for WW) 1020 x 1220 x 1400 mm, 395.8 kg (forUSA) (by Sea) 1200 x 1100 x 2110 mm, 586.2 kg (for WW) 1020 x 1220 x 1780 mm, 586.2 kg (for USA) INTRASTAT weight: kg (Box) Packing material weight (Box): - Paper: 0.675 kg - Plastic: 0.097 kg - Mixed: 0.151 kg |