# Product Announcement

Date: 2005/1/20

To: USA, DI, PRC

Subject: 24-Port 10/100Mpbs + 2 Combo 1000BASE-T/SFP PoE Web Smart Switch

PART NO.: BES1526A..A1, IES1526A..A1, IES1526B..A1, IES1526E..A1, IES1526N..A1, NES1526CN.A1, TES1526A..A1

### About DES-1526

DES-1526 is a 24-port 10/100M + 2 Combo 1000BASE-T/SFP Web Smart Switch, and all of its 10/100M port are with 802.3af PoE capability, which deliver power over CAT5 Ethernet cable for D-Link PoE capable wireless APs, IP Cams, IP phones or other 802.3af compliant powered devices. By working with DWL-P50 [PoE adapter], DES-1526 can include D-Link non-802.3af capable wireless APs, IP Cams and IP Phones into D-Link’s PoE solution. Also its Web GUI enable customers to perform easy and straight forward remote management.

Hardware Specifications

| Key Components |  |
| --- | --- |
| Switching Capacity | 8.8Gbps |
| Max. Forwarding Rate | 6.55 million packets per second |
| Forwarding Mode | Store and Forward |
| SDRAM for CPU | 128Kbytes |
| Flash Memory | 1MB |
| LAN | 24 x 10/100BaseT ports Compliant to following standards: - IEEE 802.3 compliance - IEEE 802.3u compliance - Support Full-Duplex operations - IEEE 802.3x Flow Control support for Full-Duplex mode |
| Combo ports | 2 combo 1000Base-T/SFP ports 1000Base-T ports compliant to following standards: - IEEE 802.3 compliance - IEEE 802.3u compliance - IEEE 802.3ab compliance - Support Full-Duplex operations - IEEE 802.3x Flow Control support for Full-Duplex mode SFP Transceivers Supported: - DEM-310GT (1000BASE-LX) - DEM-311GT (1000BASE-SX) - DEM-314GT (1000BASE-LH) - DEM-315GT (1000BASE-ZX) Compliant to following standards: - IEEE 802.3z compliance |
| MAC Address Table | 4K |
| Packet Buffer memory | 768Kbytes |
| PoE Features |  |
| PoE Capable Ports | Port 1-24 |
| PoE Specification | - Supplies power to PD device up to 15.4W per port, meeting IEEE802.3af standards and more sufficiently is able to provide power to PD devices - Power Budget: Full power for up to 12 ports, or up to 7W for 24 ports - Auto discovery feature, automatically recognize the connection of PD device and immediately sends power to it - Auto disable port if the port current is over 350mA while other ports remain active - Active circuit protection, automatically disables the port if there is a short while other ports remain active - Provide the power following the classification below Class; Usage; Max power used by PD. 0; Default; 15.4W. 1; Optional; 4.0W. 2; Optional; 7.0W. 3; Optional; 15.4W. 4; Reserved; 15.4W. - DES-1526 follow the PSE pinout standard of Alternative A which is sending out power over number 1,2,3,6 pins of 8 wires of CAT5 UTP cable |

Product Features

| L2 Features |  |
| --- | --- |
| 802.3ad Link Aggregation | Yes [Static Mode] [Note: SFP ports can not be trunked] |
| Port mirroring | Yes |
| VLAN |  |
| 802.1Q | Yes |
| VLAN Group | 64 |
| Quality Of Service |  |
| Priority queues number | 2 |
| Based on Port | Yes |
| 802.1p support | Yes |
| Management |  |
| Single IP to Manage Whole Stack | No |
| Web-GUI | Yes |
| CLI | Yes |
| Telnet | Yes |
| SNMP v1 | Yes |
| BooTP/DHCP client | Yes |

Physical and Environmental

| Physical and Environmental |  |
| --- | --- |
| AC inputs: | 100-240 VAC, 50/60Hz Internal universal power supply |
| Operating Temperature: | 0 ~ 40°C |
| Storage Temperature: | -10~70°C |
| Humidity: | Operation: 10%-90% RH Storage: 5% ~ 90% RH |
| Dimensions: | 441mm(Length) x 310mm(Width) x 44mm (Depth) |
| EMI: | FCC Class A, CE Class A, VCCI Class A |
| Safety: | CUL International |

Box Includes

## One DES-1526 Device

## One set accessory including 2 Rack mount ears, 4 Rubber feet and 8 Screws

## One AC power cord

## One CD-ROM for Manual/QIG/Utility

## One hard copy QIG(BES1526A..A1 doesn’t packed with QIG)

## Ordering Information

BES1526A..A1	24-Port 10/100Mpbs + 2 Combo 1000BASE-T/SFP PoE Web Smart Switch, with USA power cord

IES1526A..A1	24-Port 10/100Mpbs + 2 Combo 1000BASE-T/SFP PoE Web Smart Switch, with DI QIG and USA power cord

IES1526B..A1	24-Port 10/100Mpbs + 2 Combo 1000BASE-T/SFP PoE Web Smart Switch, with DI QIG and UK power cord

IES1526E..A1	24-Port 10/100Mpbs + 2 Combo 1000BASE-T/SFP PoE Web Smart Switch, with DI QIG and European power cord

IES1526N..A1	24-Port 10/100Mpbs + 2 Combo 1000BASE-T/SFP PoE Web Smart Switch, with DI QIG and Australian power cord

NES1526CN.A1	24-Port 10/100Mpbs + 2 Combo 1000BASE-T/SFP PoE Web Smart Switch, with DI QIG and CN power cord

TES1526A..A1	24-Port 10/100Mpbs + 2 Combo 1000BASE-T/SFP PoE Web Smart Switch, with DI QIG and USA power cord

## Ordering Price

Please refer to the Price Book

## Lead Time:

4 ~ 5 weeks (with forecast)