# DES-3010F/3010FL/3010G/3018/3026 & DHS-3010F/3010G/3018/3026

# Firmware Release Note

Firmware: 3.00.032 (PROM：1.01.006)

Hardware: A1/A2 for DES-3000 series, D1/D2 for DHS-3000 series

Date: July. 10, 2006

Problems Resolved:

1.DES-3018 .BIN file write to DES-3010F, port link fail

2. PROM code upgrade to v1.01.006 for new BCM532x series P12 IC introduce, its hardware version is A2/D2.

3. DES-3000 series IP-MAC issue.

Firmware: 3.00.025 (PROM：1.01.005)

Hardware: A1 for DES-3000 series, D1 for DHS-3000 series

Date: Jun. 5, 2006

Add new features:

1.MAC notification

2.Cos based on MAC DA/TOS/DSCP

3.IP/MAC Binding

4.BPDU filtering per port

5.Multicast filtering mode

6. Add save log command.

7. Support new module: DEM-201FL

8. Support chip BCM532x series P12 IC

Problems Resolved:

1. Fix: Incorrect SNMP value in Bridge MIB -dot1d STP Port Designated Bridge/Root

2. Fix: STP LBD function but CPU 100% periodically.

3. Fix: factory test issue: DES-3010FL MAC address error.

4. Fix: The source port of DES-3010F SNMP trap was = "0"

5. Fix IGMP Snooping issue - Stop a stream may also stop another stream play in the same port/PC.

Firmware: 2.00.026 (PROM：1.01.005)

Hardware: A1 for DES-3000 series, D1 for DHS-3000 series

Date: Feb. 27, 2006

## Fixed LED issue when reboot

Led bright one times when DUT enter CLI (As R1).

Firmware: 2.00.023 (PROM：1.01.005)

Hardware: A1 for DES-3000 series, D1 for DHS-3000 series

Date: Feb. 21, 2006

Add support new flash:

S29GL032M90TFIR40

W19B320ABT7H

AT49BV322D-70TU

Firmware: 2.00.020 (PROM：1.01.003)

Hardware: A1 for DES-3000 series, D1 for DHS-3000 series

Date: Dec. 26, 2005

Problems Resolved:

- Fix D-Link Russia reported security issue. Correction actions:

* Remove super password

* Change backdoor password to console access only.

2. Fix “trusted_host” IP works in telnet/web, but fails in SNMP problem.

3. Fix WinXP 802.1x client showing wrong message when running “local 802.1x”

4. Fix message error (IP=0.0.0.0) when sending “linkup/down” trap.

5. Fix static FDB table problem when co-working with port-security function.

6. Fix failed to configure “Unicast/Multicast static filtering table” from D-View SNMP tool.

7. Fix Web UI Java script security issue: If using user-level account login from Web UI (eg., Firefox) and then disable Java script of web, that user-level account can have admin-level access right.

8. Fix incorrectly sending “port security access violation trap” issue.

Enhancements:

- DHCP Auto-configuration

- Web UI supports non-IE browser

- STP loopback prevention

- Bandwidth control GUI/CLI message remind

- SMTP client Log enhancement

- Log admin password/IP change activity enhancement

- SIM commander

- CPU interface filtering

- Port Security Violation Trap/Log

Notes:

1. For firmware upgrade from R1 to R2, please disable 802.1x state first. Then re-configure the 802.1x function in R2 firmware version.

2. After upgrade firmware from R1 to R2, user need re-configure the SIM and syslog function. Because these functions be upgraded.

DES-30xx Firmware 1.01.021 (Released: June 15, 2005)

First release version. For features, please check the Data Sheet.