DES-3000 Series SNMP MIB User Guide for v3.00.036

This document provides example of how to use the SNMP MIB Spec as a reference to find out where the function locates in SNMP (DES-3000 Series MIB Tree Table_v3.00.036 .zip). Examples in the document include:

1. Save Configuration

2. Forwarding Table

3. CPU Utilization

4. Port Security

5. IP-MAC-Port Binding

For detailed software features with corresponding firmware version, users should also refer to the external specification.

Example 1. Save Configuration

(1) In the “DES-3000 Series MIB Tree Table_v3.00.036.xls” file, user can search “dlkAgent” label for “savecfg” and the result of the specific MIB will be shown below:

(2) In the “private” folder, using notepad or text editor to open “Agent.mib”, and search for “savecfg”. User can find out how to save the configuration:

agentStatusSaveCfg OBJECT-TYPE

SYNTAX INTEGER {

other(1),

proceeding(2),

completed(3),

changed-not-save(4),

failed(5)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This indicates the status of the device configuration.

other(1) - this entry is currently in use but the conditions

under which it will remain so are different from each

of the following values.

proceeding(2) - the device configuration is being currently saved into the NV-RAM.

completed(3) – all of the device configuration parameters have been

saved into NV-RAM.

changed-not-save(4)– some of the device configuration parameters have

been changed but not saved into NV-RAM.

failed(5) - The process to save device configuration is failed."

::= { agentBasicInfo 4 }

Example 2. Forwarding Table

(1) In the “DES-3000 Series MIB Tree Table_v3.00.036.xls” file, user can search “RFC1493” label for “fdbtable” and the result of the specific MIB will be shown below:

(2) In the “Public” folder, using notepad or text editor to open “RFC1493.MIB” and search for “fdbtable”. User can find the detail functions are described.

dot1dTpFdbTable OBJECT-TYPE

SYNTAX  SEQUENCE OF Dot1dTpFdbEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"A table that contains information about unicast

entries for which the bridge has forwarding and/or

filtering information.  This information is used

by the transparent bridging function in

determining how to propagate a received frame."

::= { dot1dTp 3 }

dot1dTpFdbEntry OBJECT-TYPE

SYNTAX  Dot1dTpFdbEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"Information about a specific unicast MAC address

for which the bridge has some forwarding and/or

filtering information."

INDEX   { dot1dTpFdbAddress }

::= { dot1dTpFdbTable 1 }

Dot1dTpFdbEntry ::=

SEQUENCE {

dot1dTpFdbAddress

MacAddress,

dot1dTpFdbPort

INTEGER,

dot1dTpFdbStatus

INTEGER

}

dot1dTpFdbAddress OBJECT-TYPE

SYNTAX  MacAddress

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"A unicast MAC address for which the bridge has

forwarding and/or filtering information."

REFERENCE

"IEEE 802.1D-1990: Section 3.9.1, 3.9.2"

::= { dot1dTpFdbEntry 1 }

Example 3. CPU Utilization

(1) In the “DES-3000 Series MIB Tree Table_v3.00.036.xls” file, user can search “L2mgmt” label for “cpuutilizatoin” and the result of the specific MIB will be shown below:

(2) If it is DES-3026, in the “private” folder. Using notepad or text editor to open “des3026-l2mgmt.mib”, and search for “cpuutilizatoin”user can find out how to display the 5 sec/1 min/5min CPU Utilization:

swL2CPUutilization          OBJECT IDENTIFIER ::= { swL2DevMgmt 3 }

swL2CPUutilizationIn5sec OBJECT-TYPE

SYNTAX  Integer32

MAX-ACCESS  read-only

STATUS  current

DESCRIPTION

"Time scale is 5 seconds.The value will be between 0%(idle) and 100%

(very busy)."

::= { swL2CPUutilization 1 }

swL2CPUutilizationIn1min OBJECT-TYPE

SYNTAX  Integer32

MAX-ACCESS  read-only

STATUS  current

DESCRIPTION

"Time scale is 1 minute.The value will be between 0%(idle) and 100%

(very busy)."

::= { swL2CPUutilization 2 }

swL2CPUutilizationIn5min OBJECT-TYPE

SYNTAX  Integer32

MAX-ACCESS  read-only

STATUS  current

DESCRIPTION

"Time scale is 5 minute.The value will be between 0%(idle) and 100%

(very busy)."

::= { swL2CPUutilization 3 }         MAX-ACCESS  read-only

STATUS  current

DESCRIPTION

"Time scale is 5 minute.The value will be between 0%(idle) and 100%

(very busy)."

::= { agentCPUutilization 3 }

Example 4. Port Security

(1) In the “DES-3000 Series MIB Tree Table_v3.00.036.xls” file, user can search “L2mgmt” label for “portsecurity” and the result of the specific MIB will be shown below:

(2) Similarly, if it is DES-3026, in the “private” folder. Using notepad or text editor to open “des3026-l2mgmt.mib”, and search for “portsecurity”. User can check for how to use port security function.

swL2PortSecurityControlTable OBJECT-TYPE

SYNTAX  SEQUENCE OF SwL2PortSecurityControlEntry

MAX-ACCESS  not-accessible

STATUS  current

DESCRIPTION

"port security feature which controls the address leaning capability and the

traffic forwarding decision. Each port can have this function enabled or disabled.

When it is enabled and a number is given said N, which allows N addresses to be

learned at this port, the first N learned addresses are locked at this port as

static entry. When the learned addresses number reach N, any coming packet with

not learned source addresses are discarded (e.g. dropped) and no more new addresses

can be learned at this port."

::= { swL2PortSecurityMgmt 1 }

swL2PortSecurityControlEntry OBJECT-TYPE

SYNTAX  SwL2PortSecurityControlEntry

MAX-ACCESS  not-accessible

STATUS  current

DESCRIPTION

"A list of information contained in swL2PortSecurityControlTable."

INDEX   { swL2PortSecurityPortIndex }

::= { swL2PortSecurityControlTable 1 }

SwL2PortSecurityControlEntry ::=

SEQUENCE {

swL2PortSecurityPortIndex

INTEGER,

swL2PortSecurityMaxLernAddr

INTEGER,

swL2PortSecurityMode

INTEGER,

swL2PortSecurityAdmState

INTEGER

}

swL2PortSecurityPortIndex OBJECT-TYPE

SYNTAX  INTEGER (1..255)

MAX-ACCESS  read-only

STATUS  current

DESCRIPTION

"Indicates the secured port to lock address learning."

::= { swL2PortSecurityControlEntry 1 }

swL2PortSecurityMaxLernAddr OBJECT-TYPE

SYNTAX  INTEGER (0..10)

MAX-ACCESS  read-write

STATUS  current

DESCRIPTION

"Indicates allowable number of addresses to be learned at this port."

::= { swL2PortSecurityControlEntry 2 }

swL2PortSecurityMode OBJECT-TYPE

SYNTAX  INTEGER {

other(1),

permanent(2),

deleteOnTimeout(3),

deleteOnReset(4)

}

MAX-ACCESS  read-write

STATUS  current

DESCRIPTION

"Indicates the mode of locking address.

In deleteOnTimeout(3) mode - the locked addresses can be aged out after aging timer

expire. In this mode, when the locked address is aged

out, the number of address can be learned has to be

increased by one.

In deleteOnReset(4) mode - never age out the locked addresses unless restart

the system to prevent from port movement or intrusion."

::= { swL2PortSecurityControlEntry 3 }

swL2PortSecurityAdmState OBJECT-TYPE

SYNTAX  INTEGER {

other(1),

enable(2),

disable(3)

}

MAX-ACCESS  read-write

STATUS  current

DESCRIPTION

"Indicates administration state of locking address."

::= { swL2PortSecurityControlEntry 4 }

Example 5. IP-MAC-Port Binding

(1) In the “DES-3000 Series MIB Tree Table_v3.00.036.xls” file, user can search “IPMacBind” label for “ipmacbind” and the result of the specific MIB will be shown below:

(2) In the “private” folder, using notepad or text editor to open “IPMacBind.mib”, and search for “ipmacbind”. User can find out detailed functions.

swIpMacBindingPortTable OBJECT-TYPE

SYNTAX		SEQUENCE OF SwIpMacBindingPortEntry

MAX-ACCESS  not-accessible

STATUS		current

DESCRIPTION

"The table specifies port's ip-mac binding function."

::= { swIpMacBindingPortMgmt 2 }

swIpMacBindingPortEntry OBJECT-TYPE

SYNTAX		SwIpMacBindingPortEntry

MAX-ACCESS  not-accessible

STATUS		current

DESCRIPTION

"A list of information about port's ip-mac binding function."

INDEX { swIpMacBindingPortIndex }

::= { swIpMacBindingPortTable 1 }

SwIpMacBindingPortEntry ::=

SEQUENCE {

swIpMacBindingPortIndex

INTEGER,

swIpMacBindingPortState

INTEGER

}

swIpMacBindingPortIndex OBJECT-TYPE

SYNTAX		INTEGER(0..65535)

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"This object indicates the module's port number.(1..Max port

number in the module)."

::= { swIpMacBindingPortEntry 1 }

swIpMacBindingPortState OBJECT-TYPE

SYNTAX		INTEGER{

other(1),

enable(2),

disable(3)

}

MAX-ACCESS	read-write

STATUS		current

DESCRIPTION

"This object enable\disable the IP-MAC binding on the specified port."

::= { swIpMacBindingPortEntry 2 }

On the MIB browser, it shows like below:

The example above is to enable the IP-MAC-Port binding ACL mode for ip address “192.168.0.100” and MAC address “AABBCCDDEEFF” on port 1.

Each ports digit ”xxxxxxxx” (Hex) represents four ports, in this example, the first ”8” is ”1000” in binary.

１	０	０	０

			

p1	p2	p3	p4

=> 80000000 indicates Port 1