# CLI Command List

ERPS Function v1.00 Command Summary Table:

| ID | Command Syntax | Note |
| --- | --- | --- |
| 1 | enable erps |  |
| 2 | disable erps |  |
| 3 | create erps raps_vlan <vlanid> |  |
| 4 | delete erps raps_vlan <vlanid> |  |
| 5 | config erps raps_vlan <vlanid> ring_mel <value 0 - 7> |  |
| 6 | config erps raps_vlan <vlanid> ring_port [west <port> ¦ east <port>] |  |
| 7 | config erps raps_vlan <vlanid> [rpl_port [west ¦ east¦ none] ¦ rpl_owner [enable¦disable] ] |  |
| 8 | config erps raps_vlan <vlanid> protected_vlan [add¦delete] vlanid <vidlist> |  |
| 9 | config erps raps_vlan <vlanid> timer { holdoff_time <value 0 - 10000> ¦ guard_time <value 10 - 2000 > ¦ wtr_time <value 5 - 12>} (1) |  |
| 10 | config erps log [enable ¦ disable] |  |
| 11 | show erps |  |

## Command

### enable erps

#### Purpose

Used to enable ERPS function on a switch

#### Format

enable erps

#### Description

This command is used to enable ERPS function on a switch. STP and LBD should be disabled on the ring ports before enabling ERPS.

The ERPS cannot be enabled before the R-APS VLAN is created, and ring ports, RPL port, RPL owner, are configured. Note that these parameters cannot be changed when ERPS is enabled.

In order to guarantee correct operation, the following integrity will be checked when ERPS is enabled.

1. R-APS VLAN is created.

2. The Ring port is the tagged member port the R-APS VLAN.

3. The RPL port is specified if RPL owner is enabled.

The default state is disabled.

#### Parameters

None

#### Help message

Used to enable ERPS.

#### Restrictions

2-level administrator

3-level operator

#### Example

DXS-XXXX:N#enalbe erps Command: enable erps Success. DXS-XXXX:N#

### disable ERPS

#### Purpose

Used to disable ERPS function on a switch

#### Format

disable erps

#### Description

This command is used to disable ERPS function on a switch.

#### Parameters

None

#### Help message

Used to disable ERPS.

#### Restrictions

2-level administrator

3-level operator

#### Example

DXS-XXXX:N#disable erps Command: disable erps Success. DXS-XXXX:N#

### create raps VLAN

#### Purpose

Used to create R-APS VLAN on a switch

#### Format

create erps raps_vlan <vlanid>

#### Description

This command is used to create R-APS VLAN on a switch. There should be only one R-APS VLAN used to transfer R-APS messages.

Note that the R-APS VLAN must be an existed VLAN created by the create vlan command.

This command can only be issued when ERPS is disabled.

Engineering Note: If multiple ERPS instances are supported in the future, then multiple RAPS-VLAN, each for one instance, can be created.

#### Parameters

| Parameters | Description |
| --- | --- |
| raps_vlan | Specifies the VLAN which will be the R-APS VLAN. |

#### Help message

Used to create R-APS VLAN.

#### Restrictions

2-level administrator

3-level operator

#### Example

DXS-XXXX:N#create erps raps_vlan 4094 Command: create erps raps_vlan 4094 Success. DXS-XXXX:N#

### delete raps VLAN

#### Purpose

Used to delete R-APS VLAN on a switch

#### Format

delete erps raps_vlan <vlanid>

#### Description

This command is used to delete R-APS VLAN on a switch. When R-APS VLAN is deleted, all parameters relate to this R-APS VLAN also be deleted.

This command can only be issued when ERPS is disabled.

#### Parameters

| Parameters | Description |
| --- | --- |
| raps_vlan | Specifies the VLAN which will be the R-APS VLAN. |

#### Help message

Used to delete R-APS VLAN.

#### Restrictions

2-level administrator

3-level operator

#### Example

DXS-XXXX:N#delete erps raps_vlan 4094 Command: delete erps raps_vlan 4094 Success. DXS-XXXX:N#

### config erps ring_mel

#### Purpose

Used to configure the MEL of the ERPS ring for a specific RAPS VLAN.

#### Format

config erps raps_vlan <vlanid> ring mel <value 0 - 7>

#### Description

This command is used to configure the ring MEL for a RAPS VLAN. The ring MEL is one field in the R-APS PDU.

Note that if CFM (Connectivity Fault Management) and ERPS are used at the same time, R-APS PDU is one of a suite of Ethernet OAM PDU. The behavior for forwarding of R-APS PDU should follow the Ethernet OAM. If the ring MEL is not higher than the highest MEL of the MEPs on the ring ports, the R-APS PDU cannot be forwarded on the ring.

#### Parameters

| Parameters | Description |
| --- | --- |
| ring mel | Specifies the ring MEL of the R-APS function. The default ring MEL is 1. |

#### Help message

Used to configure the MEL of the ERPS ring for a specific RAPS VLAN.

#### Restrictions

2-level administrator

3-level operator

#### Example

DXS-XXXX:N#config erps raps_vlan 4094 ring mel 2 Command: config erps raps_vlan 4094 ring mel 2 Success. DXS-XXXX:N#

### config erps ring_port

#### Purpose

Used to configure the ports of the ERPS ring for a specific RAPS VLAN.

#### Format

config erps raps_vlan <vlanid> ring_port [west <port> | east <port>]

#### Description

This command is used to configure the port that participates the ERPS ring.

If user can specify a link aggregation group as the ring port by specifying the master port of link aggregation group as the ring port. Other member port of the link aggregation group cannot be specified as the ring port. If the link aggregation group is assigned as the ring port, but the link aggregation group is destroyed later, the master port remains as the ring port.

Note that the ring ports cannot be modified when ERPS is enabled.

#### Parameters

| Parameters | Description |
| --- | --- |
| west | Specifies the port as the west ring port. |
| east | Specifies the port as the east ring port. |

#### Help message

Used to configure the ports of the ERPS ring for a specific RAPS VLAN.

#### Restrictions

2-level administrator

3-level operator

#### Example

DXS-XXXX:N#config erps raps_vlan 4094 ring_port west 5 east 7 Command: config erps raps_vlan 4094 ring_port west 5 east 7 Success. DXS-XXXX:N#

### config erps rpl

#### Purpose

Used to configure the RPL port or the RPL owner for a specific RAPS VLAN.

#### Format

config erps raps_vlan <vlanid> [rpl_port [west | east| none] | rpl_owner [enable|disable] ]

#### Description

This command is used to configure the RPL port and the RPL owner.

RPL port

Specifies one of the ring port as the RPL port or deselect the RPL port.

RPL owner

Specifies the node as the RPL owner.

Note that the RPL port and RPL owner cannot be modified when ERPS is enabled.

#### Parameters

| Parameters | Description |
| --- | --- |
| port | west: specifies that west ring port as the RPL port. east:: specified the east ring port as the RPL port. none: No RPL port on this node. By default, the node has no RPL port. |
| owner | enable: specifies the device as RPL owner node. disable: This node is not RPL owner. By default, the RPS owner is disabled. |

#### Help message

Used to configure the RPL port or the RPL owner for a specific RAPS VLAN.

#### Restrictions

2-level administrator

3-level operator

#### Example

DXS-XXXX:N#config erps raps_vlan 4094 rpl port west owner enable Command: config erps raps_vlan 4094 rpl port west owner enable Success. DXS-XXXX:N#

### config erps protected_vlan

#### Purpose

Used to configure the protected VLAN for a specific RAPS VLAN.

#### Format

config erps raps_vlan <vlanid> protected_vlan [add|delete] vlanid <vidlist>

#### Description

This command is used to configure the VLANs that are protected by the ERPS function.

The R-APS VLAN cannot be the protected VLAN.

The protected VLAN does not need to be existed.

#### Parameters

| Parameters | Description |
| --- | --- |
| protected_vlan | add: Add VLANs to the protected VLAN group. delete: Delete VLANs from the protected VLAN group. |

#### Help message

Used to configure the protected VLAN for a specific RAPS VLAN.

#### Restrictions

2-level administrator

3-level operator

#### Example

DXS-XXXX:N#config erps raps_vlan 4094 protected_vlan add vlanid 10-20 Command: config erps raps_vlan 4094 protected_vlan add vlanid 10-20 Success. DXS-XXXX:N#

### config erps timer

#### Purpose

Used to configure the timers of the ERPS for a specific RAPS VLAN.

#### Format

config erps raps_vlan <vlanid> timer { holdoff_time <value 0 - 10000> | guard_time <value 10 - 2000 > | wtr_time <value 5 - 12>} (1)

#### Description

This command is used to configure the protocol timers.

Hold off timer

Hold-off timer is used to filter out intermittent link faults when link failure occurs. This timer is used during the protection switching process when link failure occurs. When a ring node detects link’s failure, it will start the hold off timer. It will report the link failure event (R-APS BPDU with SF flag) after the link failure is confirmed within this period of time.

Guard timer

Guard timer is used to prevent ring nodes from receiving outdated R-APS messages. This timer is used during the protection switching process when link failure recovers. When the link node detects that the link failure is recovered, it will report the link failure recovery event(R-APS PDU with NR flag) and start the guard timer, before the guard timer is expired, all received R-APS messages are ignored by this ring node. Therefore, the blocking state of the recovered link will not be recovered within this period of time. This time should be greater than the maximum expected forwarding delay for which one R-APS message circles around the ring.

WTR timer

WTR timer is used to prevent frequent operation of the protection switch due to an intermittent defect. This timer is used during the protection switching process when link failure recovers. This timer is only used by the RPL owner. When the RPL owner in protection state receives R-APS PDU with NR flag, it will start the WTR timer. The RPL owner will block the original unblocked RPL port and start to send R-APS PDU with RB flag after the link recovery is confirmed within this period of time.

#### Parameters

| Parameters | Description |
| --- | --- |
| holdoff_time | Specifies the hold off time of the R-APS function. The default hold off time is 0 milliseconds |
| guard_time | Specifies the guard time of the R-APS function. The default guard time is 500 milliseconds |
| wtr_time | Specifies the WTR time of the R-APS function. The range is from 5-12 minutes The default WTR time is 5 minutes. |

#### Help message

Used to configure the timers of the ERPS for a specific RAPS VLAN.

#### Restrictions

2-level administrator

3-level operator

#### Example

DXS-XXXX:N#config erps raps_vlan 4094 holdoff_time 100 guard_time 1000 wtr_time 10 Command: config erps raps_vlan 4094 holdoff_time 100 guard_time 1000 wtr_time 10 Success. DXS-XXXX:N#

### config ERPS log state

#### Purpose

Used to configure the log state of the ERPS.

#### Format

config erps log [enable | disable]

#### Description

This command is used to configure log state of ERPS events.

#### Parameters

| Parameters | Description |
| --- | --- |
| log | enable/disable log state. The default value is disabled. |

#### Help message

Used to configure the log state of the ERPS.

#### Restrictions

2-level administrator

3-level operator

#### Example

DXS-XXXX:N#config erps log enable Command: config erps log enable Success. DXS-XXXX:N#

### show ERPS

#### Purpose

Used to display the information of the ERPS.

#### Format

show erps

#### Description

This command is used to display ERPS configuration and operation information.

#### Parameters

None

#### Help message

Used to show the information of the ERPS.

#### Restrictions

User level.

#### Example

DXS-XXXX:N#show erps Command: show erps ERPS Informations ------------------------ ERPS Status : Enabled R-APS VLAN : 4094 West Port : 5 (Blocking) East Port : 7 (Forwarding) RPL port : West Port Owner : Enabled Protected VLANs : 100-200 Ring MEL : 2 Holdoff time : 0 ms Guard time : 500 ms WTR time : 5 minutes Log : Disabled Current Ring State : Protection DXS-XXXX:N#