ERPS topology

- 4 DUTs (A, B, C, D ) form a ring.

- DUT B is RPL_Owner; RAPS_VLAN is vid 3; ring_port : 3-4; RPL_PORT : west port ( port 3 ); Protected_Vlan : vid 3, 5.

- The other DUTs ( A, C, D ) are non-RPL_Owner.

D-------- A ---- rpl ---- B(rpl_owner) -------- C -------- D -------- A

RPL_Owner( B ) CLI configuration :

reset force_agree

config serial_port auto_logout never

create vlan vlanid 3

config vlan vlanid 3 add tagged 3-4

create vlan vlanid 5

config vlan vlanid 5 add tagged 5-6

create erps raps_vlan 3

config erps raps_vlan 3 ring_port west 3

config erps raps_vlan 3 ring_port east 4

config erps raps_vlan 3 protected_vlan add vlanid 5

config erps raps_vlan 3 protected_vlan add vlanid 1

config erps raps_vlan 3 rpl_port west

config erps raps_vlan 3 rpl_owner enable

enable erps

save

non-RPL_Owner( A, C, D ) CLI configuration :

reset force_agree

config serial_port auto_logout never

create vlan vlanid 3

config vlan vlanid 3 add tagged 3-4

create vlan vlanid 5

config vlan vlanid 5 add tagged 5-6

create erps raps_vlan 3

config erps raps_vlan 3 ring_port west 3

config erps raps_vlan 3 ring_port east 4

config erps raps_vlan 3 protected_vlan add vlanid 5

config erps raps_vlan 3 protected_vlan add vlanid 1

enable erps

save