How to Update the Firmware

Update the PROM code:

- Modify the configure file for PROM code updating

Specify the directory path and image file name for PROM code.

Code_type=PROM

Image_file="d:\des3624\3624prom.202"

#Code_type=RUNTIME

#Image_file="d:\switch\des3624\3624run.221"

#Code_type=CONFIG

#Image_file="g:\d-link\des-32~1\basic_25.bc"

# specify IP address

#Ip_addr = 10.9.68.25

# specify subnet mask

#Subnet_mask = 255.0.0.0

# specify default gateway

#Default_gateway = 10.254.254.254

- Enable your TFTP server.

- Enter console program; select Update Firmware and Configuration Files in the main menu.

- Set the Software Update Mode to “Network”.

- Specify the IP address of your TFTP server.

- Set Use Config Files to “Enable”

In File Name field, specify the path and file name of config file. The following screen appears:

- Press RESET SWITCH TO START UPDATE to initiate the update sequence.

- Restart the switch. Check the version of PROM code. is it the correct version you want to update?

Update the Runtime code:

- Modify the configure file for RunTime code updating

Specify the directory path and image file name for RunTime code.

#Code_type=PROM

#Image_file="d:\des3624\3624prom.202"

Code_type=RUNTIME

Image_file="d:\des3624\3624run.221"

#Code_type=CONFIG

#Image_file="g:\d-link\des-32~1\basic_25.bc"

# specify IP address

#Ip_addr = 10.9.68.25

# specify subnet mask

#Subnet_mask = 255.0.0.0

# specify default gateway

#Default_gateway = 10.254.254.254

- Enable your TFTP server.

- Enter console program; select Update Firmware and Configuration Files in the main menu.

- Set the Software Update Mode to “Network”.

- Specify the IP address of your TFTP server.

- Set Use Config Files to “Enable”

In File Name field, specify the path and file name of config file. The following screen appears:

- Press RESET SWITCH TO START UPDATE to initiate the update sequence.

- Restart the switch. Check the version of PROM code. is it the correct version you want to update?