|  | DES-5600 |
| --- | --- |
| Controller | MMC AF5400 Series chipset |
| Ports | Maximum of 64 Fast Ethernet 100Base-TX ports |
| Auto Negotiation | Yes. |
| Console Port | RS-232 DCE, 19200 |
| SLIP Supported | # Yes. |
| Port Trunking | 8 trunk groups, max 8 ports per group |
| VLAN IEEE802.1P supported | ## Not yet |
| MAC Base VLAN supported | Not yet |
| Flow Control | Yes. 802.1X and Back-Pressure |
| Spanning Tree Supported | Yes. IEEE 802.1D |
| Broadcast Storm supported | Not yet |
| IGMP Multicast support | Not yet |
| MIB Supported | - Bridge MIB (RFC 1493) - RMON MIB (RFC 1757) 4 groups - MIB-II (RFC 1213) - Repeater (RFC 1516) - Ethernet MIB (RFC 1643) - Proprietary MIB |
| Module | - 100Base-TX board: 16 ports - 100Base-FX board: 12 ports - 1000Base-SX board: 4 ports |
| Backplane | 20 Gbps |
| Buffer size | 3 MB per system |
| Address table size | 16K active MAC address entry |
| Architecture | Shared Memory |
| Packet Forwarding | Store and Forward |
| Firmware Upgrade | BOOTP/TFTP protocol |

### 16-TX/12-FX ports function

- 16 port 100Base-TX or 12 port 100Base-FX module

- MT-RJ small form factor transceivers on FX module.

- Auto-polarity detection and correction on 10/100M TP ports

- Auto-negotiation function provided on 10/100M TP ports

- IEEE802.3,802.3u compliance on 10/100M TP or FX ports

- Enable, speed, duplex, flow control settings could be configurable via the management agent.

- TP port default setting : auto-negotiation for speed / duplex / flow control

- FX port default setting : speed / duplex / flow control = 100M / full / enabled

### Gigabit port function:

- 4 port 1000Base-SX Gigabit module

- MT-RJ small form factor transceivers on SX module

- Support Full Duplex only

- Single or multi mode fibers are supported.

- Flow control with auto-negotiation or manual mode

### Management function:

- In bend SNMP management.

- Out of bend management.

- Compliant to IEEE802.1D Spanning Tree Protocol.

- SNMP and RMON (group 1,2,3,9)

- Supports Web Management

- Supports port based VLAN configuration

- Supports remote boot (Bootp) capability and boot configuration.

- Supports Telnet.

- Supports port/switch configuration

- Supports port/switch traffic monitoring

- Supports link aggregation/port trunking

- Supports security

- Supports traps

- Field upgradable management firmware by in bend or out of bend.

- Console port (EIA-232) support device configuration and local management.