# DES-6500 Firmware Release Note

Firmware: V2.50B12 (Runtime), V2.00B20 (Prom)

Hardware: A1 (DES-6500/6502/6505/6507/6509/6512)

Hardware: B1 (DES-6504/6508/6510)

Date: Aug 31, 2005

Enhancements:

- Support new Line card <DES-6512>

Firmware: V2.00B20 (Runtime), V2.00B20 (Prom)

Hardware: A1 (DES-6500/6502/6505/6507/6509)

Hardware: B1 (DES-6504/6508/6510)

Date: May 3, 2005

Enhancements:

- Support Floating Static Route

- Support “Show Config” command

- Support SSH

- Support Single IP Management

- Support Cisco Like Port Security

- Support Multiple IP interfaces per VLAN

- Support SSL

- Support TACACS/TACACS+ Authentication on switch admin access

- Support RADIUS Authentication on switch admin access

- Support CPU interface filtering

- Support 802.1X MAC-based Access Control

- Support DHCP Auto Configuration

- Support 802.1v (IPv6)

- Support 802.1S Multiple Spanning Tree support

- Support Be able to categorize log to 3 levels and to configure which level to trigger logging

- Support Static ARP entries enlarge from 32 to 255

- Enlarge static route (from 32 to 128)

- Enlarge IP interface number (from 32 to 256 more)

Warning:

- To work with R2.00-B20 runtime code, we suggest upgrade PROM code version to 2.00-B20.

- User MUST upgrade runtime code version of CPU card and Line card(s) to R2.00-B20 first, then user can upgrade PROM code to 2.00-B20 by TFTP. And ONLY support upgrade PROM code in CLI.

- If the runtime code version of the DES-6500 system is older than 1.30-B02, user MUST upgrade the runtime code to 1.30-B02 before upgrade it to 2.00-B20. In this version, the CPU card upgrading MUST be early than line cards.

Firmware: R1.40-B03 (Runtime), B1.50-B19 (Prom)

Hardware: A1

Date: March 29, 2005

Problems Resolved:

1. DES-6500 D-Link IQC 1.30B02 tested found items:

(1) IP Address Invalid  (IE, Safari, Firefox, Netscape)

(2) ACL select IP type has no effect  (Firefox, Netscape)

(3) ACL Invalid MAC Address (Firefox, Netscape)

2. Incorrect spelling in the web interface to both the DES-6500 and all switches in the xStack series.

Location: Configuration - Layer 3 IP Networking - Static/Default Route

Original incorrect spelling: "IP Adderss "

Correct spelling: "IP Address"

3. DES-6500" Vlan Name" is not displayed in website

The Vlan name in "Switch IP Settings" is not displayed. <- - - Problem

Web interface path: Configuration - - -> Switch IP Settings

Firmware: R1.30-B02 (Runtime), B1.50-B19 (Prom)

Hardware: A1

Date: January 24, 2005

Enhancements:

- Add DES-6504 Line card support

- Support Port description

- Support CPU utilization monitoring

- Support ACL - Best Case per port 100 ACL entries times port number

Warning:

1. The whole DES-6500 system must be upgraded to same version number of runtime code.

2. For all users, it is necessary that upgrade your system to R1.20-B09 before upgrade to R1.30-B02.

Firmware: R1.20-B09 (Runtime), B1.50-B19 (Prom)

Hardware: A1

Date: December 03, 2004

Problems Resolved:

- Multicast traffic was found packet loss while sending more than 210 multicast streams with wirespeed over PIM distribution.

- While distributing multicast traffic via DVMRP, packet will lose while sending wirespeed multicast streams and meanwhile there's IGMP join & leave.

- GVRP update wasn't complete if the networks were built in loop topology.

- Multicast streams were distributed via multicast routing protocol in L3 loop architecture that there'll be packet loss.

- GuangZhou HW Default Issue: While setting a default route, DES-6500 will learn each destination IP address and write it into ARP table. When ARP table is full, packets which destination is not learned will be send to CPU for software routing. This caused the CPU utilization high.

- GuangZhou Virus Attack Issue: Virus packets will be sent to CPU due to LookUp Fail. It makes DES-6500 CPU utilization very high. Even user set up ACL rules to filter this kind of packets, the CPU utilization is still high.

- WEB UI display error

- GVRP state can not save after saving configuration and reboot.

- WEB Display Scenario after device reboot and login again.

- DES-6500 (with DES-6507) Configure as L3 Switch has problem to perform NFS for file transfer between two difference subnet.

Enhancements:

- Add DES-6508 line card support

- Add DES-6510 line card support

- Support VRRP function

- Support HW default route

===== F/W Ver. 1.01-B40 (July, 2004) ==============================================

New features:

Add DES-6505 Line card support

Fixed:

- Multicast software routing

- PIM-DM multiple path, Link down the original path, it will not choose another one.

- Download configuration of 4k vlan, device will reboot

===== F/W Ver. 1.01-B26 (June, 2004) ==============================================

- First release version, support DES-6507/DES-6509 line cards. For the function/features, please refer to the Manual or datasheet.

- When upgrading, upgrade the line cards firmware and then CPU board firmware. Line cards and CPU board must have the same firmware version.