DES-6500 R1.30 Upgrade Instruction

1. About the version of PROM code:

This version of runtime code can work with different RPOM.  If the PROM versions between the CPU card and line cards or between line cards are not the same, you DO NOT need to upgrade the PROM.

2. DES-6500 supported line cards under each runtime code version:

| Runtime Code Version | Supported Line Card |
| --- | --- |
| R1.01-B37 | DES-6507/6509 |
| R1.01-B40 | DES-6505/6507/6509 |
| R1.20-B09 | DES-6505/6507/6508/6509/6510 |
| R1.30-B02 (this version) | DES-6504/6505/6507/6508/6509/6510 |

The model in Bold means new supported line cards. The new supported line card model(s) can be inserted into DES-6500 chassis only when the chassis (CPU card and line cards) has been upgraded to the supported version.  For example, DES-6504 can be inserted into chassis only when the chassis (CPU card and line cards) has been upgraded to R1.30-B02. If DES-6504 is inserted to chassis having R1.01B40 will result in DES-6500 system boot-up failure.

3. About the “S” or “U” in runtime code filename:

The DES-6500 has different runtime code files for CPU card and line cards.

6500rS_1.30-B02.had    	--> The letter 'S' indicates the line card.

6500rU_1.30-B02.had    	--> The letter 'U' indicates the CPU card.

Please specify the correct file name when upgrading for CPU or for Line card(s), respectively. In this version, the upgrading CPU code must be early than line cards’. This ordering is different from the procedure on R1.2, where upgrading line card is earlier than CPU card.  The DES-6500 CPU and line cards at same chassis MUST have same runtime version to work correctly.

4. A Special Case: Adding DES-6505/6507/6509 with R1.01 runtime to R1.30 chassis:

If DES-6500 chassis (CPU and Line cards) has been upgraded to R1.3 runtime but inserted DES-6505/6507/6509 line cards with R1.01 runtime, the DES-6504/6508/6510 cannot be recognized by DES-6500 system.  For this case, please un-plug the DES-6504/6508/6510 line cards first, upgrade the DES-6505/6507/6509 to R1.3, and then insert DES-6504/6508/6510 to recover.

5. Files: