D-Link DGS-1005D

5-Port 10/100/1000Mbps

Gigabit Ethernet Switch

Manual

Rev. 053003

Building Networks for People

RECYCLABLE

Table of Contents

About This Manual	1

Terms	1

Overview of this Manual	1

Introduction	2

Gigabit Ethernet Technology	2

Switching Technology	3

Features	4

Unpacking and Setup	5

Unpacking	5

Setup	5

Identifying External Components	6

Front Panel	6

Rear Panel	7

Mounting the Switch on a Wall	8

Placing The Switch in a stack	9

Technical Specifications	10

Warranty and Registration	12

# About This Manual

This Manual instructs you on the installation of your DGS-1005D 5-Port 1000BASE-T Gigabit Ethernet Switch and how to connect it to your Gigabit Ethernet network.

## Terms

For simplicity, this documentation uses the terms “Switch” (first letter upper case) to refer to the DGS-1005D 5-Port 1000BASE-T Gigabit Ethernet Switch, and “switch” (first letter lower case) to refer to all Ethernet switches, including the DGS-1005D  5-Port 1000BASE-T Gigabit Ethernet Switch.

# Overview of this Manual

Introduction. Describes the Switch and its features.

Unpacking and Setup. Helps you get started with the basic installation of the Switch.

Identifying External Components. Describes the front panel, rear panel and LED indicators of the Switch.

Technical Specifications. Lists all the technical specifications of the Switch.

# Introduction

This section describes the features of the DGS-1005D, as well as providing some background information about Gigabit Ethernet and switching technology.

## Gigabit Ethernet Technology

Gigabit Ethernet is an extension of IEEE 802.3 Ethernet utilizing the same packet structure, format, and support for CSMA/CD protocol, full duplex, flow control, and management objects, but with a tenfold increase in theoretical throughput over 100-Mbps Fast Ethernet and a hundredfold increase over 10-Mbps Ethernet. Since it is compatible with all 10-Mbps and 100-Mbps Ethernet environments, Gigabit Ethernet provides a straightforward upgrade without wasting a company’s existing investment in hardware, software, and trained personnel.

The increased speed and extra bandwidth offered by Gigabit Ethernet is essential to coping with the network bottlenecks that frequently develop as computers and their bus speeds get faster and more users use applications that generate more traffic. Upgrading key components, such as your backbone and servers to Gigabit Ethernet can greatly improve network response times as well as significantly speed up the traffic between your subnets.

Gigabit Ethernet supports video conferencing, complex imaging, and similar data-intensive applications. Likewise, since data transfers occur 10 times faster than Fast Ethernet, servers outfitted with Gigabit Ethernet NIC’s are able to perform 10 times the number of operations in the same amount of time.

## Switching Technology

Another key development pushing the limits of Ethernet technology is in the field of switching technology. A switch bridges Ethernet packets at the MAC address level of the Ethernet protocol transmitting among connected Ethernet or fast Ethernet LAN segments.

Switching is a cost-effective way of increasing the total network capacity available to users on a local area network. A switch increases capacity and decreases network loading by making it possible for a local area network to be divided into different segments which don’t compete with each other for network transmission capacity, giving a decreased load on each.

The switch acts as a high-speed selective bridge between the individual segments. Traffic that needs to go from one segment to another is automatically forwarded by the switch, without interfering with any other segments. This allows the total network capacity to be multiplied, while still maintaining the same network cabling and adapter cards.

Switching LAN technology is a marked improvement over the previous generation of network bridges, which were characterized by higher latencies.  Routers have also been used to segment local area networks, but the cost of a router and the setup and maintenance required make routers relatively impractical. Today’s switches are an ideal solution to most kinds of local area network congestion problems.

## Features

The DGS-1005D 5-Port 1000BASE-T Gigabit Ethernet Switch was designed for easy installation and high performance in an environment where traffic on the network and the number of users increase continuously.

- Five (5) 1000BASE-T Gigabit Ethernet ports

- Supports Auto-Negotiation for 10/100/1000Mbps and duplex mode

- Supports Auto-MDIX for each port

- Support Full/Half duplex transfer mode for 10 and 100Mbps

- Support Full duplex transfer mode for 1000Mbps

- Full wire speed reception and transmission

- Store-and-Forward switching method

- Supports 8K absolute MAC addresses

- Supports 256KBytes RAM for data buffering

- Extensive front-panel diagnostic LEDs

- IEEE 802.3x flow control for full-duplex

- Back pressure flow control for half-duplex

- Broadcast Storm Protection

# Unpacking and Setup

## Unpacking

Open the shipping carton of the Switch and carefully unpack its contents.  The carton should contain the following items:

1. One DGS-1005D 5-Port 1000BASE-T Gigabit Ethernet Switch

2. Four rubber feet with adhesive backing

3. One external power adapter

4. This Manual

If any item is missing or damaged, please contact your reseller for replacement.

## Setup

The setup of the Switch can be performed using the following steps:

Install the Switch in a fairly cool and dry place. See Technical Specifications for the acceptable operation temperature and humidity ranges.

Install the Switch in a site free from strong electromagnetic sources, vibration, dust, and direct sunlight.

Leave at least 4 in. (10cm) of space at the left and right hand side of the Switch for ventilation.

Visually inspect the DC power jack and make sure that it is fully secured to the power adapter.

# Identifying External Components

## Front Panel

The figure below shows the front display panel of the switch.

Front panel view of the Switch

LED Indicators

Comprehensive LED indicators display the conditions of the Switch and status of the network. The LED indicators of the Switch include Power, Link/Act, 1000Mbps and 100Mbps.

- Power Indicator

This green indicator illuminates when the Switch is receiving power.

- Link / Act

This green indicator illuminates steadily when a port is connected to a station successfully and has a good link. The indicator will blink to indicate that a port is transmitting or receiving data on the network.

- Speed — 100Mpbs/amber; 1000Mbps/green; 10Mbps/off

This indicator is amber-colored when the port is connected to a 100Mbps Fast Ethernet station. It is green when the port is connected to a 1000Mbps Ethernet station. It is not illuminated when the port is connected to a 10Mbps Ethernet station.

## Rear Panel

The rear panel of the Switch consists of five (5) Gigabit Ethernet, Auto-Negotiating ports and a DC power jack.

Rear panel view of the Switch

DC Power Jack:

Power is supplied through an external AC power adapter. Check the technical specification section for information about the AC power input voltage.

1000BASE-T Ports:

- Five (5) Gigabit Ethernet ports of 10/100/1000Mbps Auto-Negotiation interface.

- Comprehensive LED indicators display the conditions of the Switch and status of the network.

Mounting the Switch on a Wall

- 3/4 inch minimum for wood wall

(2)   3 inch minimum  for cement wall.

The DGS-1005D can also be mounted on a wall. Two mounting slots are provided on the bottom of the switch for this purpose. T3 x 15L screws and Nylon screw anchors are not provided. Please make sure that the front panel is exposed in order to view the LEDs. Please refer to the illustration below:

#### Mounting on a cement wall

  - Mount the Nylon screw anchors into a cement wall.

  - Drive the T3 x 15L screws into the Nylon screw anchors.

  - Hook the mounting holes of the switch back on the screws; you have completed the wall-mount.

#### Mounting on a wood wall

- Drive the T3 x 15 L screws into the wood wall.

- Hook the mounting holes of the switch back on the screws; you have

completed the wall-mount.

# Placing The Switch in a stack

If you intend to place the DGS-1005D Switch in a stack with other devices (see Figure 1), please adhere to the following guidelines:

- Use rubber feet provided. Position and apply rubber feet to the underside of the DGS-1008D Switch (see Figure 2).

- Place the Switch on top of the stack of devices. When stacking devices together, place the DGS-1008D Switch on the top of all other devices in the stack (see Figure 3)

- .

- Do not place any device on top of Switch. For the Switch to perform at its optimal level, do not stack any other device on top of the DGS-1008D Switch. This allows the Switch to have adequate ventilation. All other devices in the stack should go underneath the DGS-1008D Switch.

# Technical Specifications

| General |  |
| --- | --- |
| Standards: | IEEE 802.3ab 1000BASE-T IEEE 802.3u 100BASE-TX IEEE 802.3 10BASE-T IEEE 802.3x Flow Control |
| Protocol: | CSMA/CD |
| Data Transfer Rate: | Ethernet:; 10Mbps; (Half-duplex). 20Mbps; (Full-duplex). Fast Ethernet:; 100Mbps; (Half-duplex). 200Mbps; (Full-duplex). Gigabit Ethernet:; 2000Mbps; (Full-duplex). |
| Topology: | Star |
| Network Cables: | Ethernet:; 2-pair UTP Cat. 3,4,5, Unshield Twisted Pair (UTP )Cable. Fast Ethernet:; 2-pair UTP Cat. 5, Unshield Twisted Pair (UTP )Cable. Gigabit Ethernet:; 4-pair UTP Cat. 5, Unshield Twisted Pair (UTP )Cable. |
| Number of Ports: | Five(5) 1000BASE-T Gigabit Ethernet ports |

| Physical and Environmental |  |
| --- | --- |
| DC inputs: | 5V / 2.5A |
| Power Consumption: | 9 watts maximum |
| Operating Temperature: | 32F ~ 131F  (0 C ~ 55℃) |
| Storage Temperature: | 14F ~ 158F (-10C ~ 70C) |
| Humidity: | 5% ~ 95% RH, non-condensing |
| Dimensions: | 7.58 in. x 4.67 in. x 1.26 in. (192.5mm x 118.5mm x 32mm) |
| EMI | FCC Class A, CE Mark Class A, VCCI Class A |
| Safety: | cUL (UL 60950), LVD (EC 60950) |
| Performance |  |
| Transmission Method: | Store-and-forward |
| RAM Buffer: | 256K Bytes per device |
| Filtering Address Table: | 8K MAC address per device |
| Packet Filtering/Forwarding Rate: | Full wire speed |
| MAC Address Learning: | Self-learning, auto-aging |

# Warranty and Registration

Subject to the terms and conditions set forth herein, D-Link Systems, Inc. (“D-Link”) provides this Limited warranty for its product only to the person or entity that originally purchased the product from:

- D-Link or its authorized reseller or distributor

- Products purchased and delivered within the fifty states of the United States, the District of Columbia, U.S. Possessions or Protectorates, U.S. Military Installations, addresses with an APO or FPO.

Limited Warranty: D-Link warrants that the hardware portion of the D-Link products described below will be free from material defects in workmanship and materials from the date of original retail purchase of the product, for the period set forth below applicable to the product type (“Warranty Period”), except as otherwise stated herein.

5-Year Limited Warranty for the Product(s) is defined as follows:

- Hardware (excluding power supplies and fans) Five (5) Years

- Power Supplies and Fans Three (3) Year

- Spare parts and spare kits Ninety (90) days

D-Link’s sole obligation shall be to repair or replace the defective Hardware during the Warranty Period at no charge to the original owner or to refund at D-Link’s sole discretion.  Such repair or replacement will be rendered by D-Link at an Authorized D-Link Service Office.  The replacement Hardware need not be new or have an identical make, model or part.  D-Link may in its sole discretion replace the defective Hardware (or any part thereof) with any reconditioned product that D-Link reasonably determines is substantially equivalent (or superior) in all material respects to the defective Hardware.  Repaired or replacement Hardware will be warranted for the remainder of the original Warranty Period from the date of original retail purchase.  If a material defect is incapable of correction, or if D-Link determines in its sole discretion that it is not practical to repair or replace the defective Hardware, the price paid by the original purchaser for the defective Hardware will be refunded by D-Link upon return to D-Link of the defective Hardware.  All Hardware (or part thereof) that is replaced by D-Link, or for which the purchase price is refunded, shall become the property of D-Link upon replacement or refund.

Limited Software Warranty: D-Link warrants that the software portion of the product (“Software”) will substantially conform to D-Link’s then current functional specifications for the Software, as set forth in the applicable documentation, from the date of original retail purchase of the Software for a period of ninety (90) days (“Warranty Period”), provided that the Software is properly installed on approved hardware and operated as contemplated in its documentation. D-Link further warrants that, during the Warranty Period, the magnetic media on which D-Link delivers the Software will be free of physical defects.  D-Link’s sole obligation shall be to replace the non-conforming Software (or defective media) with software that substantially conforms to D-Link’s functional specifications for the Software or to refund at D-Link’s sole discretion.  Except as otherwise agreed by D-Link in writing, the replacement Software is provided only to the original licensee, and is subject to the terms and conditions of the license granted by D-Link for the Software.  Software will be warranted for the remainder of the original Warranty Period from the date or original retail purchase.  If a material non-conformance is incapable of correction, or if D-Link determines in its sole discretion that it is not practical to replace the non-conforming Software, the price paid by the original licensee for the non-conforming Software will be refunded by D-Link; provided that the non-conforming Software (and all copies thereof) is first returned to D-Link.  The license granted respecting any Software for which a refund is given automatically terminates.

Non-Applicability of Warranty: The Limited Warranty provided hereunder for hardware and software of D-Link's products will not be applied to and does not cover any refurbished product and any product purchased through the inventory clearance or liquidation sale or other sales in which D-Link, the sellers, or the liquidators expressly disclaim their warranty obligation pertaining to the product and in that case, the product is being sold "As-Is" without any warranty whatsoever including, without limitation, the Limited Warranty as described herein, notwithstanding anything stated herein to the contrary.

Submitting A Claim: The customer shall return the product to the original purchase point based on its return policy.  In case the return policy period has expired and the product is within warranty, the customer shall submit a claim to D-Link as outlined below:

- The customer must submit with the product as part of the claim a written description of the Hardware defect or Software nonconformance in sufficient detail to allow D-Link to confirm the same.

- The original product owner must obtain a Return Material Authorization (“RMA”) number from the Authorized D-Link Service Office and, if requested, provide written proof of purchase of the product (such as a copy of the dated purchase invoice for the product) before the warranty service is provided.

- After an RMA number is issued, the defective product must be packaged securely in the original or other suitable shipping package to ensure that it will not be damaged in transit, and the RMA number must be prominently marked on the outside of the package.  Do not include any manuals or accessories in the shipping package.  D-Link will only replace the defective portion of the Product and will not ship back any accessories.

- The customer is responsible for all in-bound shipping charges to D-Link.  No Cash on Delivery (“COD”) is allowed.  Products sent COD will either be rejected by D-Link or become the property of D-Link.  Products shall be fully insured by the customer and shipped to D-Link Systems, Inc., 53 Discovery Drive, Irvine, CA 92618.  D-Link will not be held responsible for any packages that are lost in transit to D-Link.  The repaired or replaced packages will be shipped to the customer via UPS Ground or any common carrier selected by D-Link, with shipping charges prepaid. Expedited shipping is available if shipping charges are prepaid by the customer and upon request.

D-Link may reject or return any product that is not packaged and shipped in strict compliance with the foregoing requirements, or for which an RMA number is not visible from the outside of the package. The product owner agrees to pay D-Link’s reasonable handling and return shipping charges for any product that is not packaged and shipped in accordance with the foregoing requirements, or that is determined by D-Link not to be defective or non-conforming.

What Is Not Covered: This limited warranty provided by D-Link does not cover: Products, if in D-Link’s judgment, have been subjected to abuse, accident, alteration, modification, tampering, negligence, misuse, faulty installation, lack of reasonable care, repair or service in any way that is not contemplated in the documentation for the product, or if the model or serial number has been altered, tampered with, defaced or removed; Initial installation, installation and removal of the product for repair, and shipping costs; Operational adjustments covered in the operating manual for the product, and normal maintenance; Damage that occurs in shipment, due to act of God, failures due to power surge, and cosmetic damage;  Any hardware, software, firmware or other products or services provided  by anyone other than D-Link; Products that have been purchased from inventory clearance or liquidation sales or other sales in which D-Link, the sellers, or the liquidators expressly disclaim their warranty obligation pertaining to the product.  Repair by anyone other than D-Link or an Authorized D-Link Service Office will void this Warranty.

Disclaimer of Other Warranties: EXCEPT FOR THE LIMITED WARRANTY SPECIFIED HEREIN, THE PRODUCT IS PROVIDED “AS-IS” WITHOUT ANY WARRANTY OF ANY KIND WHATSOEVER INCLUDING, WITHOUT LIMITATION, ANY WARRANTY OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NON-INFRINGEMENT. IF ANY IMPLIED WARRANTY CANNOT BE DISCLAIMED IN ANY TERRITORY WHERE A PRODUCT IS SOLD, THE DURATION OF SUCH IMPLIED WARRANTY SHALL BE LIMITED TO NINETY (90) DAYS. EXCEPT AS EXPRESSLY COVERED UNDER THE LIMITED WARRANTY PROVIDED HEREIN, THE ENTIRE RISK AS TO THE QUALITY, SELECTION AND PERFORMANCE OF THE PRODUCT IS WITH THE PURCHASER OF THE PRODUCT.

Limitation of Liability: TO THE MAXIMUM EXTENT PERMITTED BY LAW, D-LINK IS NOT LIABLE UNDER ANY CONTRACT, NEGLIGENCE, STRICT LIABILITY OR OTHER LEGAL OR EQUITABLE THEORY FOR ANY LOSS OF USE OF THE PRODUCT, INCONVENIENCE OR DAMAGES OF ANY CHARACTER, WHETHER DIRECT, SPECIAL, INCIDENTAL OR CONSEQUENTIAL (INCLUDING, BUT NOT LIMITED TO, DAMAGES FOR LOSS OF GOODWILL, LOSS OF REVENUE OR PROFIT, WORK STOPPAGE, COMPUTER FAILURE OR MALFUNCTION, FAILURE OF OTHER EQUIPMENT OR COMPUTER PROGRAMS TO WHICH D-LINK’S PRODUCT IS CONNECTED WITH, LOSS OF INFORMATION OR DATA CONTAINED IN, STORED ON, OR INTEGRATED WITH ANY PRODUCT RETURNED TO D-LINK FOR WARRANTY SERVICE) RESULTING FROM THE USE OF THE PRODUCT, RELATING TO WARRANTY SERVICE, OR ARISING OUT OF ANY BREACH OF THIS LIMITED WARRANTY, EVEN IF D-LINK HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES. THE SOLE REMEDY FOR A BREACH OF THE FOREGOING LIMITED WARRANTY IS REPAIR, REPLACEMENT OR REFUND OF THE DEFECTIVE OR NON-CONFORMING PRODUCT.  THE MAXIMUM LIABILITY OF D-LINK UNDER THIS WARRANTY IS LIMITED TO THE PURCHASE PRICE OF THE PRODUCT COVERED BY THE WARRANTY. THE FOREGOING EXPRESS WRITTEN WARRANTIES AND REMEDIES ARE EXCLUSIVE AND ARE IN LIEU OF ANY OTHER WARRANTIES OR REMEDIES, EXPRESS, IMPLIED OR STATUTORY

Governing Law: This Limited Warranty shall be governed by the laws of the State of California. Some states do not allow exclusion or limitation of incidental or consequential damages, or limitations on how long an implied warranty lasts, so the foregoing limitations and exclusions may not apply. This limited warranty provides specific legal rights and the product owner may also have other rights which vary from state to state.

Trademarks: D-Link is a registered trademark of D-Link Systems, Inc. Other trademarks or registered trademarks are the property of their respective manufacturers or owners.

Copyright Statement: No part of this publication or documentation accompanying this Product may be reproduced in any form or by any means or used to make any derivative such as translation, transformation, or adaptation without permission from D-Link Corporation/D-Link Systems, Inc., as stipulated by the United States Copyright Act of 1976.  Contents are subject to change without prior notice.  Copyright© 2002 by D-Link Corporation/D-Link Systems, Inc. All rights reserved.

CE Mark Warning: This is a Class B product. In a domestic environment, this product may cause radio interference, in which case the user may be required to take adequate measures.

FCC Statement: This equipment has been tested and found to comply with the limits for a Class A digital device, pursuant to part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation. This equipment generates, uses, and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communication. However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

- Reorient or relocate the receiving antenna.

- Increase the separation between the equipment and receiver.

- Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

- Consult the dealer or an experienced radio/TV technician for help.

For detailed warranty outside the United States, please contact corresponding local D-Link office.

Register online your D-Link product at http://support.dlink.com/register/