| Firmware Version: | 3.12.B039 |
| --- | --- |
| Boot Code Version: | 1.00.013 |
| MIB Version: | 3.12.B004 |
| D-View Module Version: | 3.0.0.6 |
| Published: | Sep. 18, 2015 |

These release notes include important information about D-Link switch firmware revisions. Please verify that these release notes are correct for your switch:

- If you are installing a new switch, please check the hardware version on the device label; make sure that your switch meets the system requirement of this firmware version. Please refer to Revision History and System Requirement for detailed firmware and hardware matrix.

- If the switch is powered on, you can check the hardware version by typing “show switch” command via Telnet or by checking the device information page on the web graphic user interface.

- If you plan to upgrade to the new firmware release, please refer to the Upgrade Instructions for the correct firmware upgrade procedure.

For more detailed information regarding our switch products, please refer to Error: Reference source not found.

You can also download the switch firmware, D-View modules and technical documentation from http://tsd.dlink.com.tw.

Content:

Upgrade Instructions:	2

Upgrade using CLI (via Telnet)	2

Upgrade using Web-UI	3

Upgrade using SmartConsole Utility	5

New Features:	8

Changes of MIB & D-View Module:	9

Changes of Command Line Interface:	10

Problem Fixed:	11

Known Issues:	12

Related Documentation:	13

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| Runtime: v3.12.B039 Boot: 1.00.013 | 18- Sep.-15 | DGS-1210-28P | B1 |
| Runtime: v3.12.B038 Boot: 1.00.013 | 04- Sep.-15 | DGS-1210-28P | B1 |
| Runtime: v3.12.B037 Boot: 1.00.013 | 20- Aug.-15 | DGS-1210-28P | B1 |
| Runtime: v3.12.B035 Boot: 1.00.013 | 14- Jul.-15 | DGS-1210-28P | B1 |
| Runtime: v3.12.B032 Boot: 1.00.013 | 20- May.-14 | DGS-1210-28P | B1 |
| Runtime: v3.12.B025 Boot: 1.00.013 | 03- Dec.-14 | DGS-1210-28P | B1 |
| Runtime: v3.12.B024 Boot: 1.00.013 | 04- Nov.-14 | DGS-1210-28P | B1 |
| Runtime: v3.12.B023 Boot: 1.00.013 | 11- Sep.-14 | DGS-1210-28P | B1 |
| Runtime: v3.12.B020 Boot: 1.00.013 | 02- Sep.-14 | DGS-1210-28P | B1 |
| Runtime: v3.12.B019 Boot: 1.00.013 | 22- Aug.-14 | DGS-1210-28P | B1 |
| Runtime: v3.12.B018 Boot: 1.00.013 | 1- Jul.-14 | DGS-1210-28P | B1 |
| Runtime: v3.12.B017 Boot: 1.00.013 | 10- Jun.-14 | DGS-1210-28P | B1 |
| Runtime: v3.12.B016 Boot: 1.00.013 | 16- Mar.-14 | DGS-1210-28P | B1 |
| Runtime: v3.12.B014 Boot: 1.00.013 | 11- Mar.-14 | DGS-1210-28P | B1 |
| Runtime: v3.12.B013 Boot: 1.00.013 | 7- Mar.-14 | DGS-1210-28P | B1 |
| Runtime: v3.12.B011 Boot: 1.00.012 | 17- Feb.-14 | DGS-1210-28P | B1 |
| Runtime: v3.12.B010 Boot: 1.00.012 | 12- Feb.-14 | DGS-1210-28P | B1 |
| Runtime: v3.12.B009 Boot: 1.00.012 | 06- Feb.-14 | DGS-1210-28P | B1 |
| Runtime: v3.12.B007 Boot: 1.00.012 | 18- Dec.-13 | DGS-1210-28P | B1 |
| Runtime: v3.12.B005 Boot: 1.00.009 | 13- Dec.-13 | DGS-1210-28P | B1 |
| Runtime: v3.12.B004 Boot: 1.00.009 | 09- Dec.-13 | DGS-1210-28P | B1 |
| Runtime: v3.12.003 Boot: 1.00.009 | 28- Nov.-13 | DGS-1210-28P | B1 |
| Runtime: v3.11.B006 Boot: 1.00.009 | 01- Nov.-13 | DGS-1210-28P | B1 |
| Runtime: v3.11.B005 Boot: 1.00.009 | 24- Oct. -13 | DGS-1210-28P | B1 |
| Runtime: v3.11.B004 Boot: 1.00.009 | 30- Aug.-13 | DGS-1210-28P | B1 |
| Runtime: v3.11.B003 Boot: 1.00.009 | 20- Aug.-13 | DGS-1210-28P | B1 |
| Runtime: v3.11.B002 Boot: 1.00.009 | 07- Aug.-13 | DGS-1210-28P | B1 |
| Runtime: v3.11.001 Boot: 1.00.009 | 26- Jun.-13 | DGS-1210-28P | B1 |
| Runtime: V3.10.B028 Boot: 1.00.009 | 03- Jun.-13 | DGS-1210-28P | B1 |
| Runtime: V3.10.B025 Boot: 1.00.009 | 09- Apr.-13 | DGS-1210-28P | B1 |
| Runtime: V3.10.B022 Boot: 1.00.009 | 02- Apr.-13 | DGS-1210-28P | B1 |
| Runtime: V3.10.B018 Boot: 1.00.009 | 15- Mar.-13 | DGS-1210-28P | B1 |
| Runtime: V3.10.B016 Boot: 1.00.009 | 16- Jan.-13 | DGS-1210-28P | B1 |
| Runtime: V3.10.B015 Boot: 1.00.009 | 11- Jan.-13 | DGS-1210-28P | B1 |
| Runtime: v3.10.013 Boot: 1.00.009 | 03- Oct.-12 | DGS-1210-20 | B1 |
| Runtime: v3.10.011 Boot: 1.00.009 | 19- Sep.-12 | DGS-1210-28P | B1 |

# Upgrade Instructions:

D-Link Smart Switches support firmware upgrade via TFTP server. You can download the firmware from D-Link web site http://tsd.dlink.com.tw, and copy the downloaded firmware to the TFTP server folder. Please make sure that the TFTP server is accessible from the switch via networks.

## Upgrade using CLI (via Telnet)

1. Make sure the network connection between the switch and PC is active.

2. Use software that supports telnet, for example, HyperTerminal or Telnet command in Microsoft Windows, to connect to the switch. If you are using Telnet command, type the command followed by the switch IP address, eg. telnet 10.90.90.90.

3. The logon prompt will appear.

The switch will prompt the user to enter his/her username and password. It should be noted that upon the initial connection, both the default user name and password are admin.

To upgrade the switch firmware, execute the following command:

| Command | Function |
| --- | --- |
| download{firmware_fromTFTP tftp://ip-address/filename ¦ cfg_fromTFTP tftp://ip-address/filename} | Download firmware file from the TFTP server to the switch. |

When completing firmware upgrade, the following messages will pop up.

Device will reboot after firmware upgraded successfully

Image upgraded successfully

4. Execute the following command to check the firmware version and switch’s information.

| Command | Function |
| --- | --- |
| show switch | Display the information of current firmware and boot version. |

Example:

- DGS-1210-28P:

Command: download firmware_fromTFTP tftp://10.90.90.91 DGS-1210-20 B1 3_00_012.hex

Device will reboot after firmware upgraded successfully

Image upgraded successfully

- DGS-1210-28P:

Command: show switch

DGS-1210-28P> show switch

System name                       :

System Contact                    :

System Location                   :

System up time                    : 0 days, 1 hrs, 8 min, 36 secs

System Time                       : 01/01/2009 01:58:42

System hardware version           : B1

System firmware version           : 3.12.B007

System boot version               : 1.00.012

System Protocol version           : 2.001.004

System serial number              : LABDGS1220008

MAC Address                       : 00-12-20-00-08-00

Upgrade using Web-UI

- Connect a workstation installed with java SE runtime environment to any switch port of the device.

- Open the web browser from the workstation and enter the IP address of the switch. The switch’s default IP address is 10.90.90.90.

- Enter administrator’s password when prompted. The password is admin by default.

- To update switch’s firmware or configuration file, select Tools > Firmware Backup & Upgrade from the banner.

v

- Two methods can be selected : HTTP or TFTP

- Select HTTP to upgrade the firmware to or from your local drive of PC.

- Click Browse to browse your inventories for a saved firmware file

- Click Upgrade after selecting the firmware file you want to restore

- Click OK to continue with firmware upgrade

- Click OK to continue with firmware upgrade

- Wait until the “Firmware Upgrade Successful” message pops up and login again after device boots up.

- Select TFTP to upgrade the firmware to or from a remote TFTP server.

- Enter the name of the firmware file located on the TFTP server

- Click Upgrade after selecting the firmware file you want to restore

- Click OK to continue with firmware upgrade

- Wait until the firmware upgrade ends and login again after device boots up.

Upgrade using SmartConsole Utility

1. Connect a workstation installed with java SE runtime environment to any switch port of the device

2. Execute SmartConsole Utility

  - Click Discovery button to search target switch

  - Single click the icon of Monitor column to choose the target switch

5. Click Firmware Upgrade button

6. Click Browse button and select the firmware file (Model name_HW ver._FW ver. .hex) on your local hard drive.

  - Enter password and click “Upgrade” button to start firmware upgrade. The default password is “admin”.

  - Once the message popped up, click “OK” button to complete the firmware upgrade

# New Features:

| Firmware Version | New Features |
| --- | --- |
| V3.12.B039 | N/A |
| V3.12.B038 | N/A |
| V3.12.B037 | N/A |
| V3.12.B035 | N/A |
| V3.12.B032 | N/A |
| V3.12.B025 | N/A |
| V3.12.B024 | N/A |
| V3.12.B023 | N/A |
| V3.12.B020 | N/A |
| V3.12.B019 | N/A |
| V3.12.B018 | N/A |
| V3.12.B017 | N/A |
| V3.12.B016 | N/A |
| V3.12.B014 | N/A |
| V3.12.B013 | N/A |
| V3.12.B011 | N/A |
| V3.12.B010 | N/A |
| V3.12.B009 | N/A |
| V3.12.B007 | N/A |
| V3.12.B005 | N/A |
| V3.12.B004 | N/A |
| V3.12.003 | N/A |
| V3.11.B006 | N/A |
| V3.11.B005 | N/A |
| V3.11.B004 | N/A |
| V3.11.B003 | N/A |
| V3.11.B002 | N/A |
| V3.11.001 | 1. Add UI for D-Link discover protocol setting. |
| V3.10.B028 | N/A |
| V3.10.B025 | N/A |
| V3.10.B022 | 1. Add DHCP relay and DHCP local relay. |
| V3.10.B018 | N/A |
| V3.10.B016 | N/A |
| V3.10.B015 | N/A |
| V3.10.013 | N/A |
| V3.10.011 | First Release |

# Changes of MIB & D-View Module:

The new features of MIB file are also included in the corresponding D-View module. Please download the D-View module from http://tsd.dlink.com.tw. For detailed changes of MIB content, please refer to the modification history in each MIB file.

| Firmware Version | MIB File |  | New Features |
| --- | --- | --- | --- |
| V3.12.B039 | DGS-1210-28P-BX-3-12-B004.mib | N/A |  |
| V3.12.B038 | DGS-1210-28P-BX-3-12-B004.mib | N/A |  |
| V3.12.B037 | DGS-1210-28P-BX-3-12-B004.mib | N/A |  |
| V3.12.B035 | DGS-1210-28P-BX-3-12-B004.mib | N/A |  |
| V3.12.B032 | DGS-1210-28P-BX-3-12-B004.mib | N/A |  |
| V3.12.B025 | DGS-1210-28P-BX-3-12-B004.mib | N/A |  |
| V3.12.B024 | DGS-1210-28P-BX-3-12-B004.mib | N/A |  |
| V3.12.B023 | DGS-1210-28P-BX-3-12-B004.mib | N/A |  |
| V3.12.B020 | DGS-1210-28P-BX-3-12-B004.mib | N/A |  |
| V3.12.B019 | DGS-1210-28P-BX-3-12-B004.mib | N/A |  |
| V3.12.B018 | DGS-1210-28P-BX-3-12-B004.mib | N/A |  |
| V3.12.B017 | DGS-1210-28P-BX-3-12-B004.mib | N/A |  |
| V3.12.B016 | DGS-1210-28P-BX-3-12-B004.mib | N/A |  |
| V3.12.B014 | DGS-1210-28P-BX-3-12-B004.mib | N/A |  |
| V3.12.B013 | DGS-1210-28P-BX-3-12-B003.mib | Add poePortForcePDDetect object for 59111 IOT issue with HIKVision Camera on DES-1210-28P DGS-1210-10P and DGS-1210-28P |  |
| V3.12.B011 | DGS-1210-28P-BX-3-12-002.mib | N/A |  |
| V3.12.B010 | DGS-1210-28P-BX-3-12-002.mib | N/A |  |
| V3.12.B009 | DGS-1210-28P-BX-3-12-002.mib | N/A |  |
| V3.12.B007 | DGS-1210-28P-BX-3-12-002.mib | N/A |  |
| V3.12.B005 | DGS-1210-28P-BX-3-12-002.mib | N/A |  |
| V3.12.B004 | DGS-1210-28P-BX-3-12-002.mib | N/A |  |
| V3.12.003 | DGS-1210-28P-BX-3-12-002.mib | 1.Add object “aclL2RulePriority”, “aclL2RuleReplacePriority”, “aclL3RulePriority”, “aclL3RuleReplacePriority”, “aclv6L3RulePriority” and “aclv6L3RuleReplacePriority” for ACL. |  |
| V3.11.B006 | DGS-1210-28P-BX-3-11-001.mib | N/A. |  |
| V3.11.B005 | DGS-1210-28P-BX-3-11-001.mib | N/A. |  |
| V3.11.B004 | DGS-1210-28P-BX-3_11_001.mib |  | N/A |
| V3.11.B003 | DGS-1210-28P-BX-3_11_001.mib |  | N/A |
| V3.11.B002 | DGS-1210-28P-BX-3_11_001.mib |  | N/A |
| V3.11.001 | DGS-1210-28P-BX-3_11_001.mib |  | 1. Add DDP settings. |
| V3.10.B028 | DGS-1210-20-BX-3_10_009.mib |  | N/A |
| V3.10.B025 | DGS-1210-20-BX-3_10_009.mib |  | N/A |
| V3.10.B022 | DGS-1210-20-BX-3_10_009.mib |  | 1. Add DHCP relay and DHCP local relay |
| V3.10.B018 | DGS-1210-20-BX-3_10_008.mib |  | N/A |
| V3.10.B016 | DGS-1210-20-BX-3_10_008.mib |  | N/A |
| V3.10.B015 | DGS-1210-20-BX-3_10_008.mib |  | N/A |
| V3.10.013 | DGS-1210-20-BX-3_10_008.mib |  | N/A |
| V3.10.011 | DGS-1210-20-BX-3_10_008.mib |  | First Release |

| Firmware Version | D-View Module File |  | New Features |
| --- | --- | --- | --- |
| V3.12.B039 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B038 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B037 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B035 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B032 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B025 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B024 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B023 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B020 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B019 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B018 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B017 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B016 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B014 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B013 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B011 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B010 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B009 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B007 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B005 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.B004 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.12.003 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.11.B006 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.11.B005 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.11.B004 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.11.B003 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.11.B002 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.11.001 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.10.B028 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.10.B025 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.10.B022 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.10.B018 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.10.B016 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.10.B015 | DV_DGS-1210-28P_V3.0.0.6_FW_v3.10.011.exe |  | N/A |
| V3.10.013 | DV_DGS-1210-20_V3.0.0.6_FW_v3.10.011.exe | N/A |  |
| V3.10.011 | DV_DGS-1210-20_V3.0.0.6_FW_v3.10.011.exe |  | First Release |

# Changes of Command Line Interface:

The section below only shows command line changes that may bring backward compatibility issues with configuration settings for previous version of firmware.

Any new feature commands that do not have backward compatibility issues are not included in the below section.

| Firmware Version | Changes |
| --- | --- |
| V3.12.B039 | N/A |
| V3.12.B038 | 1.LLDP/LLDP-MED default disabled. 2.Support 2-Event Physical Layer classification at high power port. |
| V3.12.B037 | N/A |
| V3.12.B035 | N/A |
| V3.12.B032 | N/A |
| V3.12.B025 | N/A |
| V3.12.B024 | N/A |
| V3.12.B023 | N/A |
| V3.12.B020 | N/A |
| V3.12.B019 | N/A |
| V3.12.B018 | N/A |
| V3.12.B017 | N/A |
| V3.12.B016 | N/A |
| V3.12.B014 | N/A |
| V3.12.B013 | N/A |
| V3.12.B011 | N/A |
| V3.12.B010 | N/A |
| V3.12.B009 | N/A |
| V3.12.B007 | N/A |
| V3.12.B005 | N/A |
| V3.12.B004 | N/A |
| V3.12.003 | N/A |
| V3.11.B006 | N/A |
| V3.11.B005 | N/A |
| V3.11.B004 | N/A |
| V3.11.B003 | N/A |
| V3.11.B002 | N/A |
| V3.11.001 | N/A |
| V3.10.B028 | N/A |
| V3.10.B025 | N/A |
| V3.10.B022 | N/A |
| V3.10.B018 | N/A |
| V3.10.B016 | N/A |
| V3.10.B015 | N/A |
| V3.10.013 | N/A |
| V3.10.011 | First Release |

# Problem Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V3.12.B039 | - [DRU20150907000005-Russia]DGS-1210 rev.B1 reboots by some special URL. - [DRU20150903000003-Russia]DES-1210-28P issue with adding port to vlan. |
| V3.12.B038 | N/A |
| V3.12.B037 | 1. Fix [HQ20150814000010]DGS-1210-10P the Admin password can be changed by everybody without knowing the Admin password. [DEUR20150813000005-Central Europe] |
| V3.12.B035 | 1. Fix issue: Fix [HQ20150522000015] DHCP Local Relay and unicast packets [DRU20150521000002-Russia]. |
| V3.12.B032 | 1. Fix [HQ20150331000019] DES series & DGS series issue with unicast DHCP packets [DRU20150325000001-Russia]. |
| V3.12.B025 | 1. [DGC20141124000002-Taiwan] DHCP Sever Screening problem. |
| V3.12.B024 | - [DRU20141016000003-Russia] DGS-1210-10P issue with DHCP Local Relay. |
| V3.12.B023 | - [DGC20140801000004-Taiwan] DGS-1210-28 can’t show the specific igmp group. - DI20130913000003[DI20140825000007-Japan]About Link change trap which is related to case. |
| V3.12.B020 | 1. Fix incorrect model on SSL certificate. |
| V3.12.B019 | 1. OpenSSL before 0.9.8za, which allows man-in-the-middle attack, and consequently hijack sessions or obtain sensitive information. 2. [DGC20140801000004-Taiwan] DGS-1210-28 can’t show the specific igmp group. |
| V3.12.B018 | 1. [DRU20140618000002-Russia] DGS-1210 WebSmart DHCP relay issue. |
| V3.12.B017 | 1. [DI20140530000002-Japan] Wrong Sys up time / same as DI20140205000006. 2. [DRU20140523000001-Russia] DGS-1210 snmp port speed issue. |
| V3.12.B016 | 1. [DEUR20140429000003-Eastern Europe] RFC3580 radius attribute mismatch. 2. [DRU20140512000003-Russia] DGS-1500-20 issue with device reboot. |
| V3.12.B014 | PoE Delay Power Detect MIB item rename and google browser display issue |
| V3.12.B013 | - [DES-1210 smart binding issue[DRU20140218000001-Russia]] - 59111 IOT issue with HIKVision Camera Please be noted, first time PoE mcu upgrade will cause initialize time delay about 6-7 minutes |
| V3.12.B011 | After inject the traffic,VLAN Summary entry still none. |
| V3.12.B010 | [DRU20140120000004]DES-1210-28 issue with ACL. |
| V3.12.B009 | 1. Add New default MAC (B0C5540 ~ B0C5547) for ASV 2. Fixed system up time issue. [DI20140205000006-Japan]. |
| V3.12.B007 | 1. [DI20131211000003-Japan] DGS-1210 reboot with loop. 2. Fixed bug for DUT still enters power saving status when system time meets the time period of assigned profile but not selected weekdays. |
| V3.12.B005 | 1. HQ20131128000015 [DEUR20131118000008-North Europe] Fixed Time profile select week day could be over night. |
| V3.12.B004 | 1. HQ20131128000015 [DEUR20131118000008-North Europe] Fixed Time profile could be over night. |
| V3.12.003 | 1. Enhance DDP to 0.24 2. ACL and QOS page modify. 3. Modify D-Track support case [DRU20131101000002-Russia]Fix telnet issue that DUT with reboot during telnet session. 4. Modify D-Track support case [DI20131031000001-Japan]:DGS-1210 does send frames of loopdetect. 5. Modify D-Track support case [DGC20131016000004-Taiwan] DGS-1210-52 with recovery system issue. 6. Fix Cameo Bug#:0027770: “Unable to set the replace priority” will pop up when editing ACL IPv6 Class rule.. 7. Add command “debug information” for Telnet / http://XX.XX.XX.XX/kernel_crash.txt for GUI. |
| V3.11.B006 | 1. [DI20131023000005-Japan] RSTP switch back will cause network disconnection. |
| V3.11.B005 | - [ Support Request ] Fix Ping / Login Failure issue in SmartSW series. Note : After confirm with D-LINK Shan, this issue can not find out the correct D-track no. and will keep asking the result for the latest firmware. - [DEUR20131008000003-Central Europe] Fixed not able to set STP external cost on Port 9&10 in LAG - [DI20130913000003-Japan] Customer has a question about SNMP v1 trap about Enterprice OID and agent-Address. - [DI20131017000001-Japan] SNMP trap overlapping. - Fix power limit issue that when configure to class 4 then unplug class 4 PD will be return to auto-mode. |
| V3.11.B004 | - [HQ20130823000016- D-Link track] IMPB DHCP snooping to Black List. Modified the behavior of IMPB. The priority of IMPB DHCP snooping higher than user configuration. - Fix D-track [DRU20130812000001-Russia]: Set more than one vlan. Static router port cannot update into multicast forwarding entry via static router port unplug and plug in again when one of the vlan status is disabled cause it cannot continue update static router port in next vlan’s multicast forwarding entry. |
| V3.11.B003 | 1. [DEUR20130711000006-Eastern Europe] power saving status, ACL a. Power saving is disabled(01.png), but status show something elese (02.png) b. If set rules according to 03.png and 04.png on hw A1 it works (00-02-9B-xx-xx-xx are allowed only). But the same dosn′t work on hw B (00-02-9B-xx-xx-xx is dropped) c. If disable igmp snooping for one VLAN then reboot device. After reboot snooping is enabled again d. The DUT does not Pop-up error message, when DUT setting a non-exist VID. e. The DUT Pop-up message is wrong, When DUT setting a blank Value. 2. [DRU20130812000001-Russia] cannot update to fwd table issue via router port linkdown and linkup. |
| V3.11.B002 | For DJP Request to cut in beta F/W as official shipping version for DGS-1210 series. |
| V3.11.001 | N/A |
| V3.10.B028 | 1. Fix the fiber port counter issue that "ifHCOutOctets" cannot support 64-Bit counters. |
| V3.10.B025 | 1. Fix The DHCP relay packet format of Remote ID type is incorrect issue. |
| V3.10.B022 | N/A |
| V3.10.B018 | 1. [DEUR20130305000005-Eastern Europe] SFP port counter issue. SFP port counter behavior changes to same as the copper port as below: While port counter out of range( 0~ 4294967295),it will be reset to zero. |
| V3.10.B016 | 1. [DRU20121226000002-Russia] Fiber port counter issue a. Fix OutUcastPkts field bug that is always the same as InUcastPkts. b. Fix InOctets field bug that is always the same as InUcastPkts. c. Fix FrameTooLongs field bug that is always "0". |
| V3.10.B015 | 1. [DRU20121214000002-Russia] IGMP Snooping Issue. |
| V3.10.013 | 1. Fix SFP interface compatibility issue. 2. Change EEE default config to disabled. 3. Fix the bug of Jmicron NIC EEE Compatibility test with Marvell Solution. |
| V3.10.011 | First Release |

  - D-Link tracking number is enclosed in ()

# Known Issues:

| Firmware Version | Issues | Workaround |
| --- | --- | --- |
| V3.12.B039 | N/A | None |
| V3.12.B038 | N/A | None |
| V3.12.B037 | N/A | None |
| V3.12.B035 | N/A | None |
| V3.12.B032 | N/A | None |
| V3.12.B025 | N/A | None |
| V3.12.B024 | N/A | None |
| V3.12.B023 | N/A | None |
| V3.12.B020 | N/A | None |
| V3.12.B019 | N/A | None |
| V3.12.B018 | N/A | None |
| V3.12.B017 | N/A | None |
| V3.12.B016 | N/A | None |
| V3.12.B014 | N/A | None |
| V3.12.B013 | N/A | None |
| V3.12.B011 | N/A | None |
| V3.12.B010 | N/A | None |
| V3.12.B009 | N/A | None |
| V3.12.B007 | N/A | None |
| V3.12.B005 | N/A | None |
| V3.12.B004 | N/A | None |
| V3.12.003 | N/A | None |
| V3.11.B006 | N/A | None |
| V3.11.B005 | N/A | None |
| V3.11.B004 | N/A | None |
| V3.11.B003 | N/A | None |
| V3.11.B002 | N/A | None |
| V3.11.001 | N/A | None |
| V3.10.B028 | N/A | None |
| V3.10.B025 | N/A | None |
| V3.10.B022 | N/A | None |
| V3.10.B018 | N/A | None |
| V3.10.B016 | N/A | None |
| V3.10.B015 | N/A | None |
| V3.10.013 | N/A | None |
| V3.10.011 | - We uplink two DUT and set one fiber port to force mode, we find still link successful. Limitation - We find uplink two DUT the port link status will link up->link down-> link up when power saving is enable. Limitation - We find Fiber port LED glitter frequency not stable. Limitation - We set group interval to 1225 and capture the packets from DUT, we find it does not follow 1225 seconds. Limitation - During DUT save configuration, we can not ping DUT. After finish save config, ping will recover. - The DUT NS List can learning other devices link local IPv6 address even DUT Link local state disable. After confirm with Nelon, please help us to modify this behavior to follow DUT Link local state to learning link local NS entries. - The DUT Voice VLAN cannot follow LLDP-MED packets to update dynamic member type when created dynamic member. Please follow below step to confirm this issue: 1. Create VID 4000 VLAN. 2. Set VID 4000 to be Voice VLAN and enable SMB uplink interface auto detection enable. 3. Use SMB forward LLDP-MED (VID 4000 Untag type priority 0) to the DUT. 4. DUT Voice VLAN created Untag dynamic member in VID 4000. 5. Use SMB forward LLDP-MED (VID 4000 Tag type priority 0) to the DUT. 6. DUT Voice VLAN dynamic member still keep in VID 4000 Untag member. - Enable Port16 LBD port base and connect with a loop Switch in the endurance topology. We find port16 still show normal. Limitation - MAC addresses of DUT SFP ports in FDB will be cleared when new link is established in DUT SFP ports. This issue also exists in copper ports randomly. Limitation - The DUT disable auto learning does not work when two ports forward unicast packets to each other state to disable one port auto learning. When enable auto learning function again, we find the FDB cannot display learning MAC address. - When the DUT auto learning disable port receives learning packets then enable auto learning again, we find FDB forward rule error. We find FDB forward rule error. When we disable port5 auto learning state then transmit packets to port5 with SA MAC 00-00-00-00-04-e7, then we enable port5 auto learning, the DUT FDB does display learned port5 MAC 00-00-00-00-04-E7, then we use port19 forward Unicast packets (DA MAC: 00-00-00-00-04-E7) to port5, we find these packets will flooding to other ports of the DUT. - Enable IGMP, Transmit 508 multicast groups to DUT by MGEN, sometimes we find DUT just learn 250 multicast groups Limitation. - We enable VID 1-2 802.1 Extension LLDP Port Settings Set but just only VID1 success. We suggest support multiple VID setting. - When create 256 VLAN groups, we find LLDP packets do not include IEEE 802.1 info. (Maximum support 156 VLAN Groups) - We suggest add data index Link in LLDP Local Port Normal Table. - When two ports transmit packets to the same destination port with the same queue, the received rate is not stable. Limitation - The DUT 802.1p QoS will affect DSCP and ToS WRR mode priority rate.(Follow D-Link functional Spec, DSCP and ToS priority should be higher than 802.1p QoS) Test Result: Normal rate should be as below result: - 1.Set port security MAX learning address to 1. 2.Use Smartbit transmit Unicast packets from port2 to port3 via 64 bytes.(Port2 SA MAC: 00-00-00-00-00-02/ Port3 SA MAC:00-00-00-00-00-03) (Please learning MAC address first) 3. Use Smartbit transmit 1000 broadcast packets to port3 of the DUT via 64 bytes. (Port3 SA MAC: 00-00-00-00-00-02) We find Port2 of the DUT in forwarding table doesn't exist, port3 keep 00-00-00-00-00-03.Limitation - 1. Control PC connect to port1 in VLAN1 2. Set port 4 max learning address to 1 in VLAN2, through SmartBit transmit 1000 MAC to port 4 (Continuous), we find ping DUT will disconnect. Limitation - The DUT Safeguard function enable/disable, the DUT reply ICMPv6 request rate is the same. - The DUT TFTP Firmware upgrade via IPv6 performance too slowly. Forwarding rate is only about 35k Byte when Safeguard Enable. - We find storm control rate is not accurate, tolerance about 24%. Limitation. - Some Web Browser cannot open DUT IPv6 SSL web page. Google Chrome 20.0.1132.47 m cannot login DUT SSL Web UI via linklocal IPv6 address. Safari 5.1.7 and Firefox 13 cannot DUT SSL Web UI via link local or Global Unicast IPv6 address.(Known issue_ Browser limitation) - The DUT Smart Binding Function IP+ARP Inspection mode white list binding entries packets will be denied after error IP packets learned. When we set SmartBinding white list IP 192.168.0.99 MAC: 00-00-00-00-00-02 Port1, then transmit error IP ARP packets 192.168.0.98 MAC:00-00-00-00-00-02 to port1, we find black list add this entries and error IP packets be denied, then we change ARP packets to fit white list forward to port1, we find white list entries also be denied. - The DUT 802.1x radius server IPv4 address display error after created IPv6 radius server then change to error IPv4 radius server. - When we create access id 1 IPv6 permit rule and id 2 MAC deny all, then transmit fit IPv6 permit rule packets to the DUT, we find the packets drop by the DUT. MAC/IPv4 do not exist this issue. IPv6 Access ID priority rule just support IPv6 packets in IPv6 ACL rule. MAC and IPv4 ACL deny rule always higher than IPv6 ACL rule. Limitation - When we test cable diagnostics, the Web UI pop up error when test control PC uplink interface via IE8, Firefox… Just only support IE9 Web Brower. Limitation - The deviation of \"Cable Fault Distance\" is +/-10 meters, therefore No cable may be displayed under Test Result, when the cable used is less than 10 m in length. Limitation - If user want to know fault cable status via run cable Diagnostics Function, the DUT need real break link (the link partner is not connected). Limitation - The DUT TFTP firmware update does not check error file, but configuration restore does support it. - We can't use Link-Local address (IPV6) to login DUT Web via Safari 5.1.7, Firefox10.0.2 and Chrome 17.0.963.56 Web Browser. Software note is Web Browser issue not firmware. Limitation - Below function’s warning message keep English on others language: 802.1x, LBD, STP, LLDP, RMON, Time Profile. - The DUT Multi-language GUI display range over GUI via German & Russia language in Basic LLDP Port Settings. |  |

# Related Documentation:

- DGS-1210-28P Series User Manual

- DGS-1210-28P Series Getting Started Guide