| Firmware Version: | 3.12.B039 |
| --- | --- |
| Boot Code Version: | 1.00.013 |
| MIB Version: | 3.12.B003 |
| D-View Module Version: | 3.0.0.6 |
| Published: | Sep. 18, 2015 |

These release notes include important information about D-Link switch firmware revisions. Please verify that these release notes are correct for your switch:

- If you are installing a new switch, please check the hardware version on the device label; make sure that your switch meets the system requirement of this firmware version. Please refer to Revision History and System Requirement for detailed firmware and hardware matrix.

- If the switch is powered on, you can check the hardware version by typing “show switch” command via Telnet or by checking the device information page on the web graphic user interface.

- If you plan to upgrade to the new firmware release, please refer to the Upgrade Instructions for the correct firmware upgrade procedure.

For more detailed information regarding our switch products, please refer to Related Documentation.

You can also download the switch firmware, D-View modules and technical documentation from http://tsd.dlink.com.tw.

Content:

Revision History and System Requirement:	2

Upgrade Instructions:	2

Upgrade using CLI (via Telnet)	2

Upgrade using Web-UI	3

New Features:	3

Changes in MIB & D-View Modules:	7

Changes in Command Line Interface:	10

Problem Fixed:	11

Known Issues:	12

Related Documentation:	13

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| Runtime: v3.12.B039 Boot: 1.00.013 | 18-Sep.-15 | DGS-1210-28 | B1 |
| Runtime: v3.12.B037 Boot: 1.00.013 | 14-July.-15 | DGS-1210-28 | B1 |
| Runtime: v3.12.B035 Boot: 1.00.013 | 14-July.-15 | DGS-1210-28 | B1 |
| Runtime: v3.12.B032 Boot: 1.00.013 | 3- Dec.-14 | DGS-1210-28 | B1 |
| Runtime: v3.12.B025 Boot: 1.00.013 | 3- Dec.-14 | DGS-1210-28 | B1 |
| Runtime: v3.12.B024 Boot: 1.00.013 | 6- Nov.-14 | DGS-1210-28 | B1 |
| Runtime: v3.12.B023 Boot: 1.00.013 | 12- Sep.-14 | DGS-1210-28 | B1 |
| Runtime: v3.12.B020 Boot: 1.00.013 | 2- Sep.-14 | DGS-1210-28 | B1 |
| Runtime: v3.12.B019 Boot: 1.00.013 | 22- Aug.-14 | DGS-1210-28 | B1 |
| Runtime: v3.12.B018 Boot: 1.00.013 | 1- Jul.-14 | DGS-1210-28 | B1 |
| Runtime: v3.12.B017 Boot: 1.00.013 | 10- Jun.-14 | DGS-1210-28 | B1 |
| Runtime: v3.12.B016 Boot: 1.00.013 | 16- May.-14 | DGS-1210-28 | B1 |
| Runtime: v3.12.B014 Boot: 1.00.013 | 14- Mar.-14 | DGS-1210-28 | B1 |
| Runtime: v3.12.B013 Boot: 1.00.013 | 7- Mar.-14 | DGS-1210-28 | B1 |
| Runtime: v3.12.B011 Boot: 1.00.012 | 17- Feb.-14 | DGS-1210-28 | B1 |
| Runtime: v3.12.B010 Boot: 1.00.012 | 12- Feb.-14 | DGS-1210-28 | B1 |
| Runtime: v3.12.B009 Boot: 1.00.012 | 6- Feb.-14 | DGS-1210-28 | B1 |
| Runtime: v3.12.B007 Boot: 1.00.012 | 18- Dec.-13 | DGS-1210-28 | B1 |
| Runtime: v3.12.B005 Boot: 1.00.009 | 13- Dec.-13 | DGS-1210-28 | B1 |
| Runtime: v3.12.B004 Boot: 1.00.009 | 9- Dec.-13 | DGS-1210-28 | B1 |
| Runtime: v3.12.003 Boot: 1.00.009 | 28- Nov.-13 | DGS-1210-28 | B1 |
| Runtime: v3.11.B006 Boot: 1.00.009 | 1- Nov.-13 | DGS-1210-28 | B1 |
| Runtime: v3.11.B005 Boot: 1.00.009 | 24- Oct.-13 | DGS-1210-28 | B1 |
| Runtime: v3.11.B004 Boot: 1.00.009 | 30- Aug.-13 | DGS-1210-28 | B1 |
| Runtime: v3.11.B003 Boot: 1.00.009 | 20- Aug.-13 | DGS-1210-28 | B1 |
| Runtime: v3.11.B002 Boot: 1.00.009 | 07- Aug.-12 | DGS-1210-28 | B1 |
| Runtime: v3.11.001 Boot: 1.00.009 | 26- Jan.-13 | DGS-1210-28 | B1 |

# Upgrade Instructions:

D-Link Smart Switches support firmware upgrade via TFTP server. You can download the firmware from D-Link web site http://tsd.dlink.com.tw, and copy the downloaded firmware to the TFTP server folder. Please make sure that the TFTP server is accessible from the switch via networks.

## Upgrade using CLI (via Telnet)

1. Make sure the network connection between the switch and PC is active.

2. Use software that supports telnet, for example, HyperTerminal or Telnet command in Microsoft Windows, to connect to the switch. If you are using Telnet command, type the command followed by the switch IP address, eg. telnet 10.90.90.90.

3. The logon prompt will appear.

The switch will prompt the user to enter his/her username and password. It should be noted that upon the initial connection, both the default user name and password are admin.

To upgrade the switch firmware, execute the following command:

| Command | Function |
| --- | --- |
| download{firmware_fromTFTP tftp://ip-address/filename ¦ cfg_fromTFTP tftp://ip-address/filename} | Download firmware file from the TFTP server to the switch. |

When completing firmware upgrade, the following messages will pop up.

Device will reboot after firmware upgraded successfully

Image upgraded successfully

4. Execute the following command to check the firmware version and switch’s information.

| Command | Function |
| --- | --- |
| show switch | Display the information of current firmware and boot version. |

Example:

- DGS-1210-28:

Command: download firmware_fromTFTP tftp://10.90.90.91 DGS-1210-28 B1 3_12_B025.hex

Device will reboot after firmware upgraded successfully

Image upgraded successfully

- DGS-1210-28:

Command: show switch

DGS-1210-28> show switch

System name                       :

System Contact                    :

System Location                   :

System up time                    : 0 days, 1 hrs, 8 min, 36 secs

System Time                       : 01/01/2014 01:58:42

System hardware version           : B1

System firmware version           : 3.12.B025

System boot version                : 1.00.013

System Protocol version           : 2.001.004

System serial number              : LABDGS1220008

MAC Address                         : 00-12-20-00-08-00

Upgrade using Web-UI

- Connect a workstation installed with java SE runtime environment to any switch port of the device.

- Open the web browser from the workstation and enter the IP address of the switch. The switch’s default IP address is 10.90.90.90.

- Enter administrator’s password when prompted. The password is admin by default.

- To update switch's firmware or configuration file, select Tools > Firmware Backup & Upgrade from the banner.

- Two methods can be selected : HTTP or TFTP

- Select HTTP to upgrade the firmware to or from your local drive of PC.

- Click Browse to browse your inventories for a saved firmware file

- Click Upgrade after selecting the firmware file you want to restore

- Click OK to continue with firmware upgrade

- Click OK to continue with firmware upgrade

- Wait until the “Firmware Upgrade Successful” message pops up and login again after device boots up.

- Select TFTP to upgrade the firmware to or from a remote TFTP server.

- Enter the name of the firmware file located on the TFTP server

- Click Upgrade after selecting the firmware file you want to restore

- Click OK to continue with firmware upgrade

- Wait until the firmware upgrade ends and login again after device boots up.

Upgrade using SmartConsole Utility

1. Connect a workstation installed with java SE runtime environment to any switch port of the device

## 2. Execute SmartConsole Utility

3. Click Discovery button to search target switch

4. Single click the icon of Monitor column to choose the target switch

5. Click Firmware Upgrade button

6. Click Browse button and select the firmware file (Model name_HW ver._FW ver. .hex) on your local hard drive.

7. Enter password and click “Upgrade” button to start firmware upgrade. The default password is “admin”.

8. Once the message popped up, click “OK” button to complete the firmware upgrade

# New Features:

| Firmware Version | New Features |
| --- | --- |
| V3.12.B039 | N/A |
| V3.12.B037 | N/A |
| V3.12.B035 | N/A |
| V3.12.B032 | N/A |
| V3.12.B025 | N/A |
| V3.12.B024 | N/A |
| V3.12.B023 | N/A |
| V3.12.B020 | N/A |
| V3.12.B019 | N/A |
| V3.12.B018 | N/A |
| V3.12.B017 | N/A |
| V3.12.B016 | N/A |
| V3.12.B014 | N/A |
| V3.12.B013 | N/A |
| V3.12.B011 | N/A |
| V3.12.B010 | N/A |
| V3.12.B009 | 1. Add New default MAC (B0C5540 ~ B0C5547) for ASV |
| V3.12.B007 | N/A |
| V3.12.B005 | N/A |
| V3.12.B004 | N/A |
| V3.12.003 | N/A |
| V3.11.B006 | N/A |
| V3.11.B005 | N/A |
| V3.11.B004 | N/A |
| V3.11.B003 | N/A |
| V3.11.B002 | N/A |
| V3.11.001 | 1. Add UI for D-Link discover protocol setting. 2. Add DHCP relay and DHCP local relay. |

# Changes of MIB & D-View Module:

The new features of MIB file are also included in the corresponding D-View module. Please download the D-View module from http://tsd.dlink.com.tw. For detailed changes of MIB content, please refer to the modification history in each MIB file.

| Firmware Version | MIB File | New Features |
| --- | --- | --- |
| V3.12.B039 | DGS-1210-28-BX-3_12_B003.mib | N/A |
| V3.12.B037 | DGS-1210-28-BX-3_12_B003.mib | N/A |
| V3.12.B035 | DGS-1210-28-BX-3_12_B003.mib | N/A |
| V3.12.B032 | DGS-1210-28-BX-3_12_B003.mib | N/A |
| V3.12.B025 | DGS-1210-28-BX-3_12_B003.mib | N/A |
| V3.12.B024 | DGS-1210-28-BX-3_12_B003.mib | N/A |
| V3.12.B023 | DGS-1210-28-BX-3_12_B003.mib | N/A |
| V3.12.B020 | DGS-1210-28-BX-3_12_B003.mib | N/A |
| V3.12.B019 | DGS-1210-28-BX-3_12_B003.mib | N/A |
| V3.12.B018 | DGS-1210-28-BX-3_12_B003.mib | N/A |
| V3.12.B017 | DGS-1210-28-BX-3_12_B003.mib | N/A |
| V3.12.B016 | DGS-1210-28-BX-3_12_B003.mib | N/A |
| V3.12.B014 | DGS-1210-28-BX-3_12_B003.mib | N/A |
| V3.12.B013 | DGS-1210-28-BX-3_12_B003.mib | N/A |
| V3.12.B011 | DGS-1210-28-BX-3_12_002.mib | N/A |
| V3.12.B010 | DGS-1210-28-BX-3_12_002.mib | N/A |
| V3.12.B009 | DGS-1210-28-BX-3_12_002.mib | N/A |
| V3.12.B007 | DGS-1210-28-BX-3_12_002.mib | N/A |
| V3.12.B005 | DGS-1210-28-BX-3_12_002.mib | N/A |
| V3.12.B004 | DGS-1210-28-BX-3_12_002.mib | N/A |
| V3.12.003 | DGS-1210-28-BX-3_12_002.mib | 1.Add object "aclL2RulePriority", "aclL2RuleReplacePriority", "aclL3RulePriority", "aclL3RuleReplacePriority", "aclv6L3RulePriority" and "aclv6L3RuleReplacePriority" for ACL. |
| V3.11.B006 | DGS-1210-28-BX-3_11_001.mib | N/A |
| V3.11.B005 | DGS-1210-28-BX-3_11_001.mib | N/A |
| V3.11.B004 | DGS-1210-28-BX-3_11_001.mib | N/A |
| V3.11.B003 | DGS-1210-28-BX-3_11_001.mib | N/A |
| V3.11.B002 | DGS-1210-28-BX-3_11_001.mib | N/A |
| V3.11.001 | DGS-1210-28-BX-3_11_001.mib | 1. Add DDP settings. 2. Add DHCP relay and DHCP local relay |

| Firmware Version | D-View Module File | New Features |
| --- | --- | --- |
| V3.12.B039 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B037 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B035 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B032 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B025 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B024 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B023 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B020 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B019 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B018 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B017 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B016 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B014 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B013 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B011 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B010 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B009 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B007 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B005 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.B004 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.12.003 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.11.B006 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.11.B005 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.11.B004 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.11.B003 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.11.B002 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |
| V3.11.001 | DV_DGS-1210-28_V3.0.0.6_FW_v3.10.011.exe | N/A |

# Changes of Command Line Interface:

The section below only shows command line changes that may bring backward compatibility issues with configuration settings for previous version of firmware.

Any new feature commands that do not have backward compatibility issues are not included in the below section.

| Firmware Version | Changes |
| --- | --- |
| V3.12.B039 | N/A |
| V3.12.B037 | N/A |
| V3.12.B035 | N/A |
| V3.12.B032 | N/A |
| V3.12.B025 | N/A |
| V3.12.B024 | N/A |
| V3.12.B023 | N/A |
| V3.12.B020 | N/A |
| V3.12.B019 | N/A |
| V3.12.B018 | N/A |
| V3.12.B017 | N/A |
| V3.12.B016 | N/A |
| V3.12.B014 | N/A |
| V3.12.B013 | N/A |
| V3.12.B011 | N/A |
| V3.12.B010 | N/A |
| V3.12.B009 | N/A |
| V3.12.B007 | N/A |
| V3.12.B005 | N/A |
| V3.12.B004 | N/A |
| V3.12.003 | N/A |
| V3.11.B006 | N/A |
| V3.11.B005 | N/A |
| V3.11.B004 | N/A |
| V3.11.B003 | N/A |
| V3.11.B002 | N/A |
| V3.11.001 | N/A |

# Problem Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V3.12.B039 | - [DRU20150907000005-Russia]DGS-1210 rev.B1 reboots by some special URL. - [DRU20150903000003-Russia]DES-1210-28P issue with adding port to vlan. |
| V3.12.B037 | - Fix [HQ20150814000010]DGS-1210-10P the Admin password can be changed by everybody without knowing the Admin password.[DEUR20150813000005-Central Europe]. |
| V3.12.B035 | - Fix [HQ20150522000015] DHCP Local Relay and unicast packets [DRU20150521000002-Russia]. |
| V3.12.B032 | - Fix [HQ20150331000019] DES series & DGS series issue with unicast DHCP packets [DRU20150325000001-Russia]. |
| V3.12.B025 | - [DGC20141124000002-Taiwan] DHCP Sever Screening problem. |
| V3.12.B024 | - [DRU20141016000003-Russia] DGS-1210-10P issue with DHCP Local Relay. |
| V3.12.B023 | - [DGC20140801000004-Taiwan] DGS-1210-28 can't show the specific igmp group. - DI20130913000003[DI20140825000007-Japan]About Link change trap which is related to case. |
| V3.12.B020 | 1. Fix incorrect model on SSL certificate. |
| V3.12.B019 | 1. OpenSSL before 0.9.8za, which allows man-in-the-middle attack, and consequently hijack sessions or obtain sensitive information. 2. [DGC20140801000004-Taiwan] DGS-1210-28 can't show the specific igmp group. |
| V3.12.B018 | 1. [DRU20140618000002-Russia] DGS-1210 WebSmart DHCP relay issue. |
| V3.12.B017 | 1. [DI20140530000002-Japan] Wrong Sys up time / same as DI20140205000006. 2. [DRU20140523000001-Russia] DGS-1210 snmp port speed issue. |
| V3.12.B016 | 1. [DEUR20140429000003-Eastern Europe] RFC3580 radius attribute mismatch. 2. [DRU20140512000003-Russia] DGS-1500-20 issue with device reboot. |
| V3.12.B014 | Pre D-Link request version update. |
| V3.12.B013 | 1. Fixed DES-1210 smart binding issue[DRU20140218000001-Russia] |
| V3.12.B011 | 1. After inject the traffic,VLAN Summary entry still none. |
| V3.12.B010 | 1. Fixed [DRU20140120000004]DES-1210-28 issue with ACL. |
| V3.12.B009 | 1. Fixed system up time issue. [DI20140205000006-Japan]. |
| V3.12.B007 | 1. [DI20131211000003-Japan] DGS-1210 reboot with loop. 2. Fixed bug for DUT still enters power saving status when system time meets the time period of assigned profile but not selected weekdays. |
| V3.12.B005 | 1. HQ20131128000015 [DEUR20131118000008-North Europe] Fixed Time profile select week day could be over night. |
| V3.12.B004 | 1. HQ20131128000015 [DEUR20131118000008-North Europe] Fixed Time profile could be over night. |
| V3.12.003 | 1. Enhance DDP to 0.24 2. ACL and QOS page modify. 3. Modify D-Track support case [DRU20131101000002-Russia]Fix telnet issue that DUT with reboot during telnet session. 4. Modify D-Track support case [DI20131031000001-Japan]:DGS-1210 does send frames of loopdetect. 5. Modify D-Track support case [DGC20131016000004-Taiwan] DGS-1210-52 with recovery system issue. 6. Fix Cameo Bug#:0027770: "Unable to set the replace priority" will pop up when editing ACL IPv6 Class rule.. 7. Add command "debug information" for Telnet / http://XX.XX.XX.XX/kernel_crash.txt for GUI. |
| V3.11.B006 | 1. [DI20131023000005-Japan] RSTP switch back will cause network disconnection. |
| V3.11.B005 | 1. [ Support Request ] Fix Ping / Login Failure issue in SmartSW series. Note: After confirm with D-LINK Shan, this issue can not find out the correct D-track no. and will keep asking the result for the latest firmware. 2. [DEUR20131008000003-Central Europe] Fixed not able to set STP external cost on Port 9&10 in LAG. - 3. [DI20130913000003-Japan] Customer has a question about SNMP v1 trap about Enterprice OID and agent-Address. - 4. [DI20131017000001-Japan] SNMP trap overlapping. |
| V3.11.B004 | - [HQ20130823000016- D-Link track] IMPB DHCP snooping to Black List. Modified the behavior of IMPB. The priority of IMPB DHCP snooping higher than user configuration.. 2. Fix D-track [DRU20130812000001-Russia]: Set more than one vlan. Static router port cannot update into multicast forwarding entry via static router port unplug and plug in again when one of the vlan status is disabled cause it cannot continue update static router port in next vlan’s multicast forwarding entry. |
| V3.11.B003 | 1. [DEUR20130711000006-Eastern Europe] power saving status, ACL a. Power saving is disabled(01.png), but status show something elese (02.png) b. If set rules according to 03.png and 04.png on hw A1 it works (00-02-9B-xx-xx-xx are allowed only). But the same dosn′t work on hw B (00-02-9B-xx-xx-xx is dropped) c. If disable igmp snooping for one VLAN then reboot device. After reboot snooping is enabled again d. The DUT does not Pop-up error message, when DUT setting a non-exist VID. e. The DUT Pop-up message is wrong, When DUT setting a blank Value. 2. [DRU20130812000001-Russia] cannot update to fwd table issue via router port linkdown and linkup. |
| V3.11.B002 | For DJP Request to cut in beta F/W as official shipping version for DGS-1210 series. |
| V3.11.001 | N/A |

* D-Link tracking number is enclosed in ()

# Known Issues:

| Firmware Version | Issues | Workaround |
| --- | --- | --- |
| V3.12.B039 | N/A | None |
| V3.12.B037 | N/A | None |
| V3.12.B035 | N/A | None |
| V3.12.B032 | N/A | None |
| V3.12.B025 | N/A | None |
| V3.12.B024 | N/A | None |
| V3.12.B023 | N/A | None |
| V3.12.B020 | N/A | None |
| V3.12.B019 | N/A | None |
| V3.12.B018 | N/A | None |
| V3.12.B017 | N/A | None |
| V3.12.B016 | N/A | None |
| V3.12.B014 | N/A | None |
| V3.12.B013 | N/A | None |
| V3.12.B011 | N/A | None |
| V3.12.B010 | N/A | None |
| V3.12.B009 | N/A | None |
| V3.12.B007 | N/A | None |
| V3.12.B005 | N/A | None |
| V3.12.B004 | N/A | None |
| V3.12.003 | N/A | None |
| V3.11.B006 | N/A | None |
| V3.11.B005 | N/A | None |
| V3.11.B004 | N/A | None |
| V3.11.B003 | N/A | None |
| V3.11.B002 | N/A | None |
| V3.11.001 | N/A | None |

# Related Documentation:

- DGS-1210-28 Series User Manual

- DGS-1210-28 Series Getting Started Guide