DGS-3100 series firmware and boot code update SOP.

Warning: DGS-3100 series switch doesn’t support update boot-code from Web GUI.

Topology:

RJ-45

RS-232

(COM 1)

DGS-3100 Series

TFTP Server

Software Application: 
1.TFTP server: 10.90.90.100. 
TFTP server must be stay with firmware and boot-code at the same folder.

2. DGS-3100 series: 
Default management IP address: 10.90.90.90
Common: 
DGS3100# show switch

3. Update boot code and firmware from CLI.

- Download boot-code: 
Common: download boot IPaddress (=TFTP server IP address) filename 
DGS3100# download boot 10.90.90.100 dgs_31xx_boot-10004.rfb

- Download firmware: 
Common: download firmware IPaddress (=TFTP server IP address) filename 
DGS3100# download firmware 10.90.90.100 dgs_31xx-20029.ros

(P.S. In stacking mode, switch support update firmware for all units at the same time but it doesn’t support update boot code for all units at the same time)