# DGS-3224TGR Firmware Release Note

Firmware: 3.01.030

Hardware: A1/A2

Date: Jan. 10, 2006

Problems Resolved:

Fix the security issues recently reported for unauthorized system access.

Enhancements:

Limit the access by passwords generated by "D-Link PWD calculator".

Before, if customers forgot their configured passwords, D-Link could generate passwords based on the MAC addresses provided by customers via D-Link HQ maintained PWD calculator. Thus customers can use those "backdoor" passwords via telnet/ web/ console to logon to their switch again.

Now the access of those passwords generated by "D-Link PWD calculator" will be limited to console access only.

This is to minimize the security concern.

Note:

This is a recommended security patch for your customers.

===============================================================

Firmware: 3.01.027

Hardware: A1/A2

Date: Oct. 28, 2005

Problems Resolved:

- STP configuration can’t be enabled by web interface.

2. After using tftp upload configuration to 3Com tftp server, DGS-3324SR might have session lock problem from Telnet/CLI

Enhancements:

New PROM code (version 0.01.007) can support both new (Atmel/ AT49BV163A-70TI) and existing Flash Rom.

=================================================================

Firmware: 3.01.019

Hardware: A1

Date: Dec. 31, 2004

Enhancements:

- IGMP Snooping support fast leave

- Single IP Management v1.0

- Port Description

- CPU Utilization Monitoring

- "Show Config" command

- Support 802.1X MAC-based Access Control

- TACACS+

- SSL

- ACL based on Port,

- ACL based on TCP/UDP payload

- ACL Destination to CPU packet has to be filtered.

Problems Resolved:

1. DGS-3224TGR will not respond to a ping or SNMP after about 2 to 12 hours of polling. The unit however will keep switching but the web management and SNMP are no longer accessible.

Note: ACL and SSH module has been upgraded. If there is related configuration in previous firmware, this part must be re-configured after upgrading to this version.

=================================================================

DGS-3224TGR runtime code release history  (updated: Apr.22, 2004)

Firmware 2.03.009

Bug fixed.

N/A

New features:

1.Copper ports support 1000 full force mode

2.Support tagged ARP packet of 64 bytes.

3.Upgrade PROM (version 0.01.005) and runtime code to support new flash and previous flash.

4.release D-view module v2.00R18 to match it.

Notes: If you use previous flash, you don’t need to update it.

DGS-3224TGR runtime code release history  (updated: Dec.26, 2003)

Firmware 2.03.005

Bug fixed.

N/A

New features:

1. 802.1w Rapid Reconfiguration of Spanning Tree.

2. Asymmetric VLAN

3. Traffic Segmentation

4. 802.3ad Link Aggregation (LACP)

5.TACACS

6. Cisco like port security

Notes: The 2.03.005 is exactly the same as the previous 1.03.005 firmware, except the version number.

The change is for the consistency with other D-Link's switches on firmware version naming.

First Release

Firmware 1.02.021 (Oct.8, 2003)

New features:

First release version. For the features, please refer to the datasheet.