| Firmware Version: | V2.70.B56 |
| --- | --- |
| Prom Code Version: | V1.00.B13 |
| Published: | Jun 30, 2010 |

These release notes include important information about D-Link switch firmware revisions. Please verify that these release notes are correct for your switch:

- If you are installing a new switch, please check the hardware version on the device label; make sure that your switch meets the system requirement of this firmware version. Please refer to Revision History and System Requirement for detailed firmware and hardware matrix

- If the switch is powered on, you can check the hardware version by typing the “show switch” command or by checking the device information page on the web graphic user interface.

- If you plan to upgrade to the new firmware release, please refer to the Upgrade Instructions for the correct firmware upgrade procedure.

For more detailed information regarding our switch products, please refer to Related Documentation.

You can also download the switch firmware, D-View modules and technical documentation from http://tsd.dlink.com.tw.

Content:

Revision History and System Requirement:	2

Upgrade Instructions:	2

Upgrade by using CLI (serial port)	2

Upgrade using Web-UI	4

New Features:	6

Changes in MIB & D-View Module:	14

Changes in Command Line Interface:	20

Problems Fixed:	21

Known Issues:	23

Related Documentation:	23

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| Runtime: V2.70.B56 Prom: v1.00.B13 | 30-Jun-10 | DGS-3426 | A1, A2, A3 |
|  |  | DGS-3426P | A1, A2 |
|  |  | DGS-3427 | A1, A2, A3 |
|  |  | DGS-3450 | A1, A2, A3 |
| Runtime: V2.60.B26 Prom: v1.00.B13 | 27-July-09 | DGS-3426 | A1, A2 |
|  |  | DGS-3426P | A1, A2 |
|  |  | DGS-3427 | A1, A2 |
|  |  | DGS-3450 | A1, A2 |
| Runtime: V2.35.B09 | 24-Oct-08 | DGS-3426 | A1, A2 |
|  |  | DGS-3426P | A1, A2 |
|  |  | DGS-3427 | A1, A2 |
|  |  | DGS-3450 | A1, A2 |
| Runtime: V2.30.B10 | 15-Oct-07 | DGS-3426 | A1, A2 |
|  |  | DGS-3426P | A1, A2 |
|  |  | DGS-3427 | A1, A2 |
|  |  | DGS-3450 | A1, A2 |
| Runtime: V2.00.B52 | 05-May-07 | DGS-3426 | A1, A2 |
|  |  | DGS-3426P | A1, A2 |
|  |  | DGS-3427 | A1, A2 |
|  |  | DGS-3450 | A1, A2 |
| Runtime: V1.20.B23 | 05-June-06 | DGS-3426 | A1, A2 |
|  |  | DGS-3426P | A1, A2 |
|  |  | DGS-3427 | A1, A2 |
|  |  | DGS-3450 | A1, A2 |
| Runtime: V1.00.B35 | 27-Jan-06 | DGS-3426 | A1 |
|  |  | DGS-3427 | A1 |
|  |  | DGS-3450 | A1 |

# Upgrade Instructions:

Caution: This version only supports direct firmware upgrade from v2.60. Direct upgrade from any version prior to v2.60 is not suggested and may result in unknown issues.  Downgrade to any version prior to v.2.60 is not supported.

If one of the switch images has the firmware version prior to v.2.60, then this image can not be selected as the boot up image.

D-Link switches support firmware upgrade via TFTP server. You can download the firmware from D-Link web site http://tsd.dlink.com.tw, and copy the downloaded firmware to the TFTP server folder. Please make sure that the TFTP server is accessible from the switch via networks.

## Upgrade by using CLI (serial port)

Connect a workstation to the switch console port and run any terminal program that can emulate a VT-100 terminal. The switch serial port default settings are as follows:

- Baud rate: 115200

- Data bits: 8

- Parity: None

- Stop bits: 1

The switch will prompt the user to enter his/her username and password. It should be noted that upon the initial connection, there is no username and password by default.

To upgrade the switch firmware, execute the following commands:

| Command | Function |
| --- | --- |
| download [ firmware_fromTFTP <ipaddr> <path_filename 64> {image_id <int 1-2>} ] | Download firmware file from the TFTP server to the switch. |
| config firmware image_id <1-2> [delete ¦ boot_up] | Change the boot up image file. |
| show firmware_information | Display the information of current boot image and configuration. |
| reboot | Reboot the switch. |

Example:

- DGS-3426P:5#download firmware from TFTP 10.90.90.91 R270B56.had image_id 2

Command: download firmware_fromTFTP 10.90.90.91 R270B56.had image_id 2

Connecting to server................... Done.

Download firmware...................... Done.  Do not power off!

Please wait, programming flash......... Done.

- DGS-3426P:5#config firmware image_id 2 boot_up

Command: config firmware image_id 2 boot_up

Success.

- DGS-3426P:5#show firmware information

Command: show firmware information

ID  Version   Size(B) Update Time          From               User

-- --------- ------- ------------------- ------------------ ---------------

1  2.60.B26  2450452 2010/02/04 17:00:26  10.90.90.91(R)     Anonymous

*2  2.70.B56  4029512 2010/03/05 02:25:85  10.90.90.91(R)     Anonymous

'*' means boot up firmware

(R) means firmware update through Serial Port(RS232)

(T) means firmware update through TELNET

(S) means firmware update through SNMP

(W) means firmware update through WEB

(SSH) means firmware update through SSH

(SIM) means firmware update through Single IP Management

- DGS-3426P:5#reboot

Command: reboot

Are you sure you want to proceed with the system reboot?(y/n) y

Please wait, the switch is rebooting...

Boot Procedure                                                    V1.00.B13

-------------------------------------------------------------------------------

Power On Self Test ........................................  100 %

MAC Address   : 00-1E-58-4F-F7-D0

H/W Version   : A1

Please wait, loading V2.70.B56 Runtime image .............  	 100 %

UART init .........................

Device Discovery ..........................................  100 %

Configuration init ........................................  100 %

Upgrade using Web-UI

- Connect a workstation installed with java SE runtime environment to any switch port of the device.

- Open the web browser from the workstation and enter the IP address of the switch. The switch’s default IP address is 10.90.90.90.

- Enter administrator’s username and password when prompted. It should be noted that the username and password are blank by default.

- To update the switch's firmware or configuration file, select Administration > TFTP Services in function tree.  Select Download Firmware in Operation.

- If the switch is under stacking mode, select the Unit ID of the switch that will be upgraded.

- Select the Image ID of the firmware. Choosing Active will download the firmware to the Boot Up Image ID.

- Select the type (IPv4 or IPv6) of IP address of the TFTP server and enter the IP address.

- Enter the firmware file name in File Name.

- Click “Start”

- Wait until the “File Transfer” status reaches 100% and the “Program Firmware” status shows “completed”.

- To select the boot up image used for next reboot, click Administration > Multiple Image Services > Config Firmware Image in the function tree.

- Enter the image ID and choose “Boot” then click “Apply”.

- To reboot the switch, click DGS-3400 Web Management Tool > Reboot System. Select “Yes” and click “Restart” button to reboot the switch.

# New Features:

| Firmware Version | New Features |
| --- | --- |
| V2.70.B56 | - Configuration enhancement: - Support the filtering keywords: include/exclude/begin when using “show config” and “upload_config” - Support “increment” option when downloading cfg_fromTFTP If “increment” is specified, then the existing configuration will not be cleared. The new configuration will cover the existing configuration. - Allow to specify “src_file”/“dst_file”/ “domain_name” in download/upload functions - Show memory/flash utilization. - Show technical_support This command is especially used by the technical support personnel to dump the device overall operation information. The information includes the following information. Basic System information system log Running configuration Layer 1 information Layer 2 information Layer 3 information Application OS status Controller’s status - Stacking enhancement: - "Change Stacking priority" can work without reboot - Stacking force master role feature This command ‘config stacking force_master_role state enable’ is used to ensure the master role is unchanged - Hot insert/Hot Remove trap/log messages include MAC information - Add new log/trap about topology change and role change - Show stack information and show log include information about stacking topology 5. Send a trap while firmware upgrade via SNMP is finished. 6. Display user-understandable account level in CLI prompt DES-XXXX:3# -> DES-xxxx:user# DES-XXXX:4# -> DES-xxxx:oper# DES-XXXX:5# -> DES-xxxx:admin# - CLI Command logging. This command can show CLI command setting history. - 8-level system log - Password recovery: allows to recover the password if the password is forgotten. - Enlarge the number of trusted host to 30. - SNMP-server & syslog source-interface appointment: allows to select an IP interface as the source interface to send syslog or trap message. - STP enhancement: - 802.1D 2004 RSTP - 802.1Q 2005 MSTP - STP Root Restriction - Source MAC of BPDUs uses port MAC instead of system MAC - Support edge port - Support BPDU address setting on NNI port when QinQ is enbled - Logging enhancement: The logs for stp topology changes include port and MAC-address - Log / show / debug Enhancement - D-LINK Unidirectional Link Detection (DULD) function. - Source MAC of L2 protocols (ERPS/LACP/STP/LBD) uses port MAC instead of system MAC - LACP support load-balancing with multicast traffic. - Storm control enhancement: - Change "countdown" to "3-30". - Change "time_interval" to "5 - 600". - Auto recovery for the shutdown port. - Add 4 counters to gather statistics of various frame sizes, such as 1519-1522, 1519-2047, 2048-4095, and 4096-9216. - Mirror enhancement: - Multiple sessions of mirroring - Link aggregation ports can be set as a target port - sFlow enhancement: - Allow to specify IPv6 server. - Support TX flow sampling. - Microsoft NLB (Network Load Balancing) support. - Add “data driven group” in IGMP/MLD snooping. - ISM-VALN enhancement: - Support Tag / Untagged member ports. - Support Tag / Untagged source ports. - Configurable Multicast VLAN priority. - Do not limit the number of total multicast addresses per ISM-VLAN entry when using “config igmp_snooping multicast_VLAN_group” - Forward protocol packets even the switch is under “filter_unregister_group mode” (Protocol packet: the packets with destination IP address in the range of reserved multicast addresses: 224.0.0.x, such as OSPF hello, PIM hello, and DVMRP probe etc.) - Support new OID to clear dynamic FDB by port/by VLAN. - VLAN Trunking - Subnet-based VLAN. - BPDU Attack Protection. - ITU-T G.8032 Ethernet Ring Protection Switching (ERPS) support. - ACL supports "IPv6 IP + UDP/TCP port" together. - Per queue egress bandwidth control. - WAC enhancement: - Identity driven policy assignment: Can assign ingress/egress bandwidth control, ACL and 802.1p default priority to the port according to the attributes dispatched from RADIUS server - Add log 1) To record system stop learning and recovery from stop learning status when reaching the maximum entries 2) To record authentication failure state for IPv4/IPv6 - Support host-based authentication mode : assign ingress/egress bandwidth control for all hosts to the port; assign VLAN or 802.1p default priority to the host after successful authentication in host-based mode(R2.50 only supports assign VLAN in port-based) - Support IPv6 - Support Per VLAN authentication - Support virtual IP: used to accept authentication requests from unauthenticated hosts. Only the requests sent to this IP will get response correctly. - Support time control for authenticated client (e.g. aging time/idle time/block time) - Support Authentication Database failover: Allows to configure the switch to check local database or bypass authentication when configured RADIUS server fails - Obsolete authentiation VLAN - Support compound authentication 32. Japanese Web-based Access Control (JWAC) enhancement (support JWAC v2.02) - Show IP address and user ID in UI and log - JWAC notification: Show notification to user automatically after passing the authentication. - Administrator can use FQDN URL to access JWAC login page instead of using IP 33. Compound authentication 34. ARP Spoofing Prevention. 35. Radius server setting supports IPv6. 36. Increase “max_learning_addr” number in port security from 16 up to 64. 37. IP-MAC-Port Binding (IMPB) DHCPv6 Snooping 38. IP-MAC-Port Binding (IMPB) IPv6 ND Snooping 39. IP-MAC-Port Binding (IMPB) 3.8 which can prevent the netcut attack 40. MAC-based Access Control (MAC) enhancement - Enlarger the number of local database from 128 to 1024 - Support Authentication Database failover: Allows to configure the switch to check local database or bypass authentication when configured RADIUS server fails - Support compound authentication - Support configurable per port/system maximum users - Delete the log when passing authentication. - Add four logs to record whether the port/system reaches to the maximum or recovers port learing. - MBAC enters stop learning state. - MBAC recovers from stop learning state. - Port < [unitID:]portNum> enters MBAC stop learning state. - Port < [unitID:]portNum> recovers from MBAC stop learning state. - IP Directed Broadcast. - ARP enhancement: - Show arpentry by mac address. - Add OIDs to clear ARP table - Proxy ARP. - RIPv1/v2. - Interface enhancement: - Support secondary IP in L3 interface. - Expand IP interface number from 32 to 64. - Route enhancement: - Allow to configure route preference - Show IP route “hardware” option : display only the routes written into the chip - Trace route supports IPv6. - IP Tunnel enhancement: - 6to4 Tunnel - Manual Tunnel - ISATAP Tunnel (Intra-Site Automatic Tunnel Addressing Protocol) - Display box and port information in “show ipv6 neighbor_cache”. - RIPng. - IPv6 static route supports backup route. - DHCPv6 Server. - DHCPv6 Relay. - DHCPv6 Client. - Ping enhancement: - Specify source IP address for ping request packet - Enalbe / disable broadcast ping reply - FQDN support - ping/tracert /tftp/telnet applications support fully qualify domain name - Remote Copy Protocol (RCP) : allow users to copy firmware images, configurations and log files between the Switch and RCP Server - DHCP server: enlarge the DHCP pool entries to 1024 along with 8 pools - DNS Relay. - DHCP/BOOTP Relay- Support DHCP local relay function that can insert option 82 information into DHCP broadcast packets from clients - Block recievied broadcast DHCP discover packets from flooding in local VLAN - DHCP Relay option 60 & 61 - RSPAN (Remote Switched Port Analyzer) support: Can monitor and analyze the traffic passing through the ports in another switch. - IGMP Snooping supports Data Driven Learning so the switch will build up the entry in its multicast table when sniffing the multicast traffic from the media server. Typically used in an environment where the multicast server is connected to the switch directly. - Route redistribution: allows routers on the network, which are running different routing protocols to exchange routing information. - Enlarge MAC Base Access Control local DB to 1024. - Add traffic control “countdown” parameter: Timer for shutdown mode (only supported in CLI). - Update sFlow version from 1.00 (IPv4) to V5 (IPv6). - Add digital signature in D-View module. Note: Suggest user to reconfigure WAC again after upgrading firmware from R2.60 to R2.7 |
| V2.60B26 | - Selective Q-in-Q -VLAN Translation - L2 Protocol Tunneling - LLDP - sFlow - IMPB v3.5 - Web-based Access Control (WAC) -Supports identity-driven QoS: Can assign ingress/egress bandwidth control and 802.1p default priority to the port according to the attributes dispatched from RADIUS server -Can enable / disable “RADIUS or Locally Assigned Information”: when enabled, the Switch will accept the parameters (if applicable) assigned from RADIUS server or Local database and treat them as highest priority. - MAC-based Access Control (MAC) enhancement -Supports identity-driven QoS: Can assign ingress/egress bandwidth control and 802.1p default priority to the port according to the attributes dispatched from RADIUS server -Can enable / disable “RADIUS or Locally Assigned Information”: when enabled, the Switch will accept the parameters (if applicable) assigned from RADIUS server or Local database and treat them as highest priority. - JWAC enhancement -Update server entries increased to 100 -Supports identity-driven QoS: Can assign ingress/egress bandwidth control and 802.1p default priority to the port (port-based) or host (host-based) according to the attributes dispatched from RADIUS server -Customizable page -Changed the default time for JWAC quarantine server error timeout from 30 seconds to 60 seconds -Increased the maximum concurrent user login to 50 per port and 100 per device -Can enable / disable “RADIUS or Locally Assigned Information”: when enabled, the Switch will accept the parameters (if applicable) assigned from RADIUS server or Local database and treat them as highest priority. - 802.1X enhancement -Able to force 802.1X client to go offline -Supports 802.1X PDU forwarding when 802.1X is disabled -Supports identity-driven QoS: Can assign ingress/egress bandwidth control and 802.1p default priority to the port according to the attributes dispatched from RADIUS server -Supports maximum of 128 clients per port, 1,024 clients per switch and 4,000 clients per stack -Compatible with Cisco ACS Server: admin can use Cisco ACS RADIUS Server for 802.1X authentication - Can enable / disable “RADIUS Assigned Information”: when enabled, the Switch will accept the parameters (if applicable) assigned from RADIUS server and treat them as highest priority. - Compound Authentication - Authentication Database Failover: Be able to switch to local database for authentication when RADIUS server fails - RADIUS Accounting: accounting and billing services for 802.1X Clients - Per-flow Bandwidth Control (ACL Flow Metering) -CIR (Committed Information Rate) -Two-rate Three Color Marker (TrTCM) -Single-rate Three Color Marker (SrTCM) - DHCP Server - DHCP Server Screening - RSPAN - IGMP Snooping enhancement -IGMPv3 Snooping -IGMP Snooping Fast Leave for IGMPv2 host -IGMP Snooping Report Suppression -IGMP Snooping dynamic group entries changed from 2K to 1K, which is shared with 64 static group entries -When a port receives unicast protocol packets (such as OSPF Hello packet), this port cannot change to dynamic router port; when a port receives multicast protocol packets (such as DVMRP probe, PIM Hello packet or IGMP query packets ), this port will change to dynamic router port - MLD Snooping enhancement -MLDv2 Snooping -MLD Snooping dynamic group entries changed from 1K to 511 - L2 Multicast VLAN Replication (Static configuration): Admin can manually configure the switch to route multicast traffic across VLANs - STP Root Restriction (defined in 802.1Q-2005) - 802.1D-2004 - Gratuitous ARP - Three-Level User Account - ACL enhancement -User-defined packet content and mask -Flow-based (ACL) mirroring -ACL Statistics (counters) -display remaining ACL rules -replace_dscp action for Ethernet ACL - 2nd IPv4 Static Default Route - DHCP Relay option 60 & 61 - Password Encryption - DHCP-NAP support - Telnet Client - Trusted Host enhancement -Can create trusted host not only for one IP but also for network range -Can delete all trusted hosts with one command - Bandwidth Control enhancement -changes per-port min. granularity from 64kbps to 1kbps - Ping MIB - Traceroute MIB - Entity MIB - Can enable / disable SNMP State; default is disable - When primary route is active, always use primary route over backup route. - When DHCP Relay is enabled, the Switch will block all broadcast DHCP packets in the local IP Interface - LBD will send traps when loops are detected and recovered - Configurable SSH Server TCP port - When configuring static multicast_fdb, typing 01005exxxxxx or 333xxxxxxxx is not allowed - ipif_ipv6_link_local_auto can be enabled or disabled; default is disabled - Added parameter ‘ip address’ to the command “show iproute” - Admin can specify which Firmware image ID the switch will use during boot-up - Added an extra /y parameter for commands which prompt (Y/N) - Admin can manually configure per-port speed (capability advertisement) used for Auto Negotiation between ports: admin can configure a port to advertise a certain speed (10_full) even if it’s connected to a port set to auto. - Network Monitoring Commands enhancement -“show utilization ports” will display TX/RX packets/second -“show error ports” will display TX/RX counters -“show ports” will display more details (auto negotiation / port transceiver type) - Enabled “Show FDB” by VID as well as by VLAN Name - Enabled “Show VLAN” by VID as well as by VLAN Name - Web-based GUI: Changed D-Link logo’s link to www.dlink.com.tw - Attack log will include IP address, MAC address and port number - Admin account can remove MAC address display from log - “Show Fan Status” command enhanced with log and trap - “Show STP ports” command is standardized for all slave and master switches in a stack - Added MIB for “Show Memory” usage and percentage (DRAM utilization, Flash utilization) - Added link up/down trap per port (RFC 2233) - Modified RPS MIB description and added traps - OID added to show port utilization - OID added to clear FDB and ARP table Notes: Please make sure all the switches in a stack are upgraded to R2.60, since some new or enhanced features might not work properly in a mixed-code stack. |
| V2.35.B09 | - MAC/Port-based MAC authentication with Switch or RADIUS - MAC-based VLAN - Cable Diagnostics - Loopback Detection 4.0 - PVID auto-assignment - Port-based JWAC function - D-View 6.0 support - 802.1X Guest VLAN - New CLI Command: “show mac based vlan” - Serial Number Display (Web, MIB and CLI) |
| V2.30.B10 | - JWAC support - ISM VLAN - Inter-VLAN routing enhancement No need to manually configure host’s MAC address |
| V2.00.B52 | - Physical Stacking via optional CX4 (or XFP) module - Allows trunking or mirroring to span multiple units of the stack - Support per-port / per-device BPDU filtering - 802.1v Protocol-based VLAN - Double VLAN - Guest VLAN - Supports 32 IP Interfaces - Time-based ACL - IP-MAC-Port Binding (IMPB) - DHCP Relay Option 82 - IPv6 Ready Logo Phase 1 - Supports Ether-like MIB, IF MIB - Enhancement for broadcast storm control logging - Provides enhanced messages about “Current Tagged ports”, “Current Untagged ports”, and “Static Tagged ports” when using “show vlan” command - Supports “Delete ACL all” command in CLI, web and SNMP - Add “ping” command to user privilege - Adding the missing “query info table” MIB |

# Changes in MIB & D-View Module:

The new features of MIB file are also included in the corresponding D-View module. Please download the D-View module on http://tsd.dlink.com.tw. For detailed changes of MIB content, please refer to the modification history in each MIB file.

| Firmware Version | MIB File | New Features |
| --- | --- | --- |
| V2.70.B56 | ACL.mib | - ACL supports "IPv6 IP + UDP/TCP port" together - Enlarge number of ACL profiles/rules |
|  | AGENT-GENERAL-MIB | Enlarge the number of trusted hosts to 30 |
|  | ARPSpoofingPrevention.mib | ARP Spoofing Prevention |
|  | Auth.mib | - Support Per VLAN authentication - Support Authentication Database failover: Allows to configure the switch to check local database or bypass authentication when configured RADIUS server fails - Support compound authentication - RADIUS server setting supports ipv6 - 802.1X - Support "force log off (supported only in MIB)" - Support "1X BPDU forwarding"" - Support configurable maximum users feature per port/system (128/4000) |
|  | BPDUProtection.mib | BPDU Attack Protection |
|  | DHCPServer.mib | DHCP server: enlarge the DHCP pool entries to 1024 along with 8 pools |
|  | DHCPv6Relay.mib | DHCPv6 Relay |
|  | DHCPv6Server.mib | DHCPv6 Server |
|  | DNSResolver.mib | DNS Client |
|  | DULD.mib | D-LINK Unidirectional Link Detection (DULD) |
|  | Equipment.mib | - "Change Stacking priority" can work without reboot - Stacking force master role feature - Show stack information and show log include information about stacking topology |
|  | ERPS.mib | ERPS: support 2 rings |
|  | Genmgmt.mib | - Allow to specify “src_file”/“dst_file”/ “domain_name” in download/upload functions - Show memory/flash utilization - CLI Command logging - Support new OID to clear dynamic FDB by port/by VLAN - Support new OIDs to clear ARP - Enable/disable broadcast ping reply - FQDN support - ping/tracert /tftp/telnet applications support fully qualify domain name - Log/Trap eight level support - Support “details “ and “media_type” parameter in “show ports” command Support “increment” when using “download cfg_fromTFTP” |
|  | IPMacBind.mib | - IP-MAC-Port Binding (IMPB) DHCPv6 Snooping - IP-MAC-Port Binding (IMPB) IPv6 ND Snooping - IP-MAC-Port Binding (IMPB) 3.8 which can prevent the netcut attack |
|  | IPv6StaticRoute.mib | Allow to create static route for IPv6 tunnel feature |
|  | JWAC.mib | JWAC enhancement(support JWAC v2.02) - Show IP address and user ID in UI and log - JWAC notification: Show notification to user automatically after passing the authentication. - System administrator can use FQDN URL to access JWAC login page instead of using IP |
|  | L2mgmtDGS3426.mib L2mgmtDGS3426P.mib L2mgmtDGS3427.mib L2mgmtDGS3450.mib | - Mirror enhancement: - Multiple sessions of mirroring - Link aggregation ports can be set as a target port - Support IGMP snooping report suppression - Support static IGMP snooping group - Support IGMP Snooping Host Based Fast Leave - Support Tagged / Untagged member ports - Support Tagged / Untagged source ports - Configurable multicast VLAN priority - Do not limit the total number of multicast addresses per ISM-VLAN entry when using “config igmp_snooping multicast_VLAN_group” - VLAN trunking - Per queue egress bandwidth control. - Port Security: changes maximum_learning_addr from 16 up to 64. - Support DHCP local relay function that can insert option 82 information into DHCP broadcast packets from clients - Enable/disable cpu_rx_rate_control (supported only in CLI/MIB) - Support “details” and “media_type” parameters in “show ports” command |
|  | l3mgmtDGS3426.mib l3mgmtDGS3426P.mib l3mgmtDGS3427.mib l3mgmtDGS3450.mib | - IP Directed Broadcast - DHCPv6 Client - DHCP Relay option 60 & 61 - Add parameter in "show ports" command - Allow to configure route preference |
|  | mba.mib | MAC-based Access Control enhancement - Enlarge the number of local authentication entries from 128 to 1024 - Support dynamic 802.1p, rate-limiting, assignment after successful authentication (with both Port-based and Host-based) - Enlarge MBAC Local DB to 1024 |
|  | Nlb.mib | Microsoft NLB support |
|  | MSTP.mib | - Support 802.1D 2004 edition - Support STP 1Q 2005 MSTP - STP Root Restriction - Support the BPDU address setting on NNI port when QinQ is enabled |
|  | PktStormCtrl.mib | Storm control enhancement: - Change "countdown" to "3-30" - Change "time_interval" to "5 - 600" Auto recovery for shutted-down port |
|  | RCP.mib | Remote Copy Protocol (RCP): allow users to copy firmware images, configurations and log files between the Switch and RCP Server |
|  | RFC1213.mib | - Show arpentry by MAC address - Microsoft NLB support |
|  | RFC2925P.mib | - Allow to specify source IP address for ping request packet - FQDN support - ping/tracert /tftp/telnet applications support fully qualify domain name. - Specify source IP address for ping request packet - Add IPv6 ping |
|  | RFC2925T.mib | - FQDN support - ping/tracert /tftp/telnet applications support fully qualify domain name. - Add IPv6 traceroute |
|  | RFC4087.mib | IP Tunnel enhancement - 6to4 Tunnel - Manual Tunnel - ISATAP Tunnel (Intra-Site Automatic Tunnel Addressing Protocol) |
|  | RIPng.mib | RIPng |
|  | RSPAN.mib | RSPAN enhancement |
|  | sFlow.mib | 1. sFlow enhancement: - Support ipv6 server - Support TX flow sampling 2. Change sFlow version from V1 to V5 |
|  | SrcIPIf.mib | SNMP-server & syslog source-interface appointment |
|  | SubnetVLAN.mib | Support subnet-based VLAN |
|  | WAC.mib | WAC enhancement: - Identity driven policy assignment: Can assign ingress/egress bandwidth control, and 802.1p default priority to the port according to the attributes dispatched from RADIUS server Add log - Support host-based authentication mode : assign ingress/egress bandwidth control for all hosts to the port; assign VLAN or 802.1p default priority to the host after successful authentication in host-based mode(R2.50 only supports assign VLAN in port-based) - Support IPv6 - Support virtual IP: used to accept authentication requests from unauthenticated hosts. Only the requests sent to this IP will get response correctly. - Support time control for authenticated client (e.g. aging time/idle time/block time) - Can enable/disable WAC authentication state |
| V2.60.B26 | Q-in-Q MIB | Selective Q-in-Q |
|  | Agent-MIB | Gratuitous ARP |
|  | LLDP-MIB LLDP-dot-MIB LLDP-dot3-MIB | LLDP |
|  | SFLOW-MIB | sFlow |
|  | DHCP-Server-MIB | DHCP Server |
|  | AUTH-MIB | - Compound Authentication - 802.1X enhancement -able to force 802.1X client to go offline -supports 802.1X PDU forwarding when 802.1X is disabled -supports identity-driven QoS: Can assign ingress/egress bandwidth control and 802.1p default priority to the port according to the attributes dispatched from RADIUS server. - supports maximum of 128 clients per port, 1,024 clients per switch and 4,000 clients per stack - Can enable / disable “RADIUS or Locally Assigned Information”: when enabled, the Switch will accept the parameters (if applicable) assigned from RADIUS server or Local database and treat them as highest priority |
|  | RSPAN-MGMT-MIB | RSPAN |
|  | RADIUS-ACCOUNTING-MIB | RADIUS Accounting |
|  | FILTER-MIB | DHCP Server Screening |
|  | ACLMGMT-MIB | - Per-flow Bandwidth Control (ACL Flow Metering) -CIR (Committed Information Rate) -Two-rate Three Color Marker (TrTCM) -Single-rate Three Color Marker (SrTCM) - ACL enhancement -User-defined packet content and mask -Flow-based (ACL) mirroring -ACL Statistics (counters) -display remaining ACL rules -replace_dscp action for Ethernet ACL |
|  | IP-MCST-VLAN-REP-MIB | L2 Multicast VLAN Replication (Static configuration): Admin can manually configure the switch to route multicast traffic across VLANs |
|  | MSTP-MIB | STP Root Restriction (defined in 802.1Q-2005) |
|  | MLD-SNOOPING-MIB | MLD Snooping enhancement -MLDv2 Snooping |
|  | IP-MAC-BIND-MIB | IMPB v3.5 |
|  | JWAC-MIB | JWAC enhancement -update server entries increased to 100 -customized page -changed the default time for JWAC -quarantine server error timeout from 30 seconds to 60 seconds -increased the maximum concurrent user login to 50 per port and 100 per device |
|  | WebBase-Access-Control-MIB | Web-based Access Control (WAC) -supports identity-driven QoS: Can assign ingress/egress bandwidth control and 802.1p default priority to the port according to the attributes dispatched from RADIUS server. |
|  | Mac-based-Authentication-MIB | MAC-based Access Control (MAC) enhancement -Supports identity-driven QoS: Can assign ingress/egress bandwidth control and 802.1p default priority to the port according to the attributes dispatched from RADIUS server. |
|  | SSH-MIB | Configurable SSH server TCP port |
|  | DGS-3426-L2MGMT-MIB DGS-3426P-L2MGMT-MIB DGS-3427-L2MGMT-MIB DGS-3450-L2MGMT-MIB | - IGMP Snooping enhancement -IGMPv3 Snooping -IGMP Snooping Fast Leave for IGMPv2 host -IGMP Snooping Report Suppression - LBD will send traps when loops are detected and recovered - Bandwidth Control: min. port granularity changed from 64kbps to 1kbps - Admin can manually configure per-port speed advertisement: used for Auto Negotiation between ports |
|  | DGS-3426-L3MGMT-MIB DGS-3426P-L3MGMT-MIB DGS-3427-L3MGMT-MIB DGS-3450-L3MGMT-MIB | - DHCP Relay option 60 & 61 - 2nd IPv4 Static Default Route - ipif_ipv6_link_local_auto can be enabled/disabled; disabled by default |
|  | AGENT-GENERAL-MIB | - Trusted Host enhancement -Can create trusted host not only for one IP but also for network range -Can delete all trusted host with one command - Admin can specify which Firmware image ID the switch will use during boot-u - MIB for Show Memory usage and percentage - OID to show port utilization - OID to clear FDB and ARP table |
|  | EQUIPMENT-MIB | - Enable logs and traps for Show Fan Status - Modified RPS MIB description and added traps |
|  | IF-MIB | Link up/down trap per port (RFC 2233) |
|  | DISMAN-PING-MIB | Ping MIB |
|  | DISMAN-TRACEROUTE-MIB | Traceroute MIB |
|  | ENTITY-MIB | Entity MIB |

# Changes in Command Line Interface:

The section below only shows command line changes that may bring backward compatibility issues with configuration settings for previous version of firmware.

Any new feature commands that do not have backward compatibility issues are not included in the below section.

| Firmware Version | Changes |
| --- | --- |
| V2.70.B56 | - Modify the "show 8021x user" command to：”show 802.1x user” - Delete the old WAC command: config wac VLAN If you have configured WAC VLAN on firmware prior to v2.7, when upgrading to v2.7, you do not need to configure it again because WAC Authentication ports will be reserved - Change the command download [firmware_fromTFTP [<ipaddr> ¦ <ipv6addr>] <path_filename 64> {{unit [<unitid 1-12> ¦ all]} <drive_id>} <pathname 64> {boot_up} ¦ cfg_fromTFTP [<ipaddr> ¦ <ipv6addr>] src_file <path_filename 64> {[dest_file {<drive_id>} <pathname 64>]} ] to download [firmware_fromTFTP [<ipaddr> ¦ <ipv6addr>] src_file <path_filename 64> {dest_file {{unit [<unitid 1-12> ¦ all]} <drive_id>} <pathname 64> {boot_up}} ¦ cfg_fromTFTP [<ipaddr> ¦ <ipv6addr>] src_file <path_filename 64> {[dest_file {<drive_id>} <pathname 64> ¦increment]} ] Note: From v2.7 onward, 2 parameters (src_file, dest_file) are added. This improvement is to avoid potential command parsing problem. If you have upgraded the firmware to V2.70 or onward, and are using script to manipulate firmware or config file, please do not forget to add those 2 parameters to the script.. |
| V2.60.B26 | None |

# Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V2.70.B56 | - Via SNMP, the counter value of the port will be 0 when the port is linked down (DI20090820000010, DI20090827000008) - When user logins via JWAC and continuously moves client PC on different ports, the switch session table will be full, and then the PC can not get IP address from the DHCP Server. User can see the log "JWAC enters stop learning state" (DI20090422000006). - After user links up/links down one port on DGS-3400, the SNMP host can not receive SNMP trap about link change from DGS-3400. (DI20090924000019) - Rebooting the slave stacking unit while JWAC authentication from clients are undertaken will cause the slave unit to show “Device Discovery” after its rebooting. (DI20091005000016) - DGS-3400 does not erase IGMP Snooping entries for LAG port (DI20091028000024) - DGS-3400 does not erase MLD Snooping entries for LAG port.(DI20091028000025) - Some stacking units are unstable when IMPB and DHCP snooping are enabled (DI20091109000006) - Some clients’ IP addresses are not written to "binding_entry" table when 12 GDS-3400 are in a stack with IMPB and DHCP Snooping enabled. (DI20091111000007) - DGS-3400 does not flood IPv6 control packets when MLD Snooping is enabled. (DI20091028000013) (DI20091028000015) - When Topology Change has occurred on the LAG port over stacking units, DGS-3400 sends many SNMP traps and Syslog packets which are related to RSTP Topology change. (DI20100125000011) - JWAC Customized Page displays error after saving and rebooting the switch (DI20090921000002) - DGS-3426 may enter exception mode when configuring some SNMP commands (DI20100125000014) - Clients cannot be authenticated by JWAC, because DGS-3400 JWAC does not respond to the VLAN-tagged packets. (DI20090724000011) - If the SNMP manager sends "SNMP get request" about "ifNumber: 0" to DGS-3400, the value of response is incorrect. (DI20090813000010) - High CPU utilization issue (DUSA20090227000001) (1) When LACP runs across stacking units (total two stacking units), powering off the master unit will make LACP re-run and it needs around 5-7 seconds to recover. (2)After powering on the Master and reconnecting the Master back to the stack topology, the LACP will lose traffic around 1.7 second. (3)The DGS-3400 CPU utilization is up to 100% when receiving a lot of broadcast traffic. - Device will send out the RADIUS packets with NAS-Identifier not as "D-Link".(DEUR20100118000006) - The character in last line of uploaded configuration file is not 0x0D0A (DI20100309000013) - When using WindowsXP with window size 80*25 to telnet the device and then execute "show config", some command characters are missing if the command length is longer than 80. (DI20100316000008) - DGS-3400 series can not recognize Cisco STP packets thus STP port role and status are not changed when DGS-3400 is connected to a Cisco Switch (DI20100316000010) - Some MAC addresses are recorded as "BlockByMACAuth" even though these MACs are already authorized by MAC Base Access Control (MAC)feature as "Authenticated". (DI20091117000012). - When STP is enabled, DGS-3400 CPU utilization becomes 100% after unplugging and plugging the port connection between the stack master and slave. (DI20091119000008) - DGS-3400 CPU becomes high utilization when cleaning the IMPB entry (DI20100118000011) - When powering off/on a stacking member unit, the STP topology was changed and the stacking unit is sometimes in loop condition for a while. (DI20100514000005) |
| V2.60.B26 | - When DGS-3400s are in stacking mode, powering off unit 1~5 and then powering them back will sometimes cause stack recovery failure. (DI20080818000001) - After user resets the switch and then enables stacking via Web GUI, the switch cannot be pinged. (DI20081224000007) - Web GUI does not display ACL rules correctly. (DI20090618000010) - Admin cannot use the CLI command “config double_vlan d169 add access 23” to add double VLAN access member port. (DT20081222000002) - SSH Login: When using OpenSSH 5.1p1 and a particular script file to test, the switch will enter exception mode. (DI20081106000011) - Loopback Detection (LBD) will not always activate if the loop traffic includes STP BPDU. (DI20081118000011) |
| V2.35B09 | - Sometimes Link Aggregation group does not function when stacking mode is enabled - Under certain setup the desired VLAN is not being assigned to authenticated wireless client but instead the AP’s managed VLAN - In stacking mode, sometimes backup master will not become master when the master fails in a stack - Remove the PoE menu from Web GUI for non-PoE DGS-3400 models |
| V2.30B10 | - Single IP Management (SIM) only works with default VLAN - Password string display disclosed when the first character has been removed - Instability issue between Intel 10G NIC and DGS-3400 Series 10G modules (DEM-410CX) which causes link down/link up frequently |
| V2.00B52 | - User logins in through SSH successfully but log shows that both SSH login and console login event happen at the same time - Switch hangs when using Telnet to create 128 ACL rules - Wrong ACL OID is retrieved via snmpwalk tool - Syslog cannot accurately classify the severity of the message - Switch enters Exception Mode when saving through Telnet - Creating ACL via Web GUI will cause the web management to go down - Creating CPU filtering ACL will cause the web management to go down - SNMP compatibility issue at ACL with Firewall DFL-1600 - Wrong warm_start trap type is sent while rebooting - DGS-3400 series cannot use web to check MAC address table by using vlan name if the vlan name is longer than 10 digits |
| V1.20B23 | - Modify the naming of “LoopBack Guard” to “Loopback Detection” on Web GUI - Change the default IP address to “10.90.90.90/8” |
| V1.00B35 | Initial Release |

* D-Link tracking number is enclosed in ()

# Known Issues:

| Firmware Version | Issues | Workaround |
| --- | --- | --- |
| V2.70.B56 | When powering off/on a stacking member unit, the STP topology was changed and the stacking unit is sometimes in loop condition for a while. (DI20100514000005) | None |
| V2.60.B26 | DGS-3450 only: Per port mapping of 802.1p priority and class is not supported when packets flowing between block 1 (port 1~24) and block 2 (port 25~48), and across devices in the same physical stack. When this happens the switch will use default mapping instead of the configured class mapping | None |

# Related Documentation:

- DGS-3400 Series User Manual

- DGS-3400 Series CLI Manual