xStack DGS-3600 Series SNMP MIB User Guide for v1.00B62

This document provides example of how to use the SNMP MIB Spec (the zip file contains one excel files:DGS-3600 MIB List.xls) as a reference to find out where the function locates in SNMP (DGS3600R1_1.00.B62.MIB.zip). Examples in the document include:

Save Configuration

SafeGuard Engine

CPU Utilization

IP-MAC-Port Binding

For detailed software features with corresponding firmware version, users should also refer to the external specification.

Example 1. Save Configuration

(1) In the “DGS-3600 MIB List.xls”, user can search for “savecfg” and the result of the specific mib will be shown below:

(2) Use notepad or text editor to open “dlkAgent.mib”, and user can find out how to save the configuration:

agentSaveCfg OBJECT-TYPE

SYNTAX INTEGER {

other(1),

idle(2),

set(3),

cfg-id1(4),

cfg-id2(5)

}

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"As the object is set to

set(3) - save the current device configuration into NV-RAM bootup ID.

cfg-id1(4) - save configuration into NV-RAM ID1.

cfg-id2(5) - save configuration into NV-RAM ID2."

::= { agentBasicConfig 6 }

Example 2. SafeGuard Engine

(1) In the “DGS-3600 MIB List.xls”, user can search for “safeguard” and the result is shown below:

(2) Use notepad or text editor to open “SafeGuard.mib” and detailed functions are described. For instance, to enable the SafeGuard Engine is to set “swSafeGuardAdminState” to “3”.

swSafeGuardAdminState OBJECT-TYPE

SYNTAX  INTEGER {

other(1),

disabled(2),

enabled(3)

}

MAX-ACCESS  read-write

STATUS  current

DESCRIPTION

"This object indicates the Safeguard engine state of the switch."

::= { swSafeGuardGblMgmt 1 }

Example 3. CPU Utilization

In the “DGS-3600 MIB List.xls”, user can search for “cpuutilization” and the result is shown below:

(2) In the “dlkAgent.mib”, user can find out how to display the 5 sec/1 min/5min CPU Utilization:

agentCPUutilizationIn5sec OBJECT-TYPE

SYNTAX  Integer32

MAX-ACCESS  read-only

STATUS  current

DESCRIPTION

"Time scale is 5 seconds. The value will be between 0%(idle) and 100%

(very busy)."

::= { agentCPUutilization 1 }

agentCPUutilizationIn1min OBJECT-TYPE

SYNTAX  Integer32

MAX-ACCESS  read-only

STATUS  current

DESCRIPTION

"Time scale is 1 minute. The value will be between 0%(idle) and 100%

(very busy)."

::= { agentCPUutilization 2 }

agentCPUutilizationIn5min OBJECT-TYPE

SYNTAX  Integer32

MAX-ACCESS  read-only

STATUS  current

DESCRIPTION

"Time scale is 5 minutes. The value will be between 0%(idle) and 100%

(very busy)."

::= { agentCPUutilization 3 }

agentDualImageStatus OBJECT-TYPE

SYNTAX  INTEGER {

not-supported(0) ,

supported(1)

}

MAX-ACCESS  read-only

STATUS  current

DESCRIPTION

"Status of dual image."

::= { agentBasicInfo 7 }

Example 4. IP-MAC-Port binding

(1) In the “DGS-3600 MIB List.xls”, user can search for  “ipmacbind” and the result is shown below:

(2) In the “IPMacBind.mib”, user can find out detailed functions such as enable/disable the IP-MAC-Port binding ACL mode:

swIpMacBindingACLMode OBJECT-TYPE

SYNTAX  INTEGER {

other(1),

enabled(2),

disabled(3)

}

MAX-ACCESS  read-write

STATUS  current

DESCRIPTION

"when enable(2), Ip Mac Binding will use acl mode"

::= { swIpMacBindingCtrl 2 }

And to setup an IP-MAC-Port entry:

swIpMacBindingEntry OBJECT-TYPE

SYNTAX		SwIpMacBindingEntry

MAX-ACCESS  not-accessible

STATUS		current

DESCRIPTION

"IP-MAC binding entry used to add\delete\configure the address

pair to the switch's authorized user database."

INDEX { swIpMacBindingIpIndex }

::= { swIpMacBindingTable 1 }

SwIpMacBindingEntry ::=

SEQUENCE {

swIpMacBindingIpIndex

IpAddress,

swIpMacBindingMac

MacAddress,

swIpMacBindingStatus

RowStatus,

swIpMacBindingPorts

PortList,

swIpMacBindingAction

INTEGER,

swIpMacBindingMode

INTEGER

}

swIpMacBindingIpIndex OBJECT-TYPE

SYNTAX		IpAddress

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"The IP address of IP-MAC binding."

::= { swIpMacBindingEntry 1 }

swIpMacBindingMac OBJECT-TYPE

SYNTAX		MacAddress

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"The MAC address of IP-MAC binding."

::= { swIpMacBindingEntry 2 }

swIpMacBindingStatus OBJECT-TYPE

SYNTAX		RowStatus

MAX-ACCESS	read-create

STATUS		current

DESCRIPTION

"The status of this entry."

::= { swIpMacBindingEntry 3 }

swIpMacBindingPorts OBJECT-TYPE

SYNTAX    PortList

MAX-ACCESS  read-create

STATUS    current

DESCRIPTION

"The port members of this entry."

::= { swIpMacBindingEntry 4 }

swIpMacBindingAction OBJECT-TYPE

SYNTAX    INTEGER{

inactive(1),

active(2)

}

MAX-ACCESS  read-only

STATUS    current

DESCRIPTION

"The action of this entry."

::= { swIpMacBindingEntry 5 }

swIpMacBindingMode OBJECT-TYPE

SYNTAX    INTEGER{

arp(1),

acl(2)

}

MAX-ACCESS  read-create

STATUS    current

DESCRIPTION

"The mode of this entry."

::= { swIpMacBindingEntry 6 }

On the MIB browser, it shows like below:

The example above is to enable the IP-MAC-Port binding ACL mode for ip address “192.168.0.100” and MAC address “AABBCCDDEEFF” on port 1.

Each ports digit ”xxxxxxxx” (Hex) represents four ports, in this example, the first ”8” is ”1000” in binary.

１	０	０	０

			

p1	p2	p3	p4

=> 80000000 indicates Port 1