DGS-3324SR

Date:   Oct. 28, 2003

| Product name: | 24-port 1000Base-T with 4 SFP Combo ports L3 Managed Stackable Switch |
| --- | --- |
| Product Group & Type: | D-Link redundant power supply system |
| Revision: | A1 |
| Available: | 2003/11/14 |
| Picture: |  |
| Why Buy This Product: | - More enterprise solutions: L3 switch supporting reliable star stacking architecture (used in conjunction with DGS-3324SRi), rich in functions with RPS support. - Similarity in technical support: this switch uses the same firmware as many of our L2 and L3 switches, so it should be easier for you to support technically. - Better altenative to DGS-3224SR L2 switch. Selling Points - Supports reliable star stacking architecture. - Combo 1000BASE-T/SFP GBIC, it's the trend. - High port density for low cost per port. - RPS is as an option does not add RPS cost to the switch itself. - An improved and much better software function set. |
| Keyfeatures: | - 24 10/100/1000BASE-T Gigabit ports - 4 combo SFP (Mini GBIC) ports - Up to 12 units per stack (ring architecture) or 6 units + 1 Stacking Master per stack (star architecture) - Recoverable ring stacking or fail-safe star stacking - Redundant power supply support - 88Gbps switching fabric - Auto MDI/MDIX uplink for all twisted-pair ports - IP routing supporting RIP-1, RIP-2, OSPF routing protocols, DVMRP, PIM Dense mode - 4K 802.1Q VLANs, IGMP snooping, eight 802.1p Priority Queues, port mirroring - Multi-layer ACL and DiffServ QoS - Administrator-definable port security - 802.3ad Link Aggregation port trunks of up to 8 Gigabit ports - Broadcast storm control - 802.3x Flow Control - Jumbo frame support - 802.1D compatible and 802.1w Rapid Spanning Tree for redundant backup bridge paths - Single IP address management per stack, SNMP v.1, v.2c,v.3 support, RMON monitoring, SYSlog, web-based management, Telnet, CLI through console port - 802.1x port-based/MAC-based access control - Per-port bandwidth control |
| Technical Details: | Hardware Device Ports - 24 auto-sensing 10/100/1000BASE-T ports (front panel) - 4 combo SFP (Mini GBIC) ports (front panel) - 2 10Gbps stacking ports (rear panel) - 1 RS-232 console port (front panel) Unit Stacking - Ring architecture: 12 units per stack - Star architecture: 6 units per stack + 1 DGS-3324SRi Stacking Master Port Standard/Function Support - IEEE 802.3 10BASE-T/802.3u 100BASE-TX/802.3ab 1000BASE-T - ANSI/IEEE 802.3 NWay auto-negotiation - IEEE 802.3x Flow Control - Auto MDI/MDIX - Port mirroring SFP (Mini GBIC) Support - IEEE 802.3z 1000BASE-LX (DEM-310GT transceiver) - IEEE 802.3z 1000BASE-SX (DEM-311GT transceiver) - IEEE 802.3z 1000BASE-LH (DEM-314GT transceiver) - IEEE 802.3z 1000BASE-ZX (DEM-315GT transceiver) Diagnostic LEDs Per device: - Power On/Off - Master - Console (login/POST status) - RPS in use/not in use Per 10/100/1000BASE-T port: - 1000Mbps speed, 10/100Mbps speed - Link/Activity Per SFP port: - Link/Activity Per stacking port: - Link/No Link 7-Segment Display To display unit ID in the stack Software IP Routing - IP v4 support - IP Fragmentation support - IP multi-netting - Routing protocols supported: Static routing RIP-1, RIP-2 OSPF v.2 VLAN - IEEE 802.1Q Tagged VLAN - GARP/GVRP - Asymmetric VLAN * - Number of VLANs: 4K static VLANs (max.) - Supports multiple IPs per VLAN * * Feature available in next firmware upgrade Priority Queues (CoS) - Standard: IEEE 802.1p - Number of queues: 8 Traffic Classification (CoS) Can be based on user-definable application types: - TOS - Diffserv (DSCP) - Port-based - MAC address - IP address - TCP/UDP port number Network Access Security - Port security features - 802.1x user authentication Port-based access control MAC-based access control * - RADIUS client - Multi-layer ACL based on: VLAN MAC address IP address Protocol type TCP/UDP port number 802.1p Diffserv (DSCP) - Other authentication protocols supported: SSH2 * TACACS * TACACS+ * SSL * * Features available in future firmware release Spanning Tree Protocol - 802.1D Spanning Tree compatible - 802.1w Rapid Spanning Tree * * 802.1w available in next firmware release IP Multicast - IGMP Snooping - IGMP v.2 - DVMRP - PIM Dense mode - PIM Sparse mode * * Function supported in future firmware upgrade MultiLink Trunking (MLT) Enables grouping of links between the switch and another switch or a server to provide higher bandwidth of up to 8 Gigabit ports with active redundant links. A multi-link is defined as trunked ports spanning multiple units of the stack for fail-safe connectivity to mission-critical servers and the network center. - Supports up to 32 trunking groups; up to 8 ports per group - Operation mode: load sharing - 802.3ad compatible Link Aggregation static mode - 802.3ad compatible Link Aggregation (LACP) * * Available in future firmware upgrade Performance Switch Fabric 88Gbps (32Mpps) Transmission Method Store-and-forward MAC Address Table 16K entries per device MAC Address Learning - Dynamic entries: automatic update - Static entries: user-defined IP Address Learning - Dynamic entries: automatic update - Supports Delete of individual IP addresses by dynamic learning Packet Filtering/Forwarding Rates (half duplex) 1,488,100 pps per port (max.) RAM Buffer 2MB per device Jumbo Frame Up to 9,216 bytes Configuration & Management Management Support - SNMP v.1, v.2c, v.3 * - Web-based management - Web GUI traffic monitoring - Web-based MAC address browsing - CLI (command line interface) - RMON monitoring - Telnet server - SYSLOG * SNMP v.3 available in next firmware release MIBs - MIB-II (RFC 1213) - Bridge MIB (RFC 1493) - RMON MIB (RFC 1757) - 802.1p Priority Queues (RFC 2674) - 802.1Q VLAN MIB (RFC 2674) - IGMP MIB (RFC 2833) - If MIB (RFC 2233) - Ethernet-like MIB (RFC 1643) - RIP MIB (RFC 1724) - OSPF MIB (RFC 1850) - CIDR MIB (RFC 2096) - D-Link enterprise MIB RMON Groups 1, 2, 3, 9 (Alarm, Statistics, History, Event) IP Number Self-identification Through DHCP client, Bootp client Firmware Upgrade TFTP Console Port DB-9 RS-232 DCE Physical & Environmental Power Input 100 to 240 VAC, 50~60 Hz Internal power supply Redundant Power Backup Support Connector to connect to external redundant power supply Power Consumption 90 watts (max.) Ventilation - 40 x 40 X10 mm DC fans x 2 - 60 x 60 X18 mm DC fans x 1 Dimensions 441 x 207 x 44 mm 19-inch rack-mount width, 1 U height Weight 3.15kg Operating Temperature 0 to 40 C Storage Temperature -25 to 55 C Humidity 5% to 95% non-condensing Emission (EMI) - FCC Class A - CE Class A Safety CSA International |
| EMI/EMC Report |  FCC No. 039H048F  CE No. _ _  VCCI No.  CTR-21 No.  C-Tick No.  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  Others |
| Safety |  TUV No.  UL No.  CSA No.( )  Others |
| Packaging Style |  Clamshell  White Box  Gift Box  Bulk  Others |
| Packaging Contents: | Cable ass'y RS232, Power cord and Switch stacking cable. 1U 19 inch rack mount ear , Flat head screw M3*6 and Rubber foot. |
| UPC Code: | DGS-3324SR/ A; 790069; 262067. DGS-3324SR/E; 790069; 262074. DGS-3324SR/N; 790069; 262081. DGS-3324SR/U; 790069; 262098. DGS-3324SR/BE; 790069; 262104. DGS-3324SR/CN; 790069; 262111. |
| Units Per Carton: | 5 |
| Cartons Per Pallet: | 24 (Pallet C Shipping per Sea) |
| Dimensions and Weight: | Box: 550*310*92 mm, 4.5 kg Carton: 573*333*500 mm, 23.5 kg Pallet: __120*110*213____ cm, _580 kg INTRASTAT weight: 4 kg (device + accessary + manual) Packing material weight - Paper: 100 g - Plastic: g - Mixed: 3900 g (device:3100g) |