DXS-3326GSR

Date: July 13, 2004

| Product name: | 24-port SFP + 4 1000BASE-T Combo ports L3 Managed Stackable Switch and optional 2-port 10GE uplink module |
| --- | --- |
| Product Group & Type: |  |
| Revision: | A1 |
| Available: |  |
| Picture: |  |
| Why Sell This Product: | - DXS-3326GSR: this is an all-fiber Gigabit switch with trendy SFP type of connections, plus combo 1000BASE-T for flexible copper Gigabit installation. - It's positioned as a high-end stackable switch for enterprise solution. - Part of the D-Link xStack; we give it a marketing slogan: "the Ultimate Stackable Alternative to Chassis." - Meets the requirements of today's enterprise switches: Robust Availability, High Security Control, High Quality of Service. - Features 10Giabit stacking and 10Gigabit uplink capability. - High density fiber Gigabit (24 SFP) & L3 switching ideal for core applications. Selling Points - Performance: 10Gigabit high bandwidth stacking, 10Gigabit uplink option. - Availability: reliable star stacking architecture, redundant power supply support. - Security & industry approach: 802.3ad Link Aggregation, 802.1D STP, 802.1W RSTP, 802.1S MSTP, RIP, OSPF, VRRP, Port/MAC based 802.1x, MAC-IP binding, Multi-layer ACL, RADIUS, TACACS+, SSH, SSL, 802.1p, DSCP, Priority queuing, Multi-layer traffic classification. - High port density, cost-effective SFP fiber Gigabit slots. - Stackable price, chassis function & performance. - Strong D-Link xStack technical support. |
| Keyfeatures: | - High port density per switch, with copper Gigabit, SFP fiber Gigabit support - Very high-speed 10G uplink option 10 times faster than Gigabit - Fault tolerant star stacking - Up to 6 units per stack, maximum 168 Gigabit ports per stack (star stacking) - Up to 12 units per stack, maximum 288 Gigabit ports per stack (ring stacking) - High bandwidth 120Gbps star stacking, 40Gbps ring stacking - Wire-speed packet routing supporting RIP v.1, v.2, OSPF, DVMRP, PIM Dense mode - Per-port bandwidth control & multi-layer (L2, L3, L4) QoS/CoS - Advanced Access Control Lists (ACL), enhanced TACACS/TACACS+ and RADIUS authentication - 802.3ad Link Aggregation port trunks - Jumbo frame support - High availability with redundant power supply, Spanning Tree support - Single IP Management, SNMP v.1, v.2c, v.3 support, RMON monitoring, Syslog, web-based management, Telnet, CLI through console port * D-Link SIM (Single IP Management) support for xStack available in firmware upgrade in the near future. |
| Technical Details: | Hardware Device Ports - 24 SFP - 4 Combo 10/100/1000BASE-T Gigabit ports - 2 10Gigabit stacking ports - 1 open slot - 1 console port Port Standard/Function Support - IEEE 802.3 10BASE-T/802.3u 100BASE-TX/802.3ab 1000BASE-T - ANSI/IEEE 802.3 NWay auto-negotiation - IEEE 802.3x Flow Control - Auto MDI/MDIX - Port mirroring SFP Support - IEEE 802.3z 1000BASE-LX (DEM-310GT transceiver) - IEEE 802.3z 1000BASE-SX (DEM-311GT transceiver) - IEEE 802.3z 1000BASE-LH (DEM-314GT transceiver) - IEEE 802.3z 1000BASE-ZX (DEM-315GT transceiver) XFP Support IEEE 802.3ae compliant transceiver Maximum Number of Units per Stack Star stacking 6 units, ring stacking 12 units * * Star stacking requires a DGS-3324SRi Stacking Master. Stacking Port Speed 20,000Mbps full-duplex per port Switch Fabric 128Gbps Packet Forwarding Rate 95Mpps Packet Buffer Memory 2MB Maximum IP Address Table Entries All models: 3K Maximum MAC Address Table Entries All models: 16K Software IP Routing - IP v.4 support - IP Fragmentation support - Multiple IP interfaces per VLAN * - Routing protocols supported: Static routing RIP v.1, RIP v.2 OSPF v.2 * * Function available in future firmware upgrade VLAN - Port-based VLAN - IEEE 802.1Q Tagged VLAN - GARP/GVRP - Maximum number of VLANs Dynamic: 255 Static: 4K Priority Queues (CoS) - Standard: IEEE 802.1p - Maximum number of queues: 8 Traffic Classification (CoS) Can be based on user-definable application types: - TOS - Diffserv (DSCP) - Port-based - MAC address - IP address - TCP/UDP port number Access Security - 802.1x port-based access control - 802.1x MAC-based access control - Port security - SSH * - SSL * * Functions available in future firmware upgrade Access Control List Multi-layer ACL based on: - MAC address - TOS - Diffserv (DSCP) - IP address - Packet protocol type - TCP/UDP port number (user-definable application types) - TCP/UDP payload (user-definable packet content) - Per port ACL definition Spanning Tree Protocol - 802.1D Spanning Tree compatible - 802.1w Rapid Spanning Tree - 802.1s Multiple Spanning Tree * * Function available in future firmware upgrade IP Multicast - IGMP v.2 - DVMRP - PIM Dense mode - VRRP * * Function supported in future firmware upgrade Port Trunk - Maximum number of ports per trunk: 8 - Maximum number of trunks per stack: 32 - Operation mode support: load sharing - 802.3ad compatible Link Aggregation (LACP) Transmission Method Store-and-Forward Jumbo Frame 9KB (max.) Configuration & Management Management Support - D-Link SIM (Single IP Management) * - SNMP v.1, v.2c, v.3 - Web-based management - Web GUI traffic monitoring - Web-based MAC address browsing - CLI (command line interface) - RMON monitoring - Telnet server (up to 8 sessions) - TACACS/TACACS+ authentication on switch admin access - RADIUS authentication on switch admin access - SYSLOG - DHCP/Bootp relay - DNS relay - Dual Images - Back-Door function - Port description - CPU utilization monitoring MIBs - MIB-II (RFC 1213) - Bridge MIB (RFC 1493) - RMON MIB (RFC 1757) - 802.1p Priority Queues (RFC 2674) - 802.1Q VLAN MIB (RFC 2674) - IGMP MIB (RFC 2833) - If MIB (RFC 2233) - Ethernet-like MIB (RFC 1643) - RIP MIB (RFC 1724) - OSPF MIB (RFC 1850) - CIDR MIB (RFC 2096) - D-Link enterprise MIB RMON Groups 1, 2, 3, 9 (Alarm, Statistics, History, Event) IP Number Self-identification Through DHCP client, Bootp client Firmware Upgrade TFTP Power Supply - 100 to 120VAC, 200 to 240VAC, 50/60Hz - Internal universal power supply - Optional external redundant power supply support Emission (EMI) - FCC Class A - CE Class A Safety CSA International Optional Redundant Power Supply DPS-500 140-watt redundant power supply DPS-800 2-slot redundant power supply chassis DPS-900 8-slot redundant power supply chassis |
| EMI/EMC Report | FCC No.  CE No.  VCCI No.  CTR-21 No.  C-Tick No.  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  Others |
| Safety |  TUV No.  UL No.  CSA No.  Others |
| Packaging Style |  Clamshell  White Box  Gift Box  Bulk  Others white box |
| Packaging Contents: | - One DXS-3326GSR Device - One set accessory including 2 Rack mount ear/4 Rubber feet and 8 Screws - One AC power cord - One Cable Ass’y RS232(DB09) - One Cable Infinband 4X50CM - One CD Kit for User’s Guide/CLI/D-View module - One CD Kit for D-View 5.1 Trial version. - One Generic QIG - One H/W hard copy User’s Guide |
| UPC Code: | XS3326GSA.A1: 790069 268519. XS3326GSBEA1: 790069 268502. XS3326GSCNA1: 790069 268557. XS3326GSE.A1: 790069 268526. XS3326GSN.A1: 790069 268533. XS3326GSU.A1: 790069 268540. |
| Units Per Carton: | 2 |
| Cartons Per Pallet: | Air: 12, Sea: 18 |
| Dimensions and Weight: | Box: 565 x 527 x 140 mm, 7 kg Carton: 580 x 560 x 310 mm, 14.5 kg Pallet: (by Air) 1220 x 1020 x 1250 mm(USA), 189 kg 1200 x 1100 x 1240 mm (WW), 189 kg (by Sea)1220 x 1020 x 1810 mm(USA), 276 kg 1200 x 1100 x 1800 mm (WW), 276 kg INTRASTAT weight: kg (Box) Packing material weight (Box): - Paper: kg - Plastic: kg - Mixed: kg |