# DWL-3500AP Firmware Release Note

=============================================================

Firmware: v2.1.0.9

Hardware: DWL-3500AP A1G/A2G

Date: February 19, 2008

Problems Resolved:

- The APs become "Connection Failed Access Points" occasionally.

- Wireless clients can not access AP after the AP starts up for several days.

- A large ping packet to the L3 tunnel will be dropped.

- The DHCP packets are dropped by the AP occasionally.

- In the presence of high amounts of traffic the user experiences high ping loss rate.

- Sometimes, when a new client associates with the AP, the switch (DWS-3000) displays a 'roam' trap message.

- The AP throughput is low.

Enhancements:

- Support Station Isolation under Managed mode.

- Support Antenna Diversity configurability.

- Enhance client association trap & syslog - following pieces of information are added: SSID, Authentication method (none / static wep / wep_8021x / WAP_personal / WPA_enterprise)

- Better MTU management – Dynamically change the MSS (Maximum Segment Size) field in the TCP SYN messages transmitted by wired clients into the tunneled VAPs as well as messages transmitted by wireless clients from the tunneled VAPs.

- Support Station Isolation under Standalone mode.

- Support Path MTU Discovery (RFC 1191).

- DWL-3500AP supports Unified AP: DWL-3500AP can work as Managed mode (managed by DWS-3000 series Unified Switch) or Standalone mode (without Unified Switch)

- Reveal ‘factory-reset’ command in the managed mode.

- Provide hardware reset button functionality (button pressed < 2 seconds - reboot.  Button pressed for >= 5 seconds - factory reset.).

Known Issues:

- Using TKIP/RC4 encryption with WPA results in roughly 30% lower throughput than using WPA2 AES encryption due to a hardware limitation.

- Turbo/superG mode does not increase traffic rate significantly for DWL-3500AP.

Note:

An AP with firmware v2.1.0.9 may have problem cooperating with a switch with firmware v1.0.2.3 or below. Be sure the switch and the AP are both upgraded to v2.1.0.9.

=============================================================

Firmware: v1.0.2.3

Hardware: DWL-3500AP A1G

Date: May 14, 2007

Problems Resolved:

- AP: Fix the EIRP value in some countries/regions.

- AP: Fix the issue that no 802.11a channels on UAE & Russia.

- Fix the WPA2 EAP-TLS

Enhancements:

- Enable the new Japan 5GHz regulation.

=============================================================

Firmware: v1.0.1.5

Hardware: DWL-8500AP A1G

Date: August 16, 2006

Problems Resolved:

None

Enhancements:

First Release, please refer to external spec for detail S/W specification.

Note:

None