| Firmware Version: | 2.2.0.17 |
| --- | --- |
| Published: | July 20, 2009 |

Content:

Revision History and System Requirement:	2

New Features:	2

Problems Fixed:	4

Known Issues:	5

Related Documentation:	5

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| Runtime: V2.2.0.17 | 20-July-09 | DWL-3500AP | A1G, A2G |
|  |  | DWL-8500AP | A1G, A2G |
| Runtime: V2.2.0.15 | 06-July-09 | DWL-3500AP | A1G, A2G |
|  |  | DWL-8500AP | A1G, A2G |
| Runtime: V2.2.0.12 [Official Release] | 20-Feb-09 | DWL-3500AP | A1G, A2G |
|  |  | DWL-8500AP | A1G, A2G |
| Runtime: V2.2.0.4 | 30-Oct-08 | DWL-3500AP | A1G, A2G |
|  |  | DWL-8500AP | A1G, A2G |
| Runtime: V2.1.0.10 | 30-April-08 | DWL-3500AP | A1G, A2G |
|  |  | DWL-8500AP | A1G, A2G |
| Runtime: V2.1.0.9 | 19-Feb-08 | DWL-3500AP | A1G, A2G |
|  |  | DWL-8500AP | A1G, A2G |
| Runtime: V2.0.0.6 (DWL-8500AP ONLY) | 30-Sept-07 | DWL-8500AP | A1G, A2G |
| Runtime: V1.0.2.3. | 14-May-07 | DWL-3500AP | A1G |
|  |  | DWL-8500AP | A1G |
| Runtime: V1.0.1.5 | 27-Oct-06 | DWL-3500AP | A1G |
|  |  | DWL-8500AP | A1G |
| Runtime: V1.0.0.5 | 16-Aug-06 | DWL-3500AP | A1G |
|  |  | DWL-8500AP | A1G |

# New Features:

| Firmware Version | New Features |
| --- | --- |
| V2.2.0.17 | - Added detailed upgrade warning message in AP CLI, GUI and Switch GUI. |
| V2.2.0.15 | - (DWL-8500AP ONLY) Added new Country Code for Russia. Russian regulation requires that all channels from 5.35GHz to 5.65GHz be disabled. |
| V2.2.0.12 | - (DWL-8500AP ONLY) Updated 802.11a regulatory information as well as power output for the following countries: TW – modified the channels to 52,56,60,64,149,153,157,161,165 SG – modified the channels to 36,40,44,48, 52,56,60,64,149, 153, 157, 161, 165 EC – modified the channels to 36,40,44,48, 52,56,60,64, 149, 153, 157, 161, 165 CR – modified the channels to 36,40,44,48, 52,56,60,64, 149, 153, 157, 161, 165 RO – modified the channels to 36, 40, 44, 48 MY – modified the channels to 36, 40, 44, 48 PK – modified the channels to 36, 40, 44, 48 - Added Brazil Domain to conform to FCC2 regulation |
| V2.2.0.4 | - (DWL-8500AP ONLY) Conforms to US Regulation (DFS-2) by blocking the related wireless channels - Creates a new factory reset option that retains the information needed to keep the AP in the same network and reachable. New CLI options added to factory reset called “keepmanagement” When the command “factory-reset keepmanagement” is issued, the following parameters on the AP are saved: system country management vlan-id management static-ip management static-mask management dhcp-status static-ip-route destination static-ip-route mask static-ip-route gateway untagged-vlan vlan-id untagged-vlan status Note: keepmanagement will not show up using “tab” key in CLI to ensure privacy. - Hosapd debug is sent to the syslog server when syslog is enabled on the AP. This allows capturing bug information in the field. |
| V2.1.0.10 | None, firmware version increment to synchronize with DWS-3000 Series |
| V2.1.0.9 | - Supports Unified AP: Can work as Managed mode (managed by DWS-3000 series Unified Switch) or Standalone mode (without Unified Switch) - Supports Station Isolation under Managed and Standalone mode. - Supports Antenna Diversity configurability - Enhance client association trap & syslog – following pieces of information are added: SSID, Authentication method (none / static wep / wep_8021x / WAP_personal / WPA_enterprise) - Better MTU management – Dynamically change the MSS (Maximum Segment Size) field in the TCP SYN messages transmitted by wired clients into the tunneled VAPs as well as messages transmitted by wireless clients from the tunneled VAPs - Supports Path MTU Discovery (RFC 1191) - Reveals ‘factory-reset’ command in the managed mode - Provides Hardware Reset button functionality (button pressed for <2 seconds – reboot, button pressed for >= 5 seconds – factory reset) - (DWL-8500AP ONLY) Enables 2 radios on each 1 antenna - (DWL-8500AP ONLY) Disables W56 channels for Japan |
| V2.0.0.6 | DWL-8500AP ONLY, not supported by DWL-3500AP - Supports Path MTU Discovery (RFC 1191) - DWL-8500AP supports Unified AP: DWL-8500AP can work as Managed mode (managed by DWS-3000 series Unified Switch) or Standalone mode (without Unified Switch) - Reveals ‘factory-reset’ command in the managed mode - Provide hardware reset button functionality (button pressed < 2 seconds – reboot. Button pressed for >= 5 seconds – factory reset.) - Support Station Isolation under Standalone mode |
| V1.0.2.3 | - (DWL-8500AP ONLY) Supports the new Japan 5GHz (W52, W53, W56) regulation |
| V1.0.1.5 | - In Sentry mode the Access Point scans all channels independent of the switch profile configuration |
| V1.0.0.5 | - First Release, please refer to External Specification for S/W features |

# Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V2.2.0.17 | - AP would timeout when upgrading flash on Web GUI. - An unnecessary save during AP startup was creating a window for the AP to reboot and corrupt configuration. |
| V2.2.0.15 | None |
| V2.2.0.12 | - Fixed a bug that caused RF Scan reports beacons to not be received on the current locked channel. [HQ20081210000014] |
| V2.2.0.4 | - Dynamic VLAN Assignment did not work properly. Also changed AP behavior to allow 240 VLANs total instead of 64 concurrently - AP reboots automatically due to memory leak |
| V2.1.0.10 | - (DWL-8500AP ONLY) If DWL-8500AP is operating with its 802.11a radio turned on in a country that requires radar detection for 802.11a channel selection (eg. GB, JP), sometimes the AP incorrectly becomes unmanaged |
| V2.1.0.9 | - The APs become “Connection Failed Access Points” occasionally - Wireless clients cannot access AP after the AP starts up for several days - A large ping packet to the L3 tunnel will be dropped - The DHCP packets are dropped by the AP occasionally - In the presence of high amounts of traffic the user experiences high ping-loss rate - Sometimes when a new client associates with the AP, the Switch (DWS-3000) displays a ‘roam’ trap message - AP Throughput is low. |
| V2.0.0.6 | DWL-8500AP ONLY, not supported by DWL-3500AP - Sometimes, when a new client associates with the AP, the switch (DWS-3000) displays a 'roam' trap message - The throughput is low under channel 11 of 802.11g band |
| V1.0.2.3 | - Fixed the EIRP value in some countries/regions - Fixed the issue that no 802.11a channels on UAE & Russia - Fixed WPA2 EAP-TLS |
| V1.0.0.5 | None |

# Known Issues:

| Firmware Version | Issues |
| --- | --- |
| V2.2.0.17 V2.2.0.15 V2.2.0.12 V2.2.0.4 V2.1.0.10 | - Using TKIP/RC4 encryption with WPA2 results in roughly 30% lower throughput than using AES encryption due to a hardware limitation - Turbo / superG mode does not increase traffic rate significantly - The hardware encryption for WEP is used only when a single BSSID is active on the radio. A second VAP precludes the use of hardware encryption causing low throughput - The APs support manual antenna selection on a per-radio basis. Automatic antenna selection is not supported - Tunneling is not expected to work with port-based routing interfaces. All tunneled routing interfaces must be VLAN routing interfaces. APs should not be attached to the switch via port-based routing interfaces as well - If the PoE cable is suddenly removed from an AP, the associated clients fail over to other APs after 6 ping losses. The roaming functionality works properly if the AP is rebooted or the radio is turned down - If a captive portal instance is associated with a disabled VAP, re-enabling the VAP does not automatically enable the captive portal instance - Channel 14 is not supported for 802.11g band for Japan country code - (DWL-3500AP ONLY) A watchdog timer has been introduced to stop the AP from hanging up occasionally |
| V2.1.0.9 | - (DWL-8500AP ONLY) If DWL-8500AP is operating with its 802.11a radio turned on in a country that requires radar detection for 802.11a channel selection (eg. GB, JP), sometimes the AP incorrectly becomes unmanaged. A patch that fixes this problem has been provided in 2.1.0.10 - An AP with firmware v2.1.0.9 may have problem cooperating with a switch with firmware v1.0.2.3 or below. Be sure the switch and the AP are both upgraded to v2.1.0.9 |
| V2.0.0.6 | - A switch with firmware v2.0.0.6 may have problem cooperating with an AP with firmware v1.0.2.6 or below. Be sure the switch and the AP are both upgraded to v2.0.0.6. (Only DWL-8500AP supports v2.0.0.6. DWL-3500AP does not support v2.0.0.6 at this moment. In an environment with DWL-3500AP, it is suggested to use v1.0.2.3 firmware for the switch |
| V1.0.2.3 V1.0.1.5 V1.0.0.5 | None |

# Related Documentation:

- DWS-3000 Series User Manual

- DWS-3000 Series CLI Manual

- DWL-3500AP & DWL-8500AP Unified AP Guide