Model Name

Date: 11/4/2009

| Product Name: | DWS-4026 |
| --- | --- |
| Revision: | A1 |
| Picture: |  |
| Why Sell This Product: | After two years of selling Unified Switch, D-Link sees a growing demand for higher throughput, more advanced switch features, the need for flexibility, scalability and 802.11n AP upgrade for wireless networks. DWS-4026 is D-Link’s next-generation Unified Switch with more advanced feature as well as 802.11n support. With DWS-4026, D-Link further expands its leadership in the Unified Wired and Wireless market. Following the success of DWS-3024 and 3026 models, which manage IEEE 802.11a/b/g Access Points, DWS-4026 is able to manage 802.11n Access Point, DWL-8600AP, for higher throughput, reliability and scalability. It is a full-featured Unified Switch that centrally manages up to 64 Access Points, delivering integrated wireless mobility and security for both wired and wireless users. |
| Key Features: | - 24 PoE Gigabit Ports with 2 10G uplink - Manages up to 64 APs per switch - Supports DWL-8600AP 802.11n Dual-band Access Point. - Manages up to 8 switches in one peer group - Switch Clustering for single-IP management - Wired and Wireless Captive Portal for robust client authentication - Adaptable Wireless enables Tunnel (L3 Tunneling) and Non-Tunnel (VLAN Forwarding) Modes - Seamless L2/L3 fast roaming from AP to AP - AP-AP Tunnel enables client traffic to be forwarded across subnets at the edge - Supports WEP, Dynamic WEP, WPA/WP2 Personal/Enterprise security - Wireless Intrusion Detection System (WIDS) - Rogue AP Detection, Classification and Mitigation - Auto Power Adjustment and Auto Channel Selection for managed AP. - Supports RIPv1/v2 dynamic routing |
| Technical Details: | Standards - IEEE 802.3, 802.3u, IEEE 802.3ab 1000BASE-T Gigabit Ethernet, 802.3z, 802.3ae compliance - Support Full-Duplex operations - IEEE 802.3x Flow Control support for Full-Duplex mode, back pressure support for Half-Duplex mode Protocol - CSMA/CD Switching Capacity - 88 Gbps Data Transfer Rates - 65.47 Mpps MAC address Table - 8K IP address Table - 128 Static Route External RPS Support - DPS-600 Physical & Environmental Max. Power Consumption 460 W Operating Temperature 0 to 40 ℃ Storage Temperature -10 to 70 ℃ Humidity - Operation: 10%-90% RH - Storage: 5% ~ 90% RH Dimensions 441mm x 389mm x 44mm |
| EMI/EMC Report | FCC No.  CE No.  VCCI No.  CTR-21 No.  C-Tick No.  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  IC No  Others: EN60601-1-2 |
| Safety |  TUV No.  UL No.  CSA No.  CB No.  CE  Others |
| Packaging Style | Clamshell  White Box Gift Box Bulk Brown Box Others |
| Packaging Contents: | - DWS-4026 device x1 - AC Power Cord x 1 - Console Cable x 1 - Rack-mount kit (1 set) - Screw for Rack-mount kit x 8 - Rubber feet x 4 - Generic Managed Switch Quick Installation Guide - CD-ROM with Manuals - Addendums for different regions |
| Country of Origin: | Taiwan |

| UPC Code: | UPC; Part Number. 790069; 325533; BWS4026A.....A1. 790069; 325540; EWS4026M.....A1G. 790069; 325557; IWS4026A.....A1G. 790069; 325564; IWS4026B.....A1G. 790069; 325571; IWS4026E.....A1G. 790069; 325588; IWS4026N.....A1G. 790069; 331565; IWS4026Z.....A1G. 790069; 325595; NWS4026C.....A1G. 790069; 325601; TWS4026A.....A1G. 458221; 7052103; JWS4026J.....A1G. |
| --- | --- |
| Units Per Carton: | For USA | For EU | For WW |
|  | 2pcs/carton | 2pcs/carton | 2pcs/carton |
| Cartons Per Pallet: | For USA | For EU | For WW |
|  | Air:12, Sea: 18 | Air:18, Sea: 18 | Air:12, Sea: 18 |
| Dimensions and Weight: | Box:  237x168x36mm (Type A)  168x237x53mm (Type A53)  225x150x75mm (Type B) 150x225x55mm (Type B1)  269x162x65mm (Type C)  274x208x68mm (Type D)  208x274x95mm(Type D95)  208x137x95mm (Type D95 half)  274x208x118mm (Type D1)  274x208x138mm (Type D2)  208x274x190mm (Type D4)  288x245x66mm (Type E)  245x285x82mm(Type E1) 245x285x87mm(Type E2)  256x356x97mm (Type F) 137x 208x 36mm (Type G)  208x250x68mm (Type H) 253x 274x247mm (Type I)  Others __565x530x145mm__, 7.725 kg Device (mm / g): Device Length; Device Width; Device Height; Device Weight. 441mm; 389mm; 44mm; 6.2930kg. Carton (cm/ kg): Carton Length; Carton Width; Carton Height; Carton Gross Weight. 589mm; 310mm; 566mm; 15.45KG. Pallet (cm/ kg): Carton per Air Pallet; Air Pallet PCS; Air Pallet Length; Air Pallet Width; Air Pallet Height. US 12 EU 18 WW 12; US 24 EU 36 WW 24; US 1020mm EU 1200mm WW 1200mm; US 1220mm EU 1100mm WW 1100mm; US 1262mm EU 1833mm WW 1267mm. Carton per Sea Pallet; Sea Pallet PCS; Sea Pallet Length; Sea Pallet Width; Sea Pallet Height. US 18 EU 18 WW 18; US 36 EU 36 WW 36; US 1020mm EU 1200mm WW 1200mm; US 1220mm EU 1100mm WW 1100mm; US 1828mm EU 1833mm WW 1833mm. INTRASTAT weight: 6.575kg Packing material weight (Box): - Paper: 0.885kg - Plastic: 0.265kg - Mixed: 0kg |