DES-6504

Date: March 25, 2005

| Product name: | 12 ports 100Base-FX (SFF) module for DES-6500 |  |
| --- | --- | --- |
| Revision: | A1 |  |
| Available: | 12/31/2004 |  |
| Picture: |  |  |
| Why Sell This Product: | - The DES-6500 L3 chassis switch with many advanced functions has been released some time ago. However, sales of this enterprise solution switch could not really take off without many optional port modules waiting in the wing for completion. Now these optional modules have been fully tested and are available, we’re announcing their release, including that of the DES-6504 12-port fiber Ethernet module. This particular module is equipped with the latest industry-standard SFF LC fiber connectors, so all 12 fiber ports can fit into a 19-inch rack-mount width card blade. Now users can take full advantage of the DES-6500’s advanced network functions, including its newly upgraded firmware with D-Link Single IP Management. - Target users: enterprise business, campus backbone integrators. Selling Points - High fiber port density on a single card blade. - Latest industry-standard SFF LC fiber connectors. - Multi-mode fiber supporting cable length up to 2 km suitable for most fiber applications (applications needing fiber cable length coverage longer than 2 km can use a DES-6509 module installation and take advantage of our SFP optic transceivers for single-mode fibers). |  |
| Key Features: | - 12 100BASE-FX Fast Ethernet ports - SFF LC connectors - Slide-in card module for DES-6500 chassis switch - Fully compliant with IEEE 802.3u 100BASE-FX - IEEE 802.3x compliant Flow Control support for full-duplex - Multi-mode fiber supporting up to 2 km cable distance |  |
| Technical Details: | Ordering Information Standards - IEEE 802.3u 100BASE-FX - IEEE 802.3x Flow Control Protocol CSMA/CD Data Transfer Rate - 100Mbps half duplex - 200Mbps full duplex Topology Star Number of Ports 12 fiber Ethernet ports Connectors 12 SFF LC Network Cable 1310nm multi-mode fiber Maximum Cable Distance Up to 2km Physical & Environmental AC Input 90 to 264 VAC, 47/63 Hz Through DES-6500 chassis switch power supply Power Consumption 30 watts (maximum) Operating Temperature 0 to 40 ℃ Storage Temperature -25 to 55 ℃ Humidity 5% to 95% RH, non-condensing Dimensions 330mm x 281mm x 27.5mm EMI - FCC Class A - CE - VCCI - C-Tick Box Includes - DES-6504 12-port 100BASE-FX (SFF) module - Quick Installation Guide |  |
| EMI/EMC Report | FCC No.  CE No.  VCCI No.  CTR-21 No.  C-Tick No.  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  Others |  |
| Safety |  TUV No.  UL No.  CSA No.  Others |  |
| Packaging Style |  Clamshell  White Box  Gift Box  Bulk  White Box with D-Link Logo  Others |  |
| Packaging Contents: | - DES-6504 - Module QIG |  |
| UPC Code: | DES6504…A1; 790069272622. NES6504...A1; 790069272639. |
| Units Per Carton: | 5 |  |
| Cartons Per Pallet: | For USA Air:18, Sea: 24 | For WW Air:12, Sea: 20 |
| Dimensions and Weight: | Box: Others , 1.9 kg Carton: 496x417x395 mm, 10.5 kg Pallet: (by Air) 1020x1220x1381 mm, 202 kg (for USA) 1100x1200x 1335 mm, 141 kg (for WW) (by Sea) 1020x1220x 2114 mm, 265 kg (for USA) 1100x1200x 2125 mm, 225 kg (for WW) INTRASTAT weight: 0.65 kg (Box) Packing material weight (Box): - Paper: 0.45 kg - Plastic: 0.1 kg - Mixed: 0.1 kg |  |