Optional Module & Switch Compatibility Table

Version 03

2002/6/18

CHECKED

DEPARTMENT             PREPARED

APPROVED

Judy Tsao

HLD

History:

| ##### Version | Description | Issue by | Date |
| --- | --- | --- | --- |
| 01 | Initial version | Judy Tsao | 2002/5/13 |
| 02 | - Adding model version for each module and product - Removing DES-3224R - Adding new products DES-3226S, DES-3326S and DHS-3224V and some new modules. | Judy Tsao | 2002/6/5 |
| 03 | - Update compatible modules for DES-3226, DES-3226S and DES-3326S | Judy Tsao | 2002/6/13 |

| Module Product |  | DES-3624i DES-3624iF | DES-3624 DES-3624F | DES-3225G DES-3225GF | DES-1218 DES-1226 | DES-1218R DES-1226R | DES-3326 | # DES-3226 | DES-3226*¹ | DES-3226S*¹ | DES-3326S*¹ | DHS-3224V*¹ | DHS-3224V*¹ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A3 | A4 | A4 | A1, A2 | A1 | A1 | A1 | B1 | B1 | B1 | A1 | B1 |
| DES-361G DES-362G (1000Base-SX) | A3 | √ (Rear Panel) |  |  |  |  |  |  |  |  |  |  |  |
| DES-361GL DES-362GL (1000Base-LX) | A3 | √ (Rear Panel) |  |  |  |  |  |  |  |  |  |  |  |
| DES-361T DES-362T (1000Base-T) | A1 | √ (Rear Panel) |  |  |  |  |  |  |  |  |  |  |  |
| DES-363S | A2 | √ (Rear Panel) |  |  |  |  |  |  |  |  |  |  |  |
| DES-3251G (1000Base-SX, 2Km) | A3 |  |  | √ (Rear Panel) |  |  |  |  |  |  |  |  |  |
| DES-3251GL (1000Base-LX, 15Km) | A3 |  |  | √ (Rear Panel) |  |  |  |  |  |  |  |  |  |
| DES-321T (1000Base-T) | A2 |  |  | √ (Rear Panel) |  |  |  |  |  |  |  |  |  |
| DES-362FM (100Base-FX, MTRJ) | A2 | √ | √ | √ |  |  |  |  |  |  |  |  |  |
| DES-122T (1000Base-T) | A1 |  |  |  | √ |  |  |  |  |  |  |  |  |
| DES-132G (1000Base-SX, 2Km) | A1 |  |  |  | √ | √ | √ | √ | √ | √ | √ |  |  |
| DES-132GL (1000Base-LX, 15Km) | A1 |  |  |  | √ | √ | √ | √ | √ | √ | √ |  |  |
| DES-132GB (GBIC) | B1 |  |  |  | √ | √ | √ | √ | √ | √ | √ |  |  |
| DES-132T (1000Base-T) | A1 |  |  |  |  | √ |  |  |  | √ | √ |  |  |
| DES-132 (100Base-TX) | A1 |  |  |  |  |  |  | TBD*² | √ | Release 2 | Release 2 |  |  |
| DES-131F (100Base-FX, 2Km) | A1 |  |  |  |  | √ |  | √ | √ | Release 2 | Release 2 |  |  |
| DES-132F (100Base-FX, 2Km) | A1 |  |  |  |  | √ |  | √ | √ | √ | √ |  |  |
| DES-131FL (100Base-FX, 15Km) | A1 |  |  |  |  | √ |  | √ | √ | Release 2 | # Release 2 |  |  |
| DES-132FL (100Base-FX, 15Km) | A1 |  |  |  |  | √ |  | √ | √ | √ | √ |  |  |
| DES-332T (1000Base-T) | A1 A2 |  |  |  |  |  | √ |  |  |  |  |  |  |
| DES-332GS (1*GBIC, 1*stacking port) | B1 |  |  |  |  |  |  |  |  | √ | √ |  |  |
| DES-322T (1000Base-T) | A1 |  |  |  |  |  |  | √ | √ (EMI fail) |  |  |  |  |
| DES-322T (1000Base-T) | B1 |  |  |  |  |  |  | √(EMI fail) | √ | √ | √ |  |  |
| DHS-321FX (100Base-FX, 2Km) | A1 |  |  |  |  |  |  |  |  |  |  | √ | √ |
| DHS-321FL (100Base-FX, 15Km) | A1 |  |  |  |  |  |  |  |  |  |  | √ | √ |
| DHS-321TX (100Base-TX) | A1 |  |  |  |  |  |  |  |  |  |  | √ | √ |
| DHS-321GL (1000Base-FX) | A1 |  |  |  |  |  |  |  |  |  |  | √ | √ |
| DHS-321S (1*stacking port) | A1 |  |  |  |  |  |  |  |  |  |  | TBD*³ | TBD*³ |
| Note: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. DES-3226/B1, DES-3326S/B1, DES-3226S/A1, DHS-3224V/A1,B1 are still in the RD developing phase. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. After upgrade firmware, DES-132 could be used in DES-3226/A2. This firmware is still in RD developing phase. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. The firmware supported DHS-321S is still in RD developing phase. |  |  |  |  |  |  |  |  |  |  |  |  |  |