ENGLISH

ENGLISH

Stand Alone Ethernet to VDSL Bridge

Quick Installation Guide

First Edition (Oct, 2001)

6DEV301ML.01

Printed In Taiwan

ENGLISH

Introduction

Please follow the instructions in this guide to install the Stand Alone Ethernet-VDSL Bridge. The VDSL Bridge is easy to install and does not require any software driver to operate. The Bridge can be used with any computer equipped with a 10/100 Mbps Ethernet interface with a standard RJ-45 port. If your system does not have a 10/100 Mbps Ethernet interface with an RJ-45 port, it will be necessary to install an Ethernet adapter that is appropriate for your system. Follow the manufacturer’s installation instructions for the Ethernet adapter.

## Packing List

Make sure the following items have been included in the VDSL Bridge package:

- One VDSL Bridge

- One 7.5V 1.0A Power Adapter (use only this adapter)

- One Ethernet Cable with RJ-45 connectors

- One RJ-11 Cable

- One bag Rubber foot for VDSL Bridge.

## Precautions!

IMPORTANT: Use only the Power Adapter included with your VDSL Bridge. Use of any other adapter can be a fire hazard and may cause irreparable damage to the device.

## Prerequisites

To install and use the VDSL Bridge, you must first have a computer with a dual speed (10/100 Mbps) Ethernet interface with a standard RJ-45 connection port. The operating system used by the computer and any installed protocols are irrelevant. The network connection operates as a normal Ethernet connection operating on Layer 2 only. The device is identified on the network by its unique machine (MAC) address. The Bridge automatically detects the polarity of the connection (MDI-X or MDI-II).

## Install the Stand Alone Ethernet-VDSL Bridge

The VDSL Bridge is a simple transparent bridge that provides a high speed VDSL connection for your computer as well as providing a connection for standard telephone services. Follow these steps to install the device:

Connect the appropriate cables (as illustrated on the next page) to your computer and telephone.

ENGLISH

ENGLISH

- Connect the Power Adapter to the Bridge.

- Turn on your computer and launch your favorite Internet browser.

## LED Display

On the front panel of the VDSL Bridge, the LED indications will be arranged as in the following diagram:

ENGLISH

 Power(Green): ON for power on, OFF for power off.

 VDSL Link(Green): ON for VDSL link, OFF for VDSL lost link.

 Ethernet Link(Green): ON for Ethernet link, OFF for Ethernet lost link.

 Ethernet Activity(Green): Blinking for transmit or receive activity,

OFF for transmit or receive no activity

## Connect the Stand Alone Ethernet-VDSL Bridge