# D-Link Corporation

Managed 24-Port N-Way Standalone Fast Ethernet with 1 slide-in module

Switch

# Product External Specifications

Product Model Name:

DHS-3224

Version A1

Version: 0.1

2001/4/17

By: Eric Huang

LAN Product Line

This document contains confidential proprietary information and is the property of D-Link Corporation. The contents of this document may not be disclosed to unauthorized persons without the written consent of D-Link Corporation.

DHS-3224 Revision History

| Version | Revised Date | # Person Name | # Content Revised |
| --- | --- | --- | --- |
| 0.1 | 2001/4/17 | Eric Huang | 1. Draft initiate |

### Product Description

This product provides a managed switch with 24 10/100M ports and an optional 2-port 100BASE-FX Fast Ethernet module. This product use fully integrated single-chip solution to make this product more cost effective and more easily in mass production. This product provides bandwidth control function to support customer’s requirement. Using 100BASE-FX provide a long distance uplink solution.

### Product Features

| Item | Feature | DHS-3224 Rev.A1 |
| --- | --- | --- |
| 1 | 10/100M Port Count | 24 |
| 2 | 100BASE-FX Port Count | 2 |
| 3 | RS232 Console Port | 1 |
| 4 | Per Device LED Indicator | Power |
| 5 | 10/100M TX LED Indicator | Speed, Link/Act |
| 6 | 100BASE-FX LED Indicator | Link, Act |
| 7 | Dimension | 441(W)x207(D)x44(H)mm (19-inch, 1U size) |
| 8 | Uplink Port | All port support auto MDI/MDIX |
| 9 | Packet Buffer | Per Device 8M bytes |
| 10 | Mac Address | 8K |
| 11 | 802.1p priority | V, 4 queues |
| 12 | Spanning Tree | V |
| 13 | VLAN | 64, 802.1Q VLAN |
| 14 | Bandwidth Control | V, Per port |
| 15 | Menu driven configuration interface/console | V |
| 16 | Telnet | V, one session |
| 17 | TFTP | V |
| 18 | Bootp/DHCP client | V |
| 19 | Management interface Authentication | V |
| 20 | Multiple access rights for different users | V |
| 21 | SNMP v1 | V |
| 22 | Firmware upgradeable | V |
| 23 | Backup/Restore Configuration file | V |
| 24 | MIB support | D-Link priority MIB, MIB-II RFC 1213, Bridge MIB RFC 1493, RIP MIB RFC1724, RMON MIBRFC 1757, IEEE802.1P RFC2674 |
| 25 | RMON | 1,2,3,9 4Group |
| 26 | Port Trunking | 1 group for 2-port optional module port, Release II |
| 27 | IGMP snooping | V, Release II |

### Port Features List

| Item | Feature | Fixed | Module |
| --- | --- | --- | --- |
|  |  | 10/100M ports | 100 FX (Single Mode) |
| 1 | Port Count | 24 | 2 |
| 2 | Interface | RJ-45 | SC |
| 3 | IEEE 802.3 10BASE-T | V | NA |
| 4 | IEEE 802.3u 100BASE-TX | V | NA |
| 5 | IEEE 802.3u 100BASE-FX | NA | V |
| 6 | Auto-Negotiation | V | V |
| 7 | IEEE 802.3x Flow Control | V | V |
| 8 | Back Pressure Flow Control | V | NA |
| 9 | Full Duplex | V | V |
| 10 | Half Duplex | V | NA |

### Physical & Environmental

#### AC input

- 100-240V AC, 50/60 Hz

- Internal universal power supply

#### DC input

- -48V DC input

- Internal power supply

- Release II

#### Operating Temperature

- 0 ~ 50 °C

#### Storage Temperature

- -25 ~ 55 °C

#### Humidity

- 5% ~ 95 °C

### Emission (EMI) Certifications

- FCC Class A (by customer request)

- VCCI Class A (by customer request)

- BSMI Class A (by customer request)

- CE Class A (by customer request)

- C-Tick Class A (by customer request)

### Safety Certifications

- UL/CUL

- TUV/GS (by customer request)

### ID design