Product Model Name:

DHS-3212V/3224V Version A1

# External Specifications Version 0.1

By: Eric Huang

2001/10/12

Digital Home/ Office LAN PLM

This document contains confidential proprietary information and is the property of D-Link Corporation. The contents of this document may not be disclosed to unauthorized persons without the written consent of D-Link Corporation.

DHS-3212V/DHS-3224V Revision History

| Version | Revised Date | # Person Name | # Content Revised |
| --- | --- | --- | --- |
| 0.1 | 2001/5/28 | Eric Huang | 1. Draft initiate |

### Product Description

This product provides a managed switch with 12/24 VDSL ports and 2-port Ethernet Uplink module. This product is D-Link first VDSL switch and is suitable for a apartment, hospital, factory, small office and home user connect to the Internet.

### Product Features

#### Hardware

| Item | Feature | DHS-3212V/3224V Rev.A1 |
| --- | --- | --- |
| 1 | VDSL Port Count | 12 (DHS-3212V) / 24 (DHS-3224V) |
| 2 | Uplink Module Port Count | 2 |
| 3 | RS232 Console Port | 1 |
| 4 | Per Device LED Indicator | Power, Console |
| 5 | VDSL Port LED Indicator | Link |
| 6 | Uplink Port LED Indicator | Link, Act |
| 7 | Dimension | 441(W) x 388(D) x 66(H) mm, 19-inch, 1.5U size |
| 8 | Packet Buffer | Per Device 8M bytes |

#### Layer 2 Switching Functions

| Item | Feature | DHS-3212V/3224V Rev.A1 |
| --- | --- | --- |
| 1 | Mac Address | 8K |
| 2 | 802.1p priority | V, 4 queues |
| 3 | IEEE802.1D Spanning Tree | V |
| 4 | VLAN | V, 802.1Q VLAN, Provide 255 static VLAN and also can run GVRP |
| 5 | Bandwidth Control | V, Per port support 5Mb/s or 10Mb/s or 15Mb/s upstream and down stream. |
| 6 | Loop Test | V |
| 7 | Auto-Negotiating | V |
| 8 | IGMP Snooping | V |
| 9 | Port Trunking | V, 2-port option module can be a trunk. |
| 10 | Switching forwarding mode | Store and forward |
| 11 | Broadcast storm control | V |
| 12 | GVRP | V |
| 13 | DiffServ | V, CoS |
| 14 | Port Mirroring | V |

#### Management Functions

| Item | Feature | DHS-3212V/3224V Rev.A1 |
| --- | --- | --- |
| 1 | Menu driven configuration interface/console | V |
| 2 | Web Configure/ Web-based management | V |
| 3 | Telnet | V, one session |
| 4 | TFTP Client | V |
| 5 | Bootp/DHCP client | V |
| 6 | Management interface Authentication | V |
| 7 | Multiple access rights for different users | V |
| 8 | SNMP v1 | V |
| 9 | Firmware upgradeable | V |
| 10 | Backup/Restore Configuration file | V |
| 11 | MIB support | D-Link MIB, SNMPv1 MIB RFC1157, MIB-II RFC 1213, Bridge MIB RFC 1493, RMON MIB RFC 1757, RFC1643 Ethernet like MIB, RFC 1155 SMI |
| 12 | RMON | 1,2,3,9 4 Groups |

### Port Features List

| Item | Feature | Fixed | Module |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | VDSL Port | 100BASE-FX Single mode | 100BASE-FX Multi mode | 1000BASE-SX Multi mode | 1000BASE-LX Single mode | GBIC BASE | 1000BASE-T | 10/100BASE-T X |
| 1 | Port Count | 12/24 | 1-port/ 2-port | 1-port/ 2-port | 2-port | 2-port | 2-port | 2-port | 2-port |
| 2 | Interface | RJ-21 | SC | SC | SC | SC | GBIC | RJ-45 | RJ-45 |
| 3 | Distance | 1km to 1.5km | 15 km | 2 km | 550 m | 5 km |  | 100 m | 100 m |
| 4 | IEEE 802.3 10BASE-T | N | N | N | N | N | N | V | V |
| 5 | IEEE 802.3u 100BASE-TX | N | N | N | N | N | N | V | V |
| 6 | IEEE 802.3ab 1000BASE-T | N | N | N | N | N | N | V | N |
| 7 | IEEE 802.3z | N | N | N | V | V | V | N | N |
| 8 | IEEE 802.3u 100BASE-FX | N | V | V | N | N | N | N | N |
| 9 | Auto- Negotiation | N | V | V | V | V | V | V | V |
| 10 | IEEE 802.3x Flow Control | N | V | V | V | V | V | V | V |
| 11 | Back Pressure Flow Control | N | N | N | N | N | N | V | V |
| 12 | Full Duplex | V | V | V | V | V | V | V | V |
| 13 | Half Duplex | N | N | N | N | N | N | V | V |

### Physical & Environmental

#### AC input

- 100-240V AC, 50/60 Hz

- Internal universal power supply

#### Support External Redundant Power Supply

- 100-240V AC, 50/60 Hz

- Internal universal power supply

- Current sharing

#### Operating Temperature

- 0 ~ 50 °C

#### Storage Temperature

- -25 ~ 55 °C

#### Humidity

- 5% ~ 95 %

#### Case

- 1.5U height, metal case.

- Dimensions: 441(W) x 388(D) x 66(H) mm (19-inch, 1.5U size)

### Emission (EMI) Certifications

- FCC Class A

- BSMI Class A

- CE Class A (by customer request)

- VCCI Class A (by customer request)

- C-Tick Class A (by customer request)

### Safety Certifications

- UL/CUL

- TUV/GS (by customer request)

### ID design

- DHS-3224V name plate

- DHS-3212V name plate