ANT24-0700

Date: November,15,2004

| Product name: | Wireless 2.4GHz 7dBi omni-directional indoor antenna |  |
| --- | --- | --- |
| Product Group & Type: | Wireless |  |
| Revision: | A1 |  |
| Available: |  |  |
| Picture: |  |  |
| Why Sell This Product: | - Optional indoor antenna for 2.4GHz wireless devices, offering higher Gain for wider operation range and flexible options for wireless users. - Expand your product line with antennas supporting D-Link and third-party vendor wireless devices. - Supports both RP-SMA and RP-TNC types of connectors, can work with a wide variety of wireless devices. Selling Points - Supports popular 802.11b and 802.11g standards. - Flexible installation options. - Adjustable antenna panel. |  |
| Key Features: | - 7dBi Gain omni-directional antenna - Compliant with 2.4GHz ISM band - For indoors use with 802.11b/g wireless devices - Extends and expands wireless networking environment range to 360 degree coverage in open space and office environment - Perfect pattern and adjustable angle for best - performance result - 4 flexible installation options: desktop on flat surface, magnetic attached, wall-mounting and direct attachment on wireless device - Stylish, unique housing design - Works with 2.4GHz wireless antennas equipped with RPSMA or RP-TNC connector - Extension cable provided for flexible and extendable antenna working environment |  |
| Technical Details: | Electrical Properties Frequency Range 2.4GHz to 2.5 GHz Directivity Omni-directional Impedance 50 ohm (normal) VSWR 1.92 (max.) Return Loss -10dB (max.) Antenna Structure Collinear Peak Gain 7dBi(without cable loss) Admitted Power 1 watt Cable Loss (1.5m) 2.1dB at 2.45GHz HPBW/E-Plane (Vertical) 24 degrees HPBW/H-Plane (Horizontal) 360 degrees Physical & Environmental Extension Cable - RG-178 50-ohm - 1.5-meter length, gray color Connector - RP-SMA - RP-SMA to RP-TNC adapter provided Antenna Material ABS, ABS+PC Color - Antenna: metallic gray and silver - Antenna base: metallic gray Operating Temperature -20 to +65 ℃ Storage Temperature -30 to +75 ℃ Operating Humidity 5% to 95% RH non-condensing Storage Temperature 5% to 95% RH non-condensing Dimensions 326 (H) x 120 (W) x 120 (D) mm (antenna base included) 9 mm (diameter) (antenna pole) Weight 226 grams (antenna base and extension cable included) |  |
| EMI/EMC Report | FCC No.  CE No.  VCCI No.  CTR-21 No.  C-Tick No.  A-Tick No.  FDA No.  R&TTE No.  BSMI No.  Others |  |
| Safety |  TUV No.  UL No.  CSA No.  Others |  |
| Packaging Style |  Clamshell  White Box  Gift Box  Bulk  White Box with D-Link Logo  Others |  |
| Packaging Contents: | - One ANT24-0700 2.4GHz indoor 7dBi omni-directional Antenna with 1.5M extension cable and RP-SMA Jack connector - One RP-SMA/ RP-TNC adaptor - One registration card and international office guide - Mounting kit includes: Two crews Two plastic fixings One sticker |  |
| UPC Code: | BANT240700A1; 790069273643. CANT240700A1; 790069273643. ANT240700.A1; 790069273667. TANT240700A1; 790069273933. NANT24-0700A1; 790069273940. |
| Units Per Carton: | 6 |  |
| Cartons Per Pallet: | For USA Air:36, Sea: 45 | For WW Air:48, Sea: 60 |
| Dimensions and Weight: | Box: , Others 365x148x145 mm, 0.412 kg Carton: 324x274x380 mm, 2.469 kg Pallet: (by Air) 1220x1020x1650 mm, 38.31 kg (for USA) 1200x1100x1670 mm, 43.25 kg (for WW) (by Sea) 1220x1020x2030 mm, 42.02 kg (for USA) 1200x1100x2050 mm, 48.19 kg (for WW) INTRASTAT weight: 0.153 kg (Box) Packing material weight (Box): - Paper: 0.123 kg - Plastic: 0.02 kg - Mixed: 0.01 kg |  |