| Firmware Version: | 1.03rc098 |
| --- | --- |
| Published Date: | 2016/10/07 |

Content:

New Features:	2

Changes of MIB & D-View Module:	2

Problems Fixed:	2

Known Issues:	3

Related Documentation:	3

Revision History and System Requirement:

| software Version | Date |
| --- | --- |
| 1.00r033 | 2014/09/23 |
| 1.01r053 | 2015/01/09 |
| 1.02rc068 | 2015/06/16 |

# New Features:

- Captive portal with Multiple SSID and VLAN

- Payment gateway(Paypal)

- Hotspot printing

- External captive portal

- Support AP load balancing

- SSID on/off by scheduling

- Country selection

- Manual channel selection on CWM

- Allow CWM admin(root, local, frontdesk) to input password as special characters(/,\,|,*,?,;,~,!,#,$,%,^,&,+,=" + ",<,>,',& )

- The online user is up to 100(CWM)

- The AP modules are embedded in CWM program

- The connected client system detection (Android,windows..etc)

- Add more measure items(bps /Bps / KBps/ MBps /GBps) in reporting page

- Add AP link down alarm in CWM(Sending email to admin)

- CWM management port(SSL) customization

- Add enable/disable selection on CWM CAPTCHA function

- ACL list import function

- The MAC ACL number limitation is 512 per radio(The original is 64 per SSID)

- Add Performance and Wireless resource settings from AP to CWM

# Changes of AP Module:

CWM_DAP2230 v1.00-R01-FWv100rc002

CWM_DAP2310B v2.05-R23-FWv205rc024

CWM_DAP2330 v1.05-R15-FWv105rc019

CWM_DAP2360B v2.05-R18-FWv205rc031

CWM_DAP2553B v3.05-R11-FWv305rc019

CWM_DAP2660 v1.10-R33-FWv110rc042

CWM_DAP2690B v3.15-R33-FWv315rc082

CWM_DAP2695 v1.15-R37-FWv115rc049

CWM_DAP3320 v1.00-R02-FWv100rc006

CWM_DAP3662 v1.01-R05-FWv101rc019

# Problems Fixed:

| Software Version | Problems Fixed |
| --- | --- |
| V1.03 | - The captive portal page will not be displayed when default home page is https page. - Fixed 403 error when using Firefox - Fixed use customization page source from CWM1.03 pages_license.tar (modify for customization) can review and display but after authentication Web UI display 'Not Found' and could not connect to internet. - Fixed use customization page source from CWM v1.02 can review and display but after enter passcode Web UI display 'Not Found' and could not connect to internet. - Fixed none Profile page is displayed on Monitor>Monitor list page. (Click Monitor list button would not redirect to anyone profile page.) - Fixed while 2.4G SSID and 5G SSID are different or Multi SSID is enabled, Band Steering should not be enabled by CWM. This cause wireless client is difficult to connect to AP by 2.4G. |

# Known Issues:

| Software Version | Known Issues |
| --- | --- |
| V1.03 | - Use customization page source from CWM v1.02 can review and display but after enter passcode Web UI display 'Not Found' and could not connect to internet. User need to upload their source file into CWM v1.03 and apply again. - [Performance]: Show wrong data rate in 5GHz. - [SSID-> User Authentication -> Paypal]: The passcode list table not list the time purchased by user. - [Multi-language]: some translate error in Multi-language. - [Front Desk Staff]: The station still can access the Internet when passcode been deleted - DWP-2360 (for China only) module will release soon |

# Related Documentation:

Central WifiManager_100_Manual_v1.03