### DAP-1160 Firmware Release Notes

Firmware: v1.30b10

Hardware: A1

Date: 2009/7/21

Problems Resolved:

- When operate as repeater or AP client mode, sometimes client will be disconnecting.

- AP Client and Repeater mode will not forward share port packets of DIR-655.

- When setting to DHCP server, the gateway info will show all zero.

- Watch dog function sometimes no response.

- Fail to connect to AP when operate as AP client mode using WPA/WPA2-PSK encryption.

- Fail to connect g only mode using Wii.

- When setting to DHCP server, the gateway info will show all zero.

Enhancements:

- Update DCC protocol to v2.0.

- Support MAC Clone on AP Client mode.

- Support UPnP IGD.

- Support Microsoft LLTD (Link Layer Topology Discovery) protocol

- Support 802.1d with spanning tree protocol

- Support WMM (Wi-Fi Multimedia) for AP Client mode.

Known Issues:

1.11n WLAN card endurance failed issue

-----------------------------------------------------------------------------------------------------------------------------------

Firmware: 1.20b06

Hardware: A1

Date: 2008/8/20

Problems Resolved:

- Fails to upgrade firmware if using the Firefox 3.0.

- Sometimes the MAC Clone function is not work when operates as WISP repeater mode.

- Sometimes user will fails to login to the web management page when operates as WISP Client router and WISP Repeater modes.

Enhancements:

- Support Traffic Manager on WISP Client Router and WISP Repeater mode.

- Add DHCP IP Reservations function.