# DAP-1522 Firmware Release Note

Firmware: v1.20-rc016

Hardware: 3A1

Date: Nov 21, 2008

Problems Resolved:

- Fix some message errors of language package.

- Mac Clone sometimes may fail when Priority by Port enabled.

Enhancements:

- Support WLAN scheduler.

- Support DHCP server (for both WLAN and LAN).

- Support QoS on Bridge mode.

Firmware: v1.11-rc0013

Hardware: A1

Date: Nov 6, 2008

Problems Resolved:

None

Enhancements:

  - Support DCC.

  - Support language package for German French, Spanish, and Italian.

Firmware: v1.10-rc009

Hardware: A1

Date: Sep 30, 2008

Problems Resolved:

- When operates as bridge mode, it will failed to connect to DGL-4500 and Linksys WRT610N with WPS.

- When operates as AP mode, the throughput is low with the DWL-AG660 adapter

- NETBEUI work failed in AP mode.

Enhancements:

  - Support DFS channel for AU.

  - Add Green Ethernet feature.

  - Support QoS (Priority by LAN port or protocol)

  - Support Traffic manager function in AP mode.

  - Support MAC cloning on bridge mode.

Firmware: v1.02-r0026

Hardware: A1

Date: July 10, 2008

Problems Resolved:

- Fixed the Bridge mode will send Prob-request for scanning in DFS band.

- Fixed the bug that PC can’t get IP through 1522 bridge mode if DHCP server sends Unicast response.

- Fixed the bug that UDP has high packet lost rate in some boards.(RGMII delay time issue between RT2880 and 8366SR)

- Fixed the bug that IGMP Snooping will drop UPnP SSDP packets.

Enhancements:

- Enable DFS channel for EU

- UK: UK (826) 5 GHz channel 36, 40, 44, 48, (DFS& TPC) channel 52, 56, 60, 64, 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140