| Firmware Version: | 1.15rc014 |
| --- | --- |
| Published Date: | 2012/08/13 |

Content:

Upgrading Instructions:	2

Upgrading by using Web-UI	2

New Features:	2

Changes of MIB & D-View Module:	2

Problems Fixed:	2

Known Issues:	3

Related Documentation:	3

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| 1.10rc009 | 2012/04/20 | DAP-2310 | A1 |

# Upgrading Instructions:

D-Link AP supports firmware upgrading via Web-UI. You may download the firmware from D-Link web site http://tsd.dlink.com.tw.

## Upgrading by using Web-UI

- Connect a work station to the LAN port of Access Point.

- Open the web browser of work station and entering the IP address of the AP. The system

default IP address is 192.168.0.50.

- Enter administrator’s username and password, user name is “admin” and leave

the password field blank by default.

To update the AP's firmware, click Maintenance > Firmware and SSL Certification

# New Features:

- Update new Wireless driver.

- Support IPv6

- Support DNS client for SNTP.

- Add Config file and F/W transfer status in MIB module.

- Add HT20/40 coexistent mechanism “Enable/Disable” in the Web UI.

# Changes of MIB & D-View Module:

None

# Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.10 | - Adjust TX power base on D-Link WiFi Frequency table - Fix MSSID missed too many beacons in WPA+OPEN security - Fix auto channel selection work failed in Multi-SSID - Fix WDS scanning caused data link break in WDS mode - Fix APC scanning caused data link break in AP client mode - Fix AP array endurance issue - Fix AP client mode low throughput issue. - Fix Low throughput issue in noise environment. - Fix AP can't switch to HT20 mode when detect leagcy 11g device. - Fix kernel panic caused by DHCP pkts forwarding in bridge. - Fix Hostapd crash issue when authentication type as 802.1X. - Fix a compatibility issue with Moto MC50 WinCE device in WPA/PSK mode. - Fix a Static IP assignment issue in WEBUI. - Fix WLAN module restart frequently when set as dynamic IP. - Fix memory leak issue in SNMP module. |

# Known Issues:

| Firmware Version | Known Issues |
| --- | --- |
| V1.15 | None |

# Related Documentation:

- DAP-2310 User Manual v1.15