| Firmware Version: | 1.16rc028 |
| --- | --- |
| Published Date: | 2014/01/06 |

Content:

Upgrading Instructions:	2

Upgrading by using Web-UI	2

New Features:	2

Changes of MIB & D-View Module:	2

Problems Fixed:	2

Known Issues:	3

Related Documentation:	3

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| 1.15rc014 | 2012/08/13 | DAP-2310 | A1 |

# Upgrading Instructions:

D-Link AP supports firmware upgrading via Web-UI. You may download the firmware from D-Link web site http://tsd.dlink.com.tw.

## Upgrading by using Web-UI

- Connect a work station to the LAN port of Access Point.

- Open the web browser of work station and entering the IP address of the AP. The system

default IP address is 192.168.0.50.

- Enter administrator’s username and password, user name is “admin” and leave

the password field blank by default.

To update the AP's firmware, click Maintenance > Firmware and SSL Certification

# New Features:

Security enhancement

- The setting key will be saved as encryption characters

- encrypt some sensitive info of WEB & SNMP

# Changes of MIB & D-View Module:

None

# Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.16 | - Fix in open system can be save success without network key - Fix in Repeater mode when SSID Visibility disabled ,Repeater can't connect to the root AP - Fix in APC mode when SSID Visibility disabled ,APC can't connect to the root AP - Fix in MultiSSID setting when only change the WEP Key Index, will pop up the warning message - Fix “Periodical Key Change” function doesn't work - Fix “MAC Clone Manual” function doesn't work - Fix “MultiSSID+WEP Spec Character” STA can't be connect to - Fix security can't be sync in AP Array - Fix in WPA-PSK 64 hex doesn't work normally - Fix in “Limit Administrator IP” for set IP in CLI and WEB - Fix in “MultiSSID's Priority” function doesn’t work - Fix WPA Authentication cannot work - Fix WDS connect issue - Fix in WDS->WPA-Personal security can't be scan - Fix in VLAN -> Disable the Multi-SSID but VLAN auto disable - Fix when set authentication to WPA-PSK and select autokey then use station to connect, it will show none. - Fix password support ASCII when the length less than 64 - Add MIB for “clientinfo tx rx byte” |

# Known Issues:

| Firmware Version | Known Issues |
| --- | --- |
| V1.16 | None |

# Related Documentation:

- DAP-2310 User Manual v1.15