| Firmware Version: | 1.06rc020 |
| --- | --- |
| Published Date: | 2015/07/09 |

Content:

Upgrading Instructions:	2

Upgrading by using Web-UI	2

New Features:	2

Changes of MIB & D-View Module:	2

Problems Fixed:	2

Known Issues:	3

Related Documentation:	3

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| 1.02rc017 | 2015/02/05 | DAP-2330 | A1 |

# Upgrading Instructions:

D-Link AP supports firmware upgrading via Web-UI. You may download the firmware from D-Link web site http://tsd.dlink.com.tw.

## Upgrading by using Web-UI

- Connect a work station to the LAN port of Access Point.

- Open the web browser of work station and entering the IP address of the AP. The system

default IP address is 192.168.0.50.

- Enter administrator’s username and password, user name is “admin” and leave

the password field blank by default.

To update the AP's firmware, click Maintenance > Firmware and SSL Certification Upload

# New Features:

Support CWM v1.02

1.	Disassociation supported on multi-SSID

2.	Support Captive portal customization

3.	Support config file upload from CWM

# Changes of MIB & D-View Module:

CWM_DAP2330 v1.06-R16

# Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.06 | - Fixed issue of client can’t access the Internet when Ethernet to WLAN disable - Modify timezone. - Fix captive portal on other SSID(including 5G primary and MSSID) will not work when captive portal on 2.4G primary SSID is not enabled |

# Known Issues:

| Firmware Version | Known Issues |
| --- | --- |
| V1.06 | None |

# Related Documentation:

- DAP-2330 User Manual v1.03