| Firmware Version: | 1.12rc073 |
| --- | --- |
| Published Date: | 2015/04/13 |

Content:

Upgrading Instructions:	2

Upgrading by using Web-UI	2

New Features:	2

Changes of MIB & D-View Module:	2

Problems Fixed:	2

Known Issues:	3

Related Documentation:	3

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| 1. 1.11rc062 | 2014/01/07 | DAP-2360 | A1 |

# Upgrading Instructions:

D-Link AP supports firmware upgrading via Web-UI. You may download the firmware from D-Link web site http://tsd.dlink.com.tw.

## Upgrading by using Web-UI

- Connect a work station to the LAN port of Access Point.

- Open the web browser of work station and entering the IP address of the AP. The system

default IP address is 192.168.0.50.

- Enter administrator’s username and password, user name is “admin” and leave

the password field blank by default.

To update the AP's firmware, click Maintenance > Firmware and SSL Certification

# New Features:

- Update OpenSSL from 0.9.8g to 0.9.8za.

# Changes of MIB & D-View Module:

None

# Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.12 | - APC mode can’t connect to AP and hides its SSID - New FW can’t match the old config, it cause WPA-PSK will change to OPEN. - AP array does not sync WPA-PSK information for MSSID and cause MSSID change to OPEN. |

# Known Issues:

| Firmware Version | Known Issues |
| --- | --- |
| V1.12 | None |

# Related Documentation:

- DAP-2360 User Manual v1.10