| Firmware Version: | 1.20rc039 |
| --- | --- |
| Published Date: | 2011/8/17 |

Content:

Upgrading Instructions:	2

Upgrading by using Web-UI	2

New Features:	2

Changes of MIB & D-View Module:	2

Problems Fixed:	2

Known Issues:	3

Related Documentation:	3

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| 1.15rc029 | 2010/11/30 | DAP-2590 | A1/2 |
| 1.20rc039 | 2011/08/17 | DAP-2590 | A1/2 |

# Upgrading Instructions:

D-Link AP supports firmware upgrading via Web-UI. You may download the firmware from D-Link web site http://tsd.dlink.com.tw.

## Upgrading by using Web-UI

- Connect a work station to the LAN port of Access Point.

- Open the web browser of work station and entering the IP address of the AP. The system

default IP address is 192.168.0.50.

- Enter administrator’s username and password, user name is “admin” and leave

the password field blank by default.

To update the AP's firmware, click Maintenance > Firmware and SSL Certification Upload

# New Features:

| Firmware Version | New Features |
| --- | --- |
| V1.20 | - Upload/download ACL file - Hide the password when login from CLI - Support APMII v2.15 - Support AP Array Tool - Web Redirection - Add PCI error, beacon stuck, cold/warm start into syslog - Reboot when meet PCI error problem - Web redirection phase 2 - Firewall on TCP/IP stack of AP itself - Multicast packet control |

# Changes of MIB & D-View Module:

None

# Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.20 | [IGMP Snooping] AP should not block LLMNR packet when IGMP snooping is enabled [ Help+ Multicast bandwidth] Add Multicast bandwidth part to Help file. [NTP] After disable NTP function, APUT still sends NTP packets. [F/W] Fix SNMP/LLTD caused memory leak [SNMP] Fix emailNotificationTable use wrong RGDB address [WDS with AP]: Remote WDS AP can't get IP address from wired. [WDS with AP]: Open/enable can built WDS with shared key. [Schedule] In schedule rule list, the name will show blank [APC]: Client AP can't get IP address from root ap. Fix when set WDS mode DAP-2590 can't forward LACP packs. [Client Information]: When Multi-SSID disable need to clear client information which connect to guest SSID. [VLAN->PVID auto assign]:Fix when add or del a Vlan in CLI, port's pvid doesn't change. [Traffic Control]: The uplink/downlink bandwidth can set to >150Mbps [WPA2-PSK+WDS WITH AP]: WDS AP always deauthent remote AP because of reason 6. [Trap Categorize Function]: The trap server use "MG-soft MIB Browser 10.0b", it can not receive any trap information. WDS throughput issue (HQ20100813000007--Australia) AP drops clients and stops accepting new connections [DEUR20100819000007-UKI] The password can be changed remotely by sending POST request. Modify Time Zone behavior |

# Known Issues:

| Firmware Version | Known Issues |
| --- | --- |
| V1.20 | None |

# Related Documentation:

- DAP-2590 User Manual v1.2