| Firmware Version: | 1.00rc011 |
| --- | --- |
| Published Date: | 2018/04/23 |

Content:

Upgrading Instructions:	2

Upgrading by using Web-UI	2

New Features:	2

Changes of MIB & D-View Module:	2

Problems Fixed:	2

Known Issues:	3

Related Documentation:	3

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |

# Upgrading Instructions:

D-Link AP supports firmware upgrading via Web-UI. You may download the firmware from D-Link web site http://tsd.dlink.com.tw.

## Upgrading by using Web-UI

- Connect a work station to the LAN port of Access Point.

- Open the web browser of work station and entering the IP address of the AP. The system

default IP address is 192.168.0.50.

- Enter administrator’s username and password, user name is “admin” and leave

the password field blank by default.

To update the AP's firmware, click Maintenance > Firmware and SSL Certification Upload

# New Features:

First Release

# Changes of MIB & D-View Module:

First Release

# Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.00 | None |

# Known Issues:

| Firmware Version | Known Issues |
| --- | --- |
| V1.00 | None |

# Related Documentation:

- DAP-2680 User Manual v1.00