| Firmware Version: | 1.11rc040 |
| --- | --- |
| Published Date: | 2012/04/23 |

Content:

Upgrading Instructions:	2

Upgrading by using Web-UI	2

New Features:	2

Changes of MIB & D-View Module:	2

Problems Fixed:	2

Known Issues:	3

Related Documentation:	3

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| 1.11rc040 | 2012/04/23 | DAP-2690 | A1 |

# Upgrading Instructions:

D-Link AP supports firmware upgrading via Web-UI. You may download the firmware from D-Link web site http://tsd.dlink.com.tw.

## Upgrading by using Web-UI

- Connect a work station to the LAN port of Access Point.

- Open the web browser of work station and entering the IP address of the AP. The system

default IP address is 192.168.0.50.

- Enter administrator’s username and password, user name is “admin” and leave

the password field blank by default.

To update the AP's firmware, click Maintenance > Firmware and SSL Certification Upload

# New Features:

- Update Web Redirection behavior

# Changes of MIB & D-View Module:

None

# Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.11 | - Fix Wireless stop service when too much interference in user environment. - Fix ARP table overflow issue. - WDS x 2 has Low throughput - Fix when enable the limit administrator vlan id but the web redirection doesn't disable - Fix downlink interface can't save success in traffic control - Fix when IGMP Snooping enable it don't block 224.0.0.252 in Multicast Bandwidth Control & IGMP Snooping - Add & Modify the commands of Multicast Bandwidth control. - Fix the AP hang up when running pressure test in Web Redirect. ( about 120~150 web sessions) - Fix Web authentication fail - Fix Web redirect + Web authentication fail - Fix LLTD packet will let br0 enter promiscuous mode, and cause memory leak. - Fix in 5G band, when set the WPA/WPA2-auto-Enterprise-TKIP,the channel width should be 20MHz and gray - Fix when delete the Static IP Pool, but the STA will always get the same static IP. - Fix when set the wireless mode is the mixed 802.11g and 802.11b, the channel width should be the gray to 20MHz at all times. - When set 802.11n only, the security-TKIP and WEP UI should be gray or would not have this item. - When build the APC mode, the signal is show error. - Fix in Time and Date Settings some TimeZone can't get the correct time. - Fix when enable the Multi SSID and modify it, the page will show error. - Fix when the HEX encrypt, set a~f, it will match upper/lower case. but when connect AP use STA, it will not match upper/lower case - Fix in 11a band/WPA-PSK/Periodical Key Change :it can't show time interval in email log info - Fix in 5Ghz Multi-SSID+Periodical Key Change when save and active, PassPhrase show null. - Fix 20 clients connect with AP, endurance test fail. |

# Known Issues:

| Firmware Version | Known Issues |
| --- | --- |
| V1.11 | None |

# Related Documentation:

- DAP-2690 User Manual v1.0