| Firmware Version: | 1.15rc039 |
| --- | --- |
| Published Date: | 2013/03/08 |

Content:

Upgrading Instructions:	2

Upgrading by using Web-UI	2

New Features:	2

Changes of MIB & D-View Module:	2

Problems Fixed:	2

Known Issues:	3

Related Documentation:	3

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| 1.03rc004 | 2010/12/14 | DAP-3520 | A1 |
| 1.02rc003 | 2010/04/01 | DAP-3520 | A1 |

# Upgrading Instructions:

D-Link AP supports firmware upgrading via Web-UI. You may download the firmware from D-Link web site http://tsd.dlink.com.tw.

## Upgrading by using Web-UI

- Connect a work station to the LAN port of Access Point.

- Open the web browser of work station and entering the IP address of the AP. The system

default IP address is 192.168.0.50.

- Enter administrator’s username and password, user name is “admin” and leave

the password field blank by default.

To update the AP's firmware, click Maintenance > Firmware and SSL Certification Upload

# New Features:

| Firmware Version | New Features |
| --- | --- |
| V1.15 | - Support AP Array Setup Utility - Upgrade Wireless driver to version 7.3.0.449 - Support “ping control” function - Add DNS setting on LAN Setting web page - Add HT20/40 coexistence enable/disable |

# Changes of MIB & D-View Module:

None

# Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.15 | - Fix the error for ACL file upload in D-View and APM2 - Fix the error info in the console log when running WDS endurance - Fix the when select year then day cannot be select in Time and Date - Correct the EG country code range - Fix when enable wireless schedule and vlan function the AP cannot be manage - Fix ”get multicastrate” always display 11Mbps in CLI - Fix ping through fail when MAC clone enabled for APC mode - Fix fail to manage AP caused by MAC confusion - Fix memory leak caused by SNMP module, - Fix bug when country as Israel, 5Ghz can be work - Fix when the current time is out of the range of the time, station still can associate to AP - Fix when AP Array Master AP leave and come back the role of Master AP issue - Fix when enable AP Array, the Limit Administrator IP range still worked - Fix application (indoor/outdoor) function bug when country code is JP in WEB and CLI and MIB - Fix set community didn't take effect when using CLI SNMP set host command - Fixe web page SNMP trap version bug. - Fixe ping delay issue when Ethernet port link down - Fixe memory leak issue caused by SNMP - Fixe that the WEB can’t disable VLan setup for Ethernet port when enable limit admin VLan id and save-active - Fix abnormal UI display in 5GHz and wireless setting cannot be saved anymore after pressing Scan button to site survey as 5GHz wireless client - Fix that DHCP discover packets without taged after save and active - Fix APMII update config fail - Fix ath0 mode setting, the invalid parameter value will be set as ath0 mode - Fix the bug that the device reset to factory default when update configuration file using APMII or D-View - Fix the bug about auto channel selection function for 2.4GHz - Fix the bug that No WDS/WDS with AP selections when wireless band as 2.4GHz - Fix 11n client disconnect issue when transfer big size file(WEP+AES) - Fix ifTable ifindex for Multi-SSID - Fix the scheduling bug when F/W upgrade from v103 to v115 - Fix the wrong IP address table in AP Array UI |

# Known Issues:

| Firmware Version | Known Issues |
| --- | --- |
| V1.15 | None |

# Related Documentation:

- DAP-3520 User Manual v1.10