| Firmware Version: | 1.10r027 |
| --- | --- |
| Published Date: | 2022/02/16 |

Content:

Upgrading Instructions:	2

Upgrading by using Web-UI	2

New Features:	2

Changes of MIB & D-View Module:	2

Problems Fixed:	3

Known Issues:	3

Related Documentation:	3

Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| v1.00r015 v1.01r018 | 2021/02/03 2021/05/18 | DAP-X2850 | A1 |

Upgrading Instructions:

D-Link AP supports firmware upgrading via Web-UI. You may download the firmware from D-Link web site http://tsd.dlink.com.tw.

Please also

- Make sure your DAP-X2850 has to manually assign a valid static IPv4 address or get a valid IPv4 address from DHCP server before upgrade FW.

- Do not power off your DAP-X2850 when FW upgrading.

Upgrading by using Web-UI

- Connect a work station to the LAN port of Access Point.

- Open the web browser of work station and entering the IP address of the AP. The system default IP address setting is dynamic ip, and you can enter http://dapx2850.local to login, if it cannot get ip address from DHCP server, it’s ip address will back to 192.168.0.50.

- Enter administrator’s username and password, user name and password are “admin” by default.

To update the AP's firmware, click Maintenance > Firmware and SSL Certification Upload

New Features:

- Upgrade SDK to SPF11.4

- Upgrade Openssl

- Support DNC 1.2.0.6 new feature

- Add channel 165 for KR country code

- Add Slovenia and Kosovo in country selection table

Changes of MIB & D-View Module:

None

Problems Fixed:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.10 | - Fix DBG21020140 - Captive Portal (MAC address authentication) UI shows incorrectly. - Fix HQ20210129000004 - Misspelling on Client Information - Fix DI20211129000009 - IPv6 setting issue - Fix DI20211130000007 - VHT Operation - Channel Width: 20 MHz or 40 MHz (0x00) - Fix DI20211202000003 – Doesn't restart udhcpc after enabling Limit Administrator VLAN. - Fix DI20211210000002 - Doesn't record "Captive portal, Client (MAC=) is in white list - Fix DI20211213000002 – Accepts W52/53 in outdoor mode w/o any error message via its CLI for JP country code - Fix DEUR20211208000005 - WLAN clients don't get IP-address |

Known Issues:

| Firmware Version | Known Issues |
| --- | --- |
| V1.10 | None |

Related Documentation: